3GPP TSG-RAN WG2 Meeting #122	R2-2305099

Inchon, South Korea, 22-26 May 2023

Agenda item:	7.19.3

Source:	Apple

Title:	R17 RedCap support of R18 eRedCap supporting gNBs

WID/SID:	NR_redcap_enh-Core

Document for:	Discussion / Decision

# Introduction

In RAN2-121-bis-e meeting the discussion did not conclude on whether the gNB supporting eRedCap should also support R17 RedCap or not, and the following was noted.

SIB1 should be able to indicate whether the cell enables access for eRedCap UEs or not (assuming that eRedCap UE is not allowed to access to the legacy cell nor the cell not supporting eRedCap). FFS on the relationship and granularity with the access control/cell barring purpose indication. Chair: P1b can be postponed, but also: Companies should consider if a cell can allow eRedCap UEs to connect, while not allowing RedCap UEs to connect? We can discuss in the next meeting.

In this paper, we provide reasons on why it is essential to have R17 RedCap support for eRedCap operation and so the support of R17 RedCap is anyway needed by the gNB.

# Dependency of R17 RedCap for eRedCap operation

It was discussed extensively during the eRedCap WID formulation and the plenary guidance was that eRedCap UE should be based on RedCap but with restrictions only the BW and PRB processing. This implies that several capabilities and expectations from eRedCap are the same as the R17 RedCap, and the gNBs which support eRedCap, need to implement these as well.

Observation 1: The further restriction in Rel-18 eRedCap UE is in the BW and PRB handling while the rest of the chracteristics and capabilities of R17 RedCap are expected to be the same for eRedCap.

Observation 2: The gNBs which support eRedCap, need to implement these R17 RedCap specific handling as the eRedCap UE is expected to operate with these as well (for eg: 12-bit PDCP. 12-bit RLC operation, the RRM relaxations etc)

While RAN2 has not yet made explicit agreements (yet), the access restriction and redcap specific broadcast information from R17 RedCap is expected to the same for eRedCap (expect for the below eRedCap specific agreements made)

Introduce R18 eRedCap UE specific IFRI in SIB1. The new R18 eRedCap UE specific IFRI functionality works as follows: - - Controls cell selection/reselection to intra-frequency cells for eRedCap UEs when this cell is considered barred by the eRedCap UE, as specified in TS 38.304 [20]. Introduce eRedcapAccessAllowed-r18 in interFreqCarrierFreqList in SIB4, about the frequency of neighbour cell supporting eRedCap, similar to R17.

We also agreed on eRedCap UE using the same initial BWP, which requires the gNB to also support the BWP that R17 RedCap UEs can use.

From RAN2 perspective, there is no need to introduce eRedCap UE specific initial BWP configuration (i.e. no R18 new field and at most one specific initial UL/DL BWP can be configured). If the R17 RedCap specific initial BWP is configured, eRedCap UEs always use it as its specific initial BWP (assuming no eRedCap UE specific initial BWP configuration field introduced).

RAN2 needs to unnecessarily spend time on discussing the different usecases where only eRedCap UEs are present, even when these eRedCap UEs would be using the R17 RedCap functionality for the most part. It would be simpler to assume that a gNB which supports R18 eRedCap is expected to build this on top of R17 RedCap.

Observation 3: eRedCap is envisioned to a subset of R17 RedCap, and not a standalone feature by itself. And it makes the RAN2 work easier by not wasting resources on different usecases which are not practical.

Based on the above reasoning, we propose that if the gNB broadcasts the support of R18 eRedCap, the UEs expect that the R17 Redcap is supported on this cell.

Proposal 1: gNB which broadcasts the support of R18 eRedCap, also broadcasts the support of R17 RedCap.

# Conclusions

Observation 1: The further restriction in Rel-18 eRedCap UE is in the BW and PRB handling while the rest of the chracteristics and capabilities of R17 RedCap are expected to be the same for eRedCap.

Observation 2: The gNBs which support eRedCap, need to implement these R17 RedCap specific handling as the eRedCap UE is expected to operate with these as well (for eg: 12-bit PDCP. 12-bit RLC operation, the RRM relaxations etc)

Observation 3: eRedCap is envisioned to a subset of R17 RedCap, and not a standalone feature by itself. And it makes the RAN2 work easier by not wasting resources on different usecases which are not practical.

Proposal 1: gNB which broadcasts the support of R18 eRedCap, also broadcasts the support of R17 RedCap.

# References

[1] Report from eRedCap breakout session - R2_121bis eRedCap (Mattias) 20230421_0545