3GPP TSG-RAN WG2 Meeting #122			R2-2305124

Incheon, Korea, May 22-26, 2023

Agenda Item:        7.10.2

Source:	Qualcomm Incorporated

Title:	FDM IDC Enhancements

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss FDM solutions that can be used to enhance the existing IDC solutions in Rel-18 .

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi).

- Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2)

Note: Enhancements to FDM solution is prioritized.

- Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4).
Note: The TDM solution is considered complementary to the FDM solution.

- Specify RRM requirements for TDM solution (RAN4)

Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

In Rel-18, it was agreed to introduce IDC enhancements in WID [1]:

In RAN2 121, the following agreements were reached:

FDM - Adopt Option 1 based frequency range reporting to the network i.e Center frequency + bandwidth in KHz/MHz for the actual affected frequencies is reported by the UE to the network for addressing IDC problem in R18. - Take the ASN.1 framework for option 1 as a starting point in the Text proposal section and work on the following enhancements - 1. Add granular values for band width (including BW in KHz/Mhz) to cover all the scenarios involving Wi-Fi, GNSS, BT - 2. Add the other IEs such as direction of interference. - 3. Add combination of frequencies’ range for addressing IMD scenarios. - 4. Check whether to reuse maxFreqIDC-r16, or define maxFreqIDC-r18 - In MR-DC scenarios, SN can also configure the UE for IDC reporting in SN, including both FDM and TDM solution. - no additional co-ordination is needed for IDC configuration, apart from the existing mechanism between MN and SN (i.e. candidateServingFreqListNR in CG-Config for EN-DC). - The gNB configures the candidate frequency ranges using (centre frequency + bandwidth) for which the UE should report IDC issues. Network may indicate the whole bandwidth of the freq. - The frequency range (centre frequency + bandwidth) reported by the UE shall at least overlap with the frequency range (centre frequency + bandwidth) configured by the network. - The centre frequency reported by the UE is within the frequency range (centre frequency + bandwidth indicated by network in the configuration) configured by the network. - If the UE detects interference in both directions for one candidate frequency range indicated by the gNB, the UE can report two affected frequency ranges with the respective interference direction, as legacy. No extra specification change is required. - LTE MN does not configure the UE with R18 NR IDC configuration.

# Configuration and Reporting Enhancements

The current running CR includes the following on reporting for FDM (and similar procedural text for TDM):

2> if there is at least one affected frequency range overlaps with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include the field affectedCarrierFreqRangeList with an entry for each affected frequency range; 3> for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth; 3> for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and set it accordingly; 2> if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or MR-DC combination overlaps with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include victimSystemType for each UL CA or MR-DC combination included in affectedCarrierFreqRangeCombList; 3> if the UE sets victimSystemType to wlan or bluetooth: 4> include affectedCarrierFreqCombRangeList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems; 4> for each affected frequency range in the UL CA or MR-DC combination, include centerFreq and affectedBandwidth; 3> else: 4> optionally include affectedCarrierFreqCombRangeList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems; 4> for each affected frequency range in the UL CA or MR-DC combination, include centerFreq and affectedBandwidth;

As can be seen from the text above, the field victimSystemType in the report can be only included when reporting of UL CA and MR DC. Thus, the UE is only restricted to reporting victimSystemType when reporting a UL-CA or MR-DC frequency issue but not when reporting adjacent frequency issues. In our view, this restriction does not make sense and is not properly motivated, there is no reason why the gNB can use this information only for frequency combination IMD reporting but not for single frequency adjacent IDC reporting.

Observation 1:   victimSystemType reporting is restricted to UL CA/MR-DC IMD reporting only in the current running CR.

Proposal 1: UE can optionally include victimSystemType field in individual frequency range reporting.

On the same topic, we note that the fields currently allowed are gps, glonass, bds, galileo, wlan, bluetooth have been inherited from Rel11 LTE.  We argue that the newer generations of UE can also now support Ultra-Wide Band UWB Tx/Rx operations which can also cause and be affected by IDC issues due to the nature of their transmission operation.

Proposal 2: “uwb” is added as a field value in victimSystemType.

Whole Bandwidth

In the last meeting, there is the following agreement [2]:

The gNB configures the candidate frequency ranges using (centre frequency + bandwidth) for which the UE should report IDC issues. Network may indicate the whole bandwidth of the freq.

In the ongoing email discussion [3] we are trying to discern the meaning of “whole” bandwidth. The argument is that the gNB can configure the UE to report center frequency only while indicating the Bandwith as “whole” or not indicating Bandwidth at all.

The issue would be:

- gNB can already configure any BW for reporting, unclear why it would not indicate this to the UE.

- RRC configuration is per cell, there is no “whole” band configuration for the RRC

- This would not be straightforward to interpret for non-serving cell

- UE can already indicate a set of carrier frequencies affected by IDC in Rel-16, unclear why the gNB would not configure that if it does not need BW information.

Proposal 3: gNB configures a specific centre frequency and BW for reporting in IDC.

# Conclusions

Observation 1:   victimSystemType reporting is restricted to UL CA/MR-DC IMD reporting only in the current running CR.

Proposal 1: UE can optionally include victimSystemType field in individual frequency range reporting.

Proposal 2: “uwb” is added as a field value in victimSystemType.

Proposal 3: gNB configures a specific centre frequency and BW for reporting in IDC.

# References

- RP-213589: New WI: In-Device Co-existence (IDC) enhancements for NR and MR-DC

- RAN2 121 Chairman Notes

[3] R2-23xxxxx [Post121][655][IDC] Discussion on Leftover issues for IDC (xiaomi)