3GPP TSG-RAN WG2 Meeting #122	R2-2305128

Incheon, Korea, May 22-26, 2023

Agenda item:	    7.21.3

Source: 		Qualcomm Incorporated

Title: 	UL Coverage Enhancements

Document for:	Discussion

# Introduction

In this contribution we discuss some views on Rel-18 Further UL Coverage Enhancements. The WID [1] asks RAN2 to handle the first objective below:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. The detailed objectives of the work item are as follows: - Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

The following was agreed in the last RAN2 meeting [2]:

- RAN2 assumes that MSG1 repetition can be applicable to all 4-step CBRA procedures (FFS for SI request) - CFRA support is FFS - RAN2 assumes that MSG1 repetition can be applicable to NUL - RAN2 assumes that MSG1 repetition can be applicable to SUL - Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE) - RAN2 waits for further inputs from RAN1 for how to associate RA-RNTI to the PRACH occasion for multiple PRACH transmissions and also for ra-ResponseWindow start point - General assumption is that various feature combinations can be configured (which is up to network implementation), unless explicitly specified otherwise - RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS - BWP selection mechanism is not impacted by PRACH coverage enhancements. Legacy BWP selection mechanism is re-used - RA type selection mechanism is not impacted by PRACH coverage enhancements. Legacy RA type selection mechanism is re-used

Also, the following LS was sent by RAN1 informing RAN2 that the following working assumptions were agreed [3]:

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

# PRACH repetition Fallback

It is important that RAN2 during this WI specify when a UE is allowed to use PRACH repetition in RACH configuration. RAN1 has already agreed the basic criteria for PRACH repetition.

Observation 1: RAN1 has agreed that at least, SS-RSRP can be used as criteria for triggering PRACH repetition.

Last meeting, the following proposal from [4] was discussed:

Proposal 15: RAN2 to discuss whether to support following fallback cases . • Case 1: Fallback from legacy RA to Msg1 repetition; • Case 2: Fallback from Msg1 repetition with lower number to Msg1 repetition with higher number; • Case 3: Fallback from 2-step RA to 4-step RA with Msg1 repetition; • Case 4: Fallback from CFRA to CBRA with Msg1 repetition.

With RAN2 deciding to explicitly agree on not supporting case 1. The three other cases are left FFS.

In our view, case 2 is useful to support. It is useful to take past PRACH failures into account when deciding on PRACH repetitions, similar to power ramping between failed RACH attempts. The idea is that the UE may modify the PRACH repetition number to a larger number according to a gNB configuration. We give some proposals on configuration in our companion control plane contribution.

Proposal 1: Fallback from Msg1 repetition with lower number to Msg1 repetition with higher number is supported.

For the other two fallback cases mentioned in [4], we do not see a strong motivation to support in the context of coverage enhacements

Proposal 2: RAN2 will not support:

•	Fallback from 2-step RA to 4-step RA with Msg1 repetition;

•	Fallback from CFRA to CBRA with Msg1 repetition.

# Other Repetition Criteria

In our view, coverage enhancements can be further improved if other criteria was further considered for PRACH repetition beyond RSRP and fallback such as UE power class or MSG3 repetition configuration.

Proposal 3: RAN2 to discuss additional PRACH repetition criteria such as UE power class or msg3 repetition configuration.

# Start of ra-ResponseWindow

In the last meeting, the following was agreed.

Agreements - RAN2 waits for further inputs from RAN1 for how to associate RA-RNTI to the PRACH occasion for multiple PRACH transmissions and also for ra-ResponseWindow start point

Looking at RAN1 #112bis agreements we see

Agreement The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

Thus, this should be modified accordingly in the MAC spec.

Proposal 4: RAN2 to confirm RAN1 agreement that the starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. RA-RNTI can be calculated based on the last PRACH attempt. FFS the details.

# Conclusion

Observation 1: RAN1 has agreed that at least, SS-RSRP can be used as criteria for triggering PRACH repetition.

Proposal 1: Fallback from Msg1 repetition with lower number to Msg1 repetition with higher number is supported.

Proposal 2: RAN2 will not support:

•	Fallback from 2-step RA to 4-step RA with Msg1 repetition;

•	Fallback from CFRA to CBRA with Msg1 repetition.

Proposal 3: RAN2 to discuss additional PRACH repetition criteria such as UE power class or msg3 repetition configuration.

Proposal 4: RAN2 to confirm RAN1 agreement that the starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. RA-RNTI can be calculated based on the last PRACH attempt. FFS the details.

# References

[1] RP-221858 Revised WID on Further NR coverage enhancements

[2] RAN2 121 bis Chair notes

[3]      R1-2304141 LS on PRACH coverage enhancement

[4]  R2-2303292	RAN2 impacts on R18 PRACH coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	NR_cov_enh2-Core