3GPP TSG-RAN WG2 Meeting #122                                                                 R2-2305156

Incheon, Korea, May 21 – 26, 2023

Agenda item: 		7.12.2.2

Source: 		Qualcomm Incorporated

Title: 	Enhancements for mobile IAB idle/inactive mode mobility

Document for:	Discussion

# 1	Introduction

RAN2 achieved the following agreements for idle/inactive cell reselection [1]:

mIAB cell type ind Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection. FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

This paper discusses remaining issues for connected-mode mobility during DU migration.

# 2	Discussion

The following discussion aims to identify at least one scenario where prioritization of mIAB-specific inter-frequency cell reselection may have benefits.

Scenario 1: Same frequencies supported by onboard cell as by surrounding cells

If the network supports the same frequencies onboard as the surrounding network, intra-frequency cell reselection should be prioritized over inter-frequency cell re-selection. Inter-frequency cell re-selection is of lower priority.

Observation 1a: In case onboard and surrounding cells support the same frequencies, intra-frequency cell reselection has highest priority. There is no need for mIAB-cell-type-specific inter-frequency cell reselection prioritization.

Scenario 2: Different frequencies supported by onboard cell than by surrounding network

In this case, inter-frequency cell reselection may be prioritized in areas where the UE is expected to embark or disembark the vehicle (e.g., at bus stops or train stations). However, since onboard cells and surrounding cells can be differentiated by frequency, the existing frequency-specific prioritization can be used for inter-frequency cell reselection.

Observation 1b: In case onboard and surrounding cells support different frequencies, mIAB-cell-type prioritization, if needed, could be based on existing inter-frequency cell reselection priorities.

Scenario 3: Only a subset of surrounding cell frequencies are used onboard.

For areas, where UEs are expected to embark or disembark from the vehicle hosting the mIAB-cell (e.g., bus stops or train stations), the following prioritization may be considered:

Case 1: Inter-frequency cell reselection of mIAB-cells may be prioritized for UEs camping on surrounding cells.

Case 2: Inter-frequency cell reselection of surrounding cells may be prioritized for UEs camping on mIAB-cells.

Case 3: Inter-frequency cell reselection of mIAB-cells may not be prioritized for UEs camping on mIAB-cells.

Case 4: Inter-frequency cell reselection of surrounding cells not be prioritized for UEs camping on surrounding cells.

For areas, where UEs are not expected to embark or disembark form the vehicle hosting the mIAB-cell, inter-frequency cell reselection to or from the mIAB-cell should not be prioritized.

Observation 1c: In case mIAB-cells uses a subset of the frequencies used by surrounding cells and in areas, where UEs are expected to embark or disembark from the vehicle, the following prioritization may be considered:

Case 1: Inter-frequency cell reselection of mIAB-cells may be prioritized for UEs camping on surrounding cells.

Case 2: Inter-frequency cell reselection of surrounding cells may be prioritized for UEs camping on mIAB-cells.

Case 3: Inter-frequency cell reselection of mIAB-cells may not be prioritized for UEs camping on mIAB-cells.

Case 4: Inter-frequency cell reselection of surrounding cells not be prioritized for UEs camping on surrounding cells.

RAN2 should discuss if these scenarios justify the introduction of mIAB-cell-specific inter-frequency cell-reselection priorities.

Proposal: RAN2 to discuss if mIAB-cell-specific inter-frequency cell reselection priorities to be introduced for the scenarios defined in cases 1 to 4.

# Conclusion

This paper discussed remaining issues for idle- and inactive-mode mobility during DU migration. The following observations and proposals have been made:

Observation 1a: In case onboard and surrounding cells support the same frequencies, intra-frequency cell reselection has highest priority. There is no need for mIAB-cell-type-specific inter-frequency cell reselection prioritization.

Observation 1b: In case onboard and surrounding cells support different frequencies, mIAB-cell-type prioritization, if needed, could be based on existing inter-frequency cell reselection priorities.

Observation 1c: In case mIAB-cells uses a subset of the frequencies used by surrounding cells and in areas, where UEs are expected to embark or disembark from the vehicle, the following prioritization may be considered:

Case 1: Inter-frequency cell reselection of mIAB-cells may be prioritized for UEs camping on surrounding cells.

Case 2: Inter-frequency cell reselection of surrounding cells may be prioritized for UEs camping on mIAB-cells.

Case 3: Inter-frequency cell reselection of mIAB-cells may not be prioritized for UEs camping on mIAB-cells.

Case 4: Inter-frequency cell reselection of surrounding cells not be prioritized for UEs camping on surrounding cells.

Proposal: RAN2 to discuss if mIAB-cell-specific inter-frequency cell reselection priorities to be introduced for the scenarios defined in cases 1 to 4.

# References

- Chair Notes; TSG RAN WG2 Meeting #121, Athens, Greece, February to March, 2023