3GPP TSG RAN WG2 #122	                             	R2-2305171

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.6.4

Source:	InterDigital, Inc.

Title:	IoT-NTN discontinuous coverage enhancements.

Document for:	Discussion

# Introduction

The WI on IoT NTN enhancements [1] includes the following objectives for discontinuous coverage enhancement:

4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

The relevant conclusions from the SA2 study [2] are as follows:

## 8.1 Conclusion on general mobility management and/or power saving The conclusions for solutions to the requirements R1, R2 for KI#1 and R5 and R6 for KI#2 are the following: Principles of system behaviour: - NTN MEO/LEO satellite or satellite constellation that provides discontinuous coverage is considered as the satellite access in 5GS and EPS for the power saving enhancement and UE unreachability period can be provided to both the UE and AMF/MME. - UE unreachability period provided to the UE and/or AMF/MME shall include either timing information when UE moves out/in of NTN coverage or information on satellite coverage at current and potential future locations of the UE. - Both network centric and UE centric procedures are used to determine and coordinate the UE unreachability period. These two approaches are not mutually exclusive, they serve different use cases and they can co-exist in the same network. In Network centric procedure, the AMF/MME determines the PSM parameters taking into account the UE unreachability period. In UE centric procedure, the UE informs the network about UE determined UE unreachability period and/or an indication when leaving/entering coverage. AMF/MME information retrieval: - AMF/MME shall not determine UE unreachability period based on satellite orbit data by itself. The AMF/MME receives UE unreachability period to be used for setting up the power saving parameters or handling mobility management of NTN UE. - AMF/MME obtains UE unreachability period information (timing information when UE moves out/in of NTN coverage) from an AF via the NEF/SCEF, from the UE or from OAM. The AMF/MME also takes into account UE location/mobility/trajectory information. The role of AMF/MME: - The AMF/MME configures the UEs mobility management and Power saving mode (PSM) parameters and sends the MICO/PSM parameters to UE before the satellite discontinuous coverage starts. - Takes into account the UE unreachability period in order to make sure the UE is in power saving mode if it is leaving network coverage, to avoid attempting to page the UE during this time and that the UE need not initiate NAS procedures excessively frequently compared to the Expected UE behavior, if known by the AMF/MME. - The mobility management and MICO/PSM parameters include: - Periodic registration update timer / Periodic TAU Timer, - MICO mode with optional Active Time and PS, - MICO mode with Extended Connected time (only for AMF case), - eDRX parameters. - AMF/MME takes the Tracking Area reported by RAN before AN release as the paging area. - The AMF/MME sets an implicit detach timer based on the UE unreachability period information to avoid de-registering or detaching the UE when it is in discontinuous coverage. NOTE: Whether the UE can be in CM-connected with RRC inactive and the AN release procedure is needed when there is no satellite coverage can be discussed during the normative phase. - Optionally, AMF can provide a backoff timer to prevent the UE from sending MO data/signalling while out of coverage. - Request the UE to inform AMF/MME about when UE unreachability periods begin and end. Regarding which information UE has and how UE retrieves it: - If possible, accurate UE coverage information is shared with UE, i.e. UE coverage information received from the network with enhanced solution compared to Rel-17. - Satellite coverage data may be transferred to a UE from an external server or an AF. - Following Rel-17, the UE has satellite assistance information for predication of discontinuous coverage from RAN broadcasts. Editor's note: LS need to have feedback from CT1 on whether UP or NAS via an AMF/MME (CP) will be used for the transfer depends on the discussion. Regarding the role of the UE: - The capability for the UE to be able to determine requested NAS timer values and MICO/PSM parameters based on its predicted UE unreachability periods needs to be specified. A UE that uses power saving initiates a NAS procedure, with sufficient time to complete the NAS procedure before the start of the unreachability period, and negotiates timer values and power saving parameters with AMF/MME as per existing specifications.

In RAN2#121 the following agreements were made:

Agreements: - RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage. - RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details - RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process) - Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID

In RAN2#121bis-e the following agreements were made

Agreements: - RAN2 will not introduce any enhancement to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap (e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement) - RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

In addition, several proposals from the offline discussion in RAN2#121bis-e were postponed due to lack of meeting time:

- Proposal 1 (For discussion): RAN2 to discuss how to support signalling of additional satellite information using one of the following options: - Option1: Dedicated signallingOption 2: Additional information in SIBs e.g. using an additional new SIB or SIB segmentation - Proposal 2a: (For discussion): If it is confirmed that RAN impact is expected, RAN2 will introduce an updated configuration for PTW modification (details are FFS) - Proposal 2b: (for discussion): RAN2 to consider further whether RAN assistance information is needed in addition to UE unreachability period in Registration Request. - Proposal 2c: (For discussion) RAN2 to send an LS to SA2, once it is clarified what potential impact is in RAN2 (i.e. what questions to ask to SA2).

In this contribution we further discuss the potential impact.

# Background

## 2.1	Discontinuous coverage enhancement in  Rel-17 NB-IoT

In Rel-17, due to lack of time, only the information broadcast in SIB32/SIB32-NB is specified, and it’s up to UE implementation what to do during discontinuous coverage in RRC_IDLE and how to handle running timers and so on. The UE may choose not to perform idle mode tasks while in discontinuous coverage. Since most of the behavior is left to UE implementation the benefit of the information broadcast in SIB32/SIB32-NB is limited.

The following parameters are provided (same IE is used in both SIB32/SIB32-NB)

SystemInformationBlockType32 field descriptions

elevationAngleLeft, elevationAngleRight Leftmost and rightmost (with reference to the satellite direction) elevation angle. Unit in degree. Step of 5 degree. Actual value = field value * 5. If the field elevationAngleLeft is absent, the value of field elevationAngleRight applies.

footprintInfo Satellite footprint. E-UTRAN may configure elevationAngles and/or radius for earth moving satellite. E-UTRAN may configure referencePoint and radius for quasi earth fixed satellite.

latitude Latitude of the reference point. Unit in degree. Step of 360 / 262144 degree. Actual value = field value * (360 / 262144).

longitude Longitude of the reference point. Unit in degree. Step of 360 / 262144 degree. Actual value = field value * (360 / 262144).

radius Distance between the reference point and the edge of the satellite or beam coverage. Unit in km. Step of 10 km. Actual value = field value * 10.

serviceInfo Information on when the satellite will provide coverage. E-UTRAN always configures tle-EphemerisParameters for a satellite with earth moving cell(s) and always configures t-ServiceStart for a quasi-earth fixed satellite.

tle-EphemerisParameters Mean values of the satellite orbital parameters based on the TLE set format for estimating in-coverage and out-of-coverage periods for a satellite with earth moving cell(s), see TS 36.304 [4].

t-ServiceStart Time information on when the incoming satellite is going to start serving the area for quasi-earth fixed satellite.

# Discussion

## 3.1	Release to RRC_IDLE and RRC_INACTIVE

For NB-IoT and eMTC, UE unreachability period prediction in RRC_IDLE and RRC_INACTIVE may provide an opportunity to save power even in R17, as the UE is allowed to suspend Idle mode tasks.

UEs may choose not to perform measurements of the serving cell or the neighbour cells during a UE unreachability period. The UE may avoid going to “any cell selection” state and start performing a scan of supported carriers/RATs during a UE unreachability period. Rather, the UE may choose to suspend measurements and reselection and rather “sleep” during the UE unreachability period in order to save battery life.

The other main benefit of suspending idle mode tasks would of course be DRX/eDRX and paging monitoring. If the UE is configured with a relatively short DRX (i.e. not eDRX) then it is likely that the paging cycle is shorter than a UE unreachability period. In this case the UE should simply be allowed not to monitor the paging occasions that fall within the UE unreachability period - this would anyway be impossible and allowing the UE to save power by avoiding trying to synchronise to a cell during this time would be beneficial.

In our understanding, the behaviour captured in R17 allows UE not to perform idle mode tasks, therefore this behaviour is already allowed. However, we should consider whether to clarify more explicitly.

Proposal 1: For eMTC and NB-IoT: A UE in R17 is allowed not to perform RRC_IDLE mode tasks during a UE unreachability period. Consider whether to explicitly clarify that this means that if a UE in RRC_IDLE or RRC_INACTIVE determines it is in a UE unreachability period, the UE may choose not to perform measurements of the serving cell or neighbour cells, and may postpone moving to “any cell selection” state, and is allowed not to attempt to monitor paging occasions which occur during a UE unreachability period.

It has been agreed as follows:

- RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one):

  - Explicit RRC Release using a new RRC Release cause

  - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

RAN2 should discuss the use case for each of the options. Currently, a UE is already allowed not to perform Idle mode tasks when in discontinuous coverage. Therefore, it is unclear to us what new behaviour is expected in case a new release cause is introduced. In addition, the behaviour may be different for the RRC_INACTIVE and RRC_IDLE cases. For example, if the RRC Release message moves the UE to RRC_INACTIVE due to discontinuous coverage, what actions are expected from the UE compared to legacy? (E.g. stop performing cell reselection measurement).

Proposal 2: RAN2 to discuss what new behaviour is expected from the UE when NW releases the UE using a new cause e.g. “discontinuous coverage” in the following cases:

Case 1: Release to RRC_IDLE

Case 2: Release to RRC_INACTIVE

Currently in RRC_CONNECTED, the UE will perform the RLF procedure upon detecting N310 out-of-sync indications (E.g. due to out of coverage). Upon detecting N310 out-of-sync indication the UE starts timer T310. If the UE does not get back in sync before T310 expires, the UE attempts to perform cell selection and starts timer T311. If the UE does not find a suitable cell before T311 expires then the UE finally enters RRC_IDLE.

If the UE is able to immediately enter RRC_IDLE without executing the RLF/re-establishment procedure then some power can be saved, particularly in the case that there are frequent coverage gaps.

Proposal 3: It is beneficial in terms of UE power saving if the UE can enter RRC_IDLE upon detecting a coverage gap without executing the RLF procedure.

## 3.2	Additional satellite information

As per the discussion in the previous meeting, it was highlighted that SA2 expects that information for more satellites will be necessary in R18. To support this, we either need additional SIBs or dedicated signalling. As a result, we propose to revisit the postponed proposal from the previous meeting:

Proposal 4: RAN2 to discuss how to support signalling of additional satellite information using one of the following options:

Option1: Dedicated signalling

Option 2: Additional information in SIBs e.g. using an additional new SIB or SIB segmentation

## 3.2	Paging

According to SA2 [2] “The AMF/MME configures the UEs mobility management and Power saving mode (PSM) parameters and sends the MICO/PSM parameters to UE before the satellite discontinuous coverage starts.” and “The mobility management and MICO/PSM parameters include eDRX parameters”

In case the UE is configured with a long DRX (i.e. eDRX) then it may be the case that the UE is configured with a paging cycle which is longer than a UE unreachability period. In some cases, the PTW may fall within the UE unreachability period, and in extreme cases e.g. when there are many relatively long UE unreachability periods this may result in the UE being unreachable for long periods of time (i.e. much longer than the configured eDRX cycle due to missed PTWs). It may be possible by configuration to avoid this some of the time, however this may not be possible in all cases.

In the last meeting, the following proposals were not treated due to lack of time [3]

Proposal 2a: (For discussion): If it is confirmed that RAN impact is expected, RAN2 will introduce an updated configuration for PTW modification (details are FFS) Proposal 2b: (for discussion): RAN2 to consider further whether RAN assistance information is needed in addition to UE unreachability period in Registration Request. Proposal 2c: (For discussion) RAN2 to send an LS to SA2, once it is clarified what potential impact is in RAN2 (i.e. what questions to ask to SA2).

In the last meeting, 4 potential approaches were provided in [4], [5], [6], [7]. Upon comparing the solutions, they are not so different.

Solution 1:

In [4] it was proposed to update the PTW calculation. To elaborate, the proposal is not to modify the PTW calculation itself, but rather to consider only the H-SFNs which fall into coverage and exclude those which fall in a coverage gap when performing the existing PH and PTW calculation. As an example, consider the illustration in figure 1.

Figure 1: “Remapping” the PH taking into account in coverage/out of coverage H-SFNs.

In the above example, 14 UEs are shown, each one of those would fall into a different PH based on UE_ID.

We can renumber H-SFNs to exclude the ones which are out of coverage – e.g. as follows

If H=SFNs are : 1, 2, 3, 4, 5, 6, 7, 8, 9, 10.

And if H-SFNs 8, 9, 10 are out of coverage, then the remapping would be as follows.

1, 2, 3, 4, 5, 6, 7, X, X, X = “H-SFN-remapped”

By using the remapped H-SFNs (instead of true H-SFN) compared to the UE-ID mod N (where N is the number of in coverage H-SFNs – in this example it’s 7) the UE would calculate a PH which falls into one of the in-coverage H-SFNs. In this example, UE 8 is shifted to the same H-SFN as UE 1, UE 9 is shifted to the same H-SFN as UE 2 – i.e. an offset of 7 ( = N ) is applied. One could also define a “re-mapping” which is applied by all UEs (not just the out of coverage ones), in order to spread the PHs evenly amongst available H-SFNs to minimise the impact on paging load. However, the most basic example as shown above is a simple offset determined for the UEs in the coverage hole, calculated based on the number of in-coverage H-SFNs.

In other words, this approach calculates an offset for the UEs with a PH in a coverage gap by removing the out of coverage H-SFNs used for the PH calculation.

Solution 2:

In [5], a configurable offset is proposed. The details are not provided, for example whether the offset is provided per UE in dedicated signalling, or whether this is enabled by the network and the UE calculates a UE specific offset based on UE-ID. If it is the latter then there is not much difference to the solution 1 above.

In other words, this approach configures an offset for the UEs with a PH in a coverage gap. The details are not provided e.g. whether UE determines a UE specific value based on a common configuration or whether the gNB configures an offset per UE.

Solution 3:

In [6] it is proposed to configure multiple DRX configurations. One would assume that the UE should not monitor multiple PTW all of the time, because this would be equivalent to just configuring a shorter eDRX cycle. The power consumption would be increased in this case. Rather, the UE would monitor the “additional PTWs” only in case a coverage gap is detected. In this case, this is equivalent to calculating an offset based on applying a different eDRX configuration while in a coverage gap.

In other words, this approach configures an offset for the UEs with a PH in a coverage gap by providing an additional DRX configuration.

Solution 4:

In [7] it is proposed that the UE autonomously extends the paging window, “if it detects a shift in the window” and reports to the NW.

In other words, this approach uses an offset determined by UE implementation and reported to the NW.

In summary, it seems all of the approaches use an offset, with differing details of how to configure or calculate the offset.

Observation: All of the proposed solutions for updating the PTW to receive paging outside of a coverage gap determine or configure an offset. Each solution may use a different approach to determine/configure the offset.

Proposal 5: Update the PTW if it falls in a coverage gap by applying an offset. FFS how to determine/configure the offset.

In terms of what to ask SA2, we can indicate the above agreement, or at least that RAN2 are considering shifting the PTW if it falls in a coverage gap. To assist this, we need to ask what the content/granularity of “UE unreachability period” is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible from SA2 point of view.

Proposal 6: Send an LS to SA2 indicating that RAN2 <have agreed/are considering> shifting the PTW if it falls in a coverage gap, asking what the content/granularity of “UE unreachability period is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible from SA2 point of view.

A draft LS is provided in [8]

# Conclusion

In this contribution we discuss the potential discontinuous coverage enhancements for R18 IoT-NTN and have the following proposals:

Proposal 1: For eMTC and NB-IoT: A UE in R17 is allowed not to perform RRC_IDLE mode tasks during a UE unreachability period. Consider whether to explicitly clarify that this means that if a UE in RRC_IDLE or RRC_INACTIVE determines it is in a UE unreachability period, the UE may choose not to perform measurements of the serving cell or neighbour cells, and may postpone moving to “any cell selection” state, and is allowed not to attempt to monitor paging occasions which occur during a UE unreachability period.

Proposal 2: RAN2 to discuss what new behaviour is expected from the UE when NW releases the UE using a new cause e.g. “discontinuous coverage” in the following cases:

Case 1: Release to RRC_IDLE

Case 2: Release to RRC_INACTIVE

Proposal 3: It is beneficial in terms of UE power saving if the UE can enter RRC_IDLE upon detecting a coverage gap without executing the RLF procedure.

Proposal 4: RAN2 to discuss how to support signalling of additional satellite information using one of the following options:

Option1: Dedicated signalling

Option 2: Additional information in SIBs e.g. using an additional new SIB or SIB segmentation

Observation: All of the proposed solutions for updating the PTW to receive paging outside of a coverage gap determine or configure an offset. Each solution may use a different approach to determine/configure the offset.

Proposal 5: Update the PTW if it falls in a coverage gap by applying an offset. FFS how to determine/configure the offset.

Proposal 6: Send an LS to SA2 indicating that RAN2 <have agreed/are considering> shifting the PTW if it falls in a coverage gap, asking what the content/granularity of “UE unreachability period is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible from SA2 point of view.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- 23.700-28-i00, “Study on Integration of satellite components in the 5G architecture; Phase 2”

- R2-2304258, “[offline-115] Discontinuous coverage enhancements”, InterDigital, Inc.

- R2-2303716, “IoT-NTN discontinuous coverage enhancements”, InterDigital, Inc.

- R2-2302822, “RAN2 enhancements for discontinuous coverage”, ZTE Corporation, Sanechips.

- R2-2303963, “Discussion on discontinuous coverage”, Huawei, Turkcell, HiSilicon.

- R2-2303193, “On RAN impacts for Discontineous coverage enhancements”, Nokia, Nokia Shanghai Bell.

- R2-2305172, “<draft> LS on PTW modification due to UE unreachability”, InterDigital, Inc.