3GPP TSG-RAN WG2 Meeting #122	R2-2305228

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.15.2

Source: 	Xiaomi

Title:  	Discussion on LCP restriction from COT sharing

Document for:	Discussion and Decision

1	Introduction

In RAN2#121 meeting, RAN2 discussed the LCP impact from COT sharing and achieved the following agreement [1].

Agreement on SL LCP and COT

1: 	UE can select 1/ either to do a changed-LCP, in order to satisfy the COT requirement, and to do the type-2 LBT (How to do the LCP can be decided after RAN1 agreement) 2/ or to do a legacy-LCP, e.g. using type-1, type-2 LBT. FFS on the need of assistance INFO to initiating UE. FFS on spec impact, e.g., conditions for UE to choose either solution.

In this contribution, we will further elaborate the detailed LCP restriction from COT sharing and provide corresponding proposals.

2	Discussion

- Case 1: COT sharing information arrives later than packet generation:

For this case, since the responding UE has already finished the LCP upon reception of the COT sharing information, if the generated packet satisfies the COT requirement, i.e., the initiating UE is the target receiver of the packet and the CAPC of the packet is equal to or smaller than the CAPC indicated in the COT sharing information, then to increase the efficiency of COT sharing, the UE should use the shared COT, i.e., performs type 2 LBT.

Actually during online discussion, there was some concern that even the PDU has been generated, there may be “other PDU” pending for transmission and utilizing the shared COT. However, we don’t think UE should prioritize “other PDU” when there is already a PDU satisfying the requirement, this is because to transmit “other PDU”, UE needs to preform type 1 LBT which is a waste of the shared COT with the access latency/complexity increased. Also there was some voice to leave this to UE implementation whether to perform type 1 LBT or type 2 LBT. However we think if the already generated MAC PDU is able to utilize the shared COT with a type 2 LBT, UE has no motivation to perform a type 1 LBT, it does not make sense to leave this to UE implementation.

Observation 1: To transmit “other PDU” instead of the PDU satisfying the COT requirement is a waste of shared COT with the access latency/complexity increased.

Regarding the decision on LBT type, there was some comment that this is more of PHY scope and transparent to MAC. We think for normal transmission on the unlicensed spectrum, it is purely PHY decision, however the story of COT sharing is different since type 2 LBT is allowed only when the generated MAC PDU can satisfy the shared COT, but actually whether the MAC PDU satisfies the shared COT or not is within the MAC scope. Therefore, we think to perform a type 2 LBT is a MAC and PHY joint decision and is some kind of internal interaction.

Observation 2: It is a MAC and PHY joint decision to perform a type 2 LBT since whether the MAC PDU satisfies the shared COT or not is within the MAC scope.

Based on the above analysis, we think for this case, if the generated packet satisfies the COT requirement, UE should utilize the shared COT with a type 2 LBT executed.

Proposal 1: If the resource to be used is within a shared COT, and if PDU is generated before COT arrival, and the PDU satisfies COT requirement, UE should transmit this generated PDU with a type 2 LBT executed.

- Case 2: COT sharing information arrives before packet generation:

For this case, the responding UE can perform an enhanced LCP to satisfy the shared COT. In last meeting, we discussed about this case and achieved the following agreement.

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

Actually there was some proposal to consider the priority/remaining PDB of the LCHs that have data for transmission when determining whether to perform an enhanced LCP. Even we have some sympathy on this proposal, the whole system can still survive if no such additional conditions are introduced. For UEs operating in both mode 1 and mode 2, either the scheduled grant or the selected grant e.g., initial transmission /retransmission should take the QoS requirement into account, so it is not that serious if the shared COT is utilized to transmit a “satisfied PDU” even not having the highest priority or the most stringent PDB requirement. The “other” LCHs can still survive with the following scheduled/selected transmission resource.

Observation 3: LCHs with high priority and stringent PDB requirement can still survive with the following scheduled/selected transmission resource.

In addition, the existing requirement to utilize the shared COT is already quite complicated, if more conditions are considered, on one hand, the impact on the specification is significant, on the other hand, it is not an easy job to define a specific rule, e.g., to consider either one or both of priority and remaining PDB. Also the bar of priority and/or remaining PDB to restrict the use of enhanced LCP should be determined. Besides priority/remaining PDB, it is also possible to rely on the NW configuration to restrict the enhanced LCP, e.g., NW can flag some LCHs and when these LCHs are having data for transmission, UE is not allowed to perform an enhanced LCP.

Observation 4: It is not an easy job to define a specific rule to restrict the usage of enhanced LCP.

Considering all the mentioned aspects above, we prefer to not define any additional rules to limit the UE to use the enhanced LCP. Or if there is really some concern we think it is also enough to leave to UE implementation whether to perform an enhanced LCP.

Proposal 2: If the resource to be used is within a shared COT, and if PDU is not generated before COT arrival, and there is data satisfying COT requirement, whether to perform enhanced LCP is up to UE implementation.

Regarding how to enhance LCP and use the shared COT, we achieved the following agreement.

Agreement:

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

However, it is not clear how to restrict the destinations to be those allowed by the COT, e.g., whether the CAPC requirement is considered or not during DST selection procedure.

- CAPC requirement is considered during destination selection:

According to the existing destination selection procedure, the UE should select the destination that having at least one of the MAC CE and the LCH with the highest priority among the LCHs that satisfy the existing destination selection conditions [see Annex], e.g., SL data available, CG/HARQ feedback limitation etc. If CAPC is considered, one more condition can be added on top of the other conditions, i.e., associated CAPC is equal to or smaller than the CACP indicated in the COT sharing information.

In this case, at the first step, when selecting the destination, the responding UE should judge whether the initiating UE has a highest priority LCH/MAC CE satisfying the existing destination selection conditions as well as the CAPC requirement. If so, the responding UE can select the initiating UE as the destination and then continue to select LCHs belonging to the initiating UE which satisfy the existing LCH selection conditions [see Annex] and have a CAPC that is equal to or smaller than the indicated CAPC, then the responding UE performs type 2 LBT. However, if the initiating UE has no highest priority LCH/MAC CE that satisfying the existing destination selection conditions as well as the CAPC requirement, i.e., the initiating UE cannot be selected as the destination, then the responding UE can fallback to legacy LCP procedure to select DST and LCHs, and performs a type 1 LBT.

Figure 1: CAPC requirement is considered during destination selection

- CAPC requirement is not considered during destination selection:

In this case, the responding UE relies on the legacy destination selection procedure to judge whether the initiating UE can be selected as the destination or not with no consideration on the CAPC requirement, i.e., whether the initiating UE has a highest priority MAC CE/LCH satisfying the existing destination selection conditions or not.

If so, the responding UE can select the initiating UE as the destination, then further check if there is any LCHs belonging to the initiating UE which satisfy the existing LCH selection conditions and the CAPC requirement. If there is any LCH can be selected, the responding UE performs a type 2 LBT; otherwise, if there is any LCHs belonging to the initiating UE which satisfy the existing LCH selection conditions but cannot meet the CAPC requirement, the responding UE select these LCHs, then performs a type 1 LBT; if the initiating UE cannot be selected as the destination based on the existing destination selection condition, the responding UE selects performs a legacy LCP to select the DST and LCHs, and performs a type 1 LBT.

Figure 2: CAPC requirement is not considered during destination selection

Based on the above analysis, it is much easier to implement the enhanced LCP procedure if CAPC requirement is considered during destination selection procedure. Therefore, we have the following proposals:

Proposal 3: If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, CAPC requirement is considered during destination selection procedure.

Proposal 4: If the initiating UE has a LCH satisfying the existing destination selection conditions as well as the CAPC requirement, the responding UE selects the initiating UE as the destination, otherwise, the responding UE falls back to legacy LCP procedure.

Then the next step after DST selection is to select LCHs, with P3/4, during DST selection, the CAPC requirement is already considered, therefore, during LCH selection procedure, the responding UE can select at least one LCH. Also, in order to guarantee the generated packet satisfies the CAPC requirement, all the other selected LCHs should have a CAPC smaller than or equal to that indicated in the shared COT information. This is because we have agreed the CAPC of the MAC PDU is the lowest priority CAPC of the LCHs included in the MAC PDU, in this case, in order to satisfy the CAPC requirement, all the selected LCHs should have a CAPC smaller than or equal to that indicated in the shared COT information. In addition, there are some existing conditions [see Annex] for LCH selection, the selected LCHs should also meeting these legacy conditions.

Agreement on SL CAPC mapping rule:

1: 	As in NR-U, the lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Proposal 5: If the initiating UE is selected as the destination, further select the LCHs satisfying the existing conditions and having CAPC smaller than or equal to the CAPC value indicated in the COT sharing information.

3	Conclusion

In this contribution, we discussed about remaining issues for LCP restriction from COT sharing and have the corresponding observations and proposals:

Observation 1: To transmit “other PDU” instead of the PDU satisfying the COT requirement is a waste of shared COT with the access latency/complexity increased.

Observation 2: It is a MAC and PHY joint decision to perform a type 2 LBT since whether the MAC PDU satisfies the shared COT or not is within the MAC scope.

Observation 3: LCHs with high priority and stringent PDB requirement can still survive with the following scheduled/selected transmission resource.

Observation 4: It is not an easy job to define a specific rule to restrict the usage of enhanced LCP.

Proposal 1: If the resource to be used is within a shared COT, and if PDU is generated before COT arrival, and the PDU satisfies COT requirement, UE should transmit this generated PDU with a type 2 LBT executed.

Proposal 2: If the resource to be used is within a shared COT, and if PDU is not generated before COT arrival, and there is data satisfying COT requirement, whether to perform enhanced LCP is up to UE implementation.

Proposal 3: If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, CAPC requirement is considered during destination selection procedure.

Proposal 4: If the initiating UE has a LCH satisfying the existing destination selection conditions as well as the CAPC requirement, the responding UE selects the initiating UE as the destination, otherwise, the responding UE falls back to legacy LCP procedure to select DST and LCHs.

Proposal 5: If the initiating UE is selected as the destination, further select the LCHs satisfying the existing conditions and having CAPC smaller than or equal to the CAPC value indicated in the COT sharing information.

4	Reference

[1] RAN2#121meeting minutes.

[2] RAN2#121bis meeting minutes.

[3] TS 38.321, MAC specification.

5 Annex: LCP procedure

5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is configured according to TS 38.331 [5]:

2>	if the new transmission is associated to a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon:

3>	select a Destination associated with NR sidelink discovery as specified in TS 23.304 [26], that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI:

4>	SL data for NR sidelink discovery is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant.

2>	else:

3>	select a Destination associated to one of unicast, groupcast and broadcast (excluding the Destination(s) associated with NR sidelink discovery as specified in TS 23.304 [26]), that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

4>	SL data for NR sidelink communication is available for transmission; and

4>	SBj > 0, in case there is any logical channel having SBj > 0; and

4>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

4>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

4>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

1>	else:

2>	select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

3>	SL data is available for transmission; and

3>	SBj > 0, in case there is any logical channel having SBj > 0; and

3>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

3>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

3>	sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

1>	select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination:

2>	SL data is available for transmission; and

2>	sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and.

2>	sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	sl-HARQ-FeedbackEnabled is set to the value that satisfies the following conditions:

3>	if PSFCH is configured for the sidelink grant associated to the SCI and the UE is capable of PSFCH reception:

4>	sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or

4>	sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions.

3>	else:

4>	sl-HARQ-FeedbackEnabled is set to disabled.

NOTE 2:	HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE or Sidelink DRX Command MAC CE or Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.