3GPP TSG-RAN WG2 Meeting #122                                                        R2-2305288

Incheon, KR, 22nd – 26th May, 2023

Source:	CATT

Title:	Discussion on Coexistence for LTE sidelink and NR sidelink

Agenda Item:	7.15.7

Document for:	Discussion and Decision

Introduction

In RAN#99 meeting, the WID of NR sidelink evolution was modified as following [1]:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In this contribution, RAN2 impacts for co-channel coexistence for LTE sidelink and NR sidelink will be discussed.

Discussion

In the RAN1#112bis meetings, the following agreements were reached:

Agreement For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed.

Considering no change in the LTE SL specifications are required in RAN1, hence, in this contribution, only the impacts on NR SL should be considered.

Proposal 1: From RAN2 perspective, only the co-channel coexistence on NR SL should be considered in Rel-18.

Regarding to the co-channel case, there are two kinds of resource sharing mechanism (semi-static resource pool sharing and dynamic resource pool sharing) are agreed in RAN1. The corresponding agreements were listed below:

Agreement Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs.

Regarding to the above two resource sharing mechanism, clarification that whether they are both needs to be specified or further down-selection will be performed is needed.

Proposal 2: Send LS to RAN1 to check whether both semi-static resource pool sharing and dynamic resource pool sharing are both needs to be specified or further down-selection is needed.

If semi-static resource pool sharing needs to be supported, if only TDM-based semi-static resource pool partitioning is allowed, the legacy resource pool configuration can be reused. There seems no further RAN2 enhancement is needed.

If dynamic resource sharing needs to be supported, the following RAN1 agreements were reached:

Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer. Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe (in which UE has not monitored due to its transmission) in LTE SL module …… Agreement In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission … …

Based on the above agreements, it is obvious that it will impact the MAC resource (re-)selection procedure. According to TS38.321 [2], there are two kinds of resource (re-)selection:

- Random selection

- (Partial) Sensing based

The corresponding description in [2] is as below:

- 5.22.1.1 SL Grant reception and SCI transmission … … 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is not configured by RRC: 4> if transmission based on random selection is configured by upper layers: 5> randomly select the time and frequency resources for one transmission opportunity from the resource pool which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> else: 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

Random selection

When RRC configure the transmission based on random selection, MAC will randomly select the time and frequency resources from the resource pool. If the resource pool is dynamic shared with LTE SL, it should avoid selecting resources overlapping with resources which was already selected by LTE SL module. Hence, it is proposed:

Proposal 3: If dynamic resource pool sharing needs to be supported, when MAC performs resource (re-)selection based on random selection, the NR SL module should exclude the resources in a slot overlapping with the resources which have been selected by LTE SL module.

(Partial) Sensing based

For (partial) sensing based resource (re-)selection, the candidate resource is provided by PHY layer. PHY is responsible for resource excluding. Hence, there is no MAC impact.

Proposal 4: If dynamic resource pool sharing needs to be supported, when MAC performs (partial) sensing based resource (re-)selection, there is no RAN2 impact.

Conclusion

According to the analysis in section 2, the proposals on the above issue are as follows:

Proposal 1: From RAN2 perspective, only the co-channel coexistence on NR SL should be considered in Rel-18.

Proposal 2: Send LS to RAN1 to check whether both semi-static resource pool sharing and dynamic resource pool sharing are both needs to be specified or further down-selection is needed.

Proposal 3: If dynamic resource pool sharing needs to be supported, when MAC performs resource (re-)selection based on random selection, the NR SL module should exclude the resources in a slot overlapping with the resources which have been selected by LTE SL module.

Proposal 4: If dynamic resource pool sharing needs to be supported, when MAC performs (partial) sensing based resource (re-)selection, there is no RAN2 impact.

Reference

- RP-230077  WID revision: NR sidelink evolution  RAN#99

- TS 38.321  Medium Access Control (MAC) Specification  3GPP