3GPP TSG-RAN WG2 Meeting #122										   R2-2305322

Incheon, Korea, May 22-26, 2023

Agenda item:		7.3.5

Title: 	Further discussion on connected mode mobility

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#98, a new WID [1] of Network energy savings for NR has been agreed. And the objective about CHO enhancement is the following:

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

In this paper, we will further discuss the CHO enhancement for NES, and discuss the impact on RAN2 specification.

- Discussion

In RAN2#121 and RAN2#121b meeting, the agreements about mobility enhancements for NES are as below:

Agreements

-	Study whether CHO enhancements are needed for the purpose of turning off the cell

-	Continue discussing CHO in the context of different NES techniques.

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

In our understanding, there are two scenarios of source cell entering NES mode as below:

- case 1: source cell will be turned off, connection between source cell and UE will be lost. At the same time, target cell could supply connection for UE.

- case 2: source cell will enable cell DTX/ DRX, the discontinuous transmission or reception would delay the transmission and decrease the user rate of UE.

In general, the handover is triggered before the UE performance deteriorates. Hence, it is necessary for UE to initiate handover before the source cell enters NES mode.

Observation 1: UE initiates handover before the source cell enters NES mode.

Moreover, from gNB perspective, the time of source cell entering NES mode could be determined by gNB at advance. Hence, the triggering time could be supplied in CHO configuration to UE before the source cell enters NES mode.

Observation 2: The triggering time could be supplied in CHO configuration to UE before the source cell enters NES mode.

As above, the triggering condition is a time condition. In NTN, a time based CHO triggering event is introduced as below:

CondEvent T1 (Time measured at UE is within a duration from threshold) Inequality T1-1 (Entering condition) Mt>Thresh1 Inequality T1-2 (Leaving condition) Mt>Thresh1+Duration

Specifically, gNB could predict the load of source cell, estimate the time (e.g.Thresh 1) that source cell is going to be turned off and then transmit a CHO command carrying Thresh 1 and duration parameters to UE at advance. UE could search for a target cell where UE’s RSRP is highest, then, trigger a handover towards it after Thresh 1, and finish the handover before Thresh 1+duration. And the value range of duration is (1..6000), it could be configured as a suitable value to distribute UE within duration in order to avoid the RACH congest in the target cell. As a result, a time based CHO triggering event (CondEvent T1) could be used in CHO for NES.

Moreover, in order to ensure the better service quality in target cell, the RSRP of the target cell should be as execution metric. Considering the NES mode of source cell couldn’t deteriorate the RSRP of source cell, CondEvent A4 could be considered as execution event.

As above, CondEvent A4 in combination with CondEvent T1 coud be used as execution event.

Proposal 1: CondEvent A4 in combination with CondEvent T1 coud be used as execution event in CHO for NES.

For the selection of the target cell, considering UE doesn’t know the load and scheduling of the cell, it is unable to distinguish a cell that could allocate the most resource for it. But gNB could obtain the information of the neighbor cell, the information could include the load, NES mode and so on. From network perspective, if the neighbor cell enables NES mode, but it still could supply better service quality for UE, it could be the candidate cell. Hence, it is up to NW implementation to select some candidate cells and configure them to UE via CHO.

Proposal 2: It is up to NW implementation to select some candidate cells.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Observation 1: UE initiates handover before the source cell enters NES mode.

Observation 2: The triggering time could be supplied in CHO configuration to UE before the source cell enters NES mode.

Proposal 1: CondEvent A4 in combination with CondEvent T1 coud be used as execution event in CHO for NES.

Proposal 2: It is up to NW implementation to select some candidate cells.

- References

[1] RP-223540 New WID proposal: Network energy savings for NR