3GPP TSG-RAN WG2 Meeting #122        	R2-2305327

Incheon, Korea, 22th – 26th May, 2023

Source:	vivo

Title:          Discussion on architecture general

Agenda Item:	7.16.2.1

Document for:	Discussion and Decision

- Introduction

At the previous RAN2 meetings, RAN2 made the following agreements regarding architecture and general aspects:

RAN2 #119bis R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. R2 assumes that a model is identified by a model ID. Its usage is FFS. RAN2 #120 R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN2 #121 RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified. RAN2 #121bis - FFS if For UE capability for AIML methods we use the UE capability mechanisms as defined for RRC reported and LPP reported capabilities. - For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model). - For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model). - R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information. - The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage. - Model ID can be used to identify model or models for the following LCM purposes: - model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step). - (e.g. for so called “model ID based LCM”) - If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose. - How to achieve globality of the Model ID is FFS. - Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point: - Direction1: Pre-defined/hard-coded global unique model ID - Direction3: Assigned global unique model ID via specific ID management node. - Note: Other global unique model ID definition is not precluded. - Model ID structure, if any, is FFS

This contribution will further discuss the general aspects of architecture from RAN2’s perspective, including:

- UE AI/ML capability

- UE-initiated vs NW-initiated model/function selection/ (de)activation/switching/fallback

- Model ID utilization

- Model meta-data

- Functional architecture

- Mapping of functionality to entities

- Discussion

- 2.1 UE AI/ML capability

In the last RAN2 meeting, UE AI/ML capability was discussed. The conclusion is as below:

- FFS if For UE capability for AIML methods we use the UE capability mechanisms as defined for RRC reported and LPP reported capabilities.

The UE AI/ML capability is used for model management. Based on UE AI/ML capability, the model management entity can, e.g., transfer suitable model to UE, activate the corresponding functionality, etc. It is still unclear whether the model management entity is a centralized node for all AI use cases or per AI use case, and whether there is a new unified AI layer which is used for model management for all use cases. We think that it is too early to discuss UE AI/ML capability and should be postponed to normative phase to be discussed after some schemes are clear.

Proposal 1: UE AI/ML capability is postponed to discuss in normative phase.

- 2.2 UE-initiated vs NW-initiated

There are the following conclusions in the last RAN2 meeting:

- For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

- For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

Generally the network which is responsible for the overall features performance. With the introduction of AI aspects, this basic design principles is applicable for AI aspects as well. Therefore, for UE-sided model, network should be able to perform the related management. Thus, NW-initiated model/function selection/(de)activation/switching /fallback are needed.

Proposal 2: For UE-sided model, NW-initiated model/function selection/(de)activation/switching /fallback are needed.

For UE-sided model, another form of network control is that the network can provide the UE with applicable conditions and the UE initiates the related action when the corresponding condition is fulfilled. Similarly, network can provide UE with some applicable condition to allow UE-initiated model magement related actions.

Therefore,

Proposal 3: For UE-sided model, UE-initiated model/function selection/(de)activation/switching /fallback are needed, and NW may provide the UE with applicable conditions to trigger the corresponding action(s).

For UE-sided model, the UE should be allowed to request NW whether to initiate some LCMrelated action, for example for power consumption reason UE may request NW to deactive a model. Then the NW can determine whether to allow UE initiate the related action or not based on NW’s strategy.

Proposal 4: For UE-sided model, the UE is allowed to request NW to select/de-activate/switch/ fallback model, for example for power consumption reason.

For NW-sided model, it is network inner operation and there is no need for UE-initiated operation. For the CSI compression and beam management use cases, the NW-initiated operation can be left to NW implementation. However, for positioning use case it is possible that the model management is performed between LMF and gNB for uplink positioning methods, which may be discussed by other WG, such RAN3.

Proposal 5: For NW-sided model, there is no need for UE-initiated operation. FFS that the NW-initiated operation is left to NW implementation, e.g., the model management between gNB and LMF for positioning use case.

- 2.3	Model ID Utilization

There are the following conclusions in the last RAN2 meeting regarding model ID:

- Model ID can be used to identify model or models for the following LCM purposes: - model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step). - (e.g. for so called “model ID based LCM”) - If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose. - How to achieve globality of the Model ID is FFS. - Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point: - Direction1: Pre-defined/hard-coded global unique model ID - Direction3: Assigned global unique model ID via specific ID management node. - Note: Other global unique model ID definition is not precluded. - Model ID structure, if any, is FFS

We can notice that SA2 is discussing format of model ID as captured in [2].

NOTE 1: The structure and format of the ML Model identifier and its uniqueness are up to stage 3.

It is better that RAN2 and SA2 aligns the structure/format of model ID by using uniform model ID definition. Thus, it is proposed that RAN2 wait for SA2 progress and send LS to SA2 to inform SA2 that RAN2 expects model ID to be globally unique and would align with SA2 on the the structure/format and ask for SA2’s opinion.

Proposal 6: RAN2 to align the structure/format of global model ID with SA2, and send LS to SA2 to coordinate the structure/format.

- 2.4 Model meta-data

There are the following conclusions in the last RAN2 meeting regarding model ID:

- R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

In the RAN2#119bis meeting, RAN2 agreed the assumption that proprietary model may be supported. For proprietary model, the model format is vendor specific and vendor info may be needed to identify the model format.

Observation 1: vendor info can be used to indicate model format in case model format is vendor specific.

Currently it is unclear whether global model ID contains vendor info or not. If global model ID contains the vendor info, there is no need for meta data to contain vendor info again, otherwise meta data should contain verdor info at least for identifying model format of proprietary model.

Proposal 7: Meta data should contain vendor info to implicitily identify the model format in case of propriatery model format.

The model functionality, introduced by RAN1, is used to indicate the usage of the model. Model functionality can be for example by network to indicate to UE to activate or deactive a model. Thus, model functionality can also be used for model management related action, e.g., fallback to legacy behavior. So, We think that model functionality info may be essential for model management that can contained in model meta data.

Proposal 8: Meta data should contain model functionality for model management related action.

- 2.5 Functional architecture

In RAN3, the General framework for RAN artificial intelligence is illustrated in Figure 1.

Figure 1: Functional Framework of AI for RAN in RAN3

For the general framework of AI for RAN in RAN3, model management is mainly an internal implementation concerning model transfer and model performance feedback. In comparison, the AI/ML for air interface involves extensive interactions between the UE and the network, which are related to model management, such as model monitoring and model activation/deactivation.

Therefore, reusing the existing framework for RAN is not suitable, and it would be beneficial to introduce additional components on top of the framework of AI for RAN. In the latest RAN2 meeting, RAN2 has agreed that the general AI/ML framework consists of: (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage. Based on the RAN2 agreements, we suggest that the architecture presented in the following Figure 2 is captured into the TR.

Figure 2: Functional architecture of AI for air interface

Proposal 9: Capture the above functional architecture of AI for air interface into the TR.

- 2.6 Mapping of functionality to entities

To our understanding, the mapping of functionality to entities will imply the signaling procedure between entities. For instance, if model is stored at the network and model inference is located at UE, model transfer/delivery will be needed between the network and the UE. That’s why we need to identify the location of model training/storage, even if it’s a consensus that offline training is up to implementation.

Besides, the mapping of functionality to entities is use case specific.

Proposal 10: In order to fully identify the potential specification impact, RAN2 is expected to identify the mapping of functionality to entities, which is use case specific.

For functionality mapping, we think the following function entities can be considered based on RAN2 agreements and the functional architecture in the above Figure 2:

- Data Source entity: This entity provides related data for model training.

- Model Training entity: This entity performs the model training.

- Model Storage entity: This entity stores the trained model.

- Model Inference entity: This entity performs the model inference.

- Model Management entity: This entity is responsible to perform the model management, e.g., model/function selection/(de)activation/switching/fallback, etc.

Thus, the following template can be considered for companies to report the mapping of functionality to entities:

Table 1: mapping of functionality to entities

|  | Data Source entity | Model Training entity | Model Storage entity | Model Inference entity | Model Management entity |
| --- | --- | --- | --- | --- | --- |
| (sub)Use Case |  |  |  |  |  |

Proposal 11: Model functionality mapping consists of the following function entities: Data Source entity, Model Training entity, Model Storage entity, Model Inference entity and Model Management entity .

Proposal 12: if Proposal 11 is agreed, endorse the template in Table 1 as a starting point to discuss the mapping of functionality to entities.

- Conclusion

UE AI/ML capability

Proposal 1: UE AI/ML capability is postponed to discuss in normative phase.

UE-initiated vs NW-initiated model/function selection/ (de)activation/switching/fallback

Proposal 2: For UE-sided model, NW-initiated model/function selection/(de)activation/switching /fallback are needed.

Proposal 3: For UE-sided model, UE-initiated model/function selection/(de)activation/switching /fallback are needed, and NW may provide the UE with applicable conditions to trigger the corresponding action(s).

Proposal 4: For UE-sided model, the UE is allowed to request NW to select/de-activate/switch/ fallback model, for example for power consumption reason.

Proposal 5: For NW-sided model, there is no need for UE-initiated operation. FFS that the NW-initiated operation is left to NW implementation, e.g., the model management between gNB and LMF for positioning use case.

Model ID Utilization

Proposal 6: RAN2 to align the structure/format of global model ID with SA2, and send LS to SA2 to coordinate the structure/format.

Model meta-data

Observation 1: vendor info can be used to indicate model format in case model format is vendor specific.

Proposal 7: Meta data should contain vendor info to implicitily identify the model format in case of propriatery model format.

Proposal 8: Meta data should contain model functionality for model management related action.

Functional architecture

Figure 2: Functional architecture of AI for air interface

Proposal 9: Capture the above functional architecture of AI for air interface into the TR.

Mapping of functionality to entities

Proposal 10: In order to fully identify the potential specification impact, RAN2 is expected to identify the mapping of functionality to entities, which is use case specific.

Proposal 11: Model functionality mapping consists of the following function entities: Data Source entity, Model Training entity, Model Storage entity, Model Inference entity and Model Management entity .

Proposal 12: if Proposal 11 is agreed, endorse the template in Table 1 as a starting point to discuss the mapping of functionality to entities.

Table 1: mapping of functionality to entities

|  | Data Source entity | Model Training entity | Model Storage entity | Model Inference entity | Model Management entity |
| --- | --- | --- | --- | --- | --- |
| (sub)Use Case |  |  |  |  |  |

Note: The further analysis of functionality mapping and potential specification impact for each use case are attached in the Annex for information.

- Reference

[1]. RP-213599		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2]. 3GPP TS 23.288 V18.1.0

- Annex

- 5.1 Functionality Mapping and Procedure of CSI compression

Based on RAN1’s agreements, we understand that the mapping of functionality to entities for CSI compression use case can be as in the following table:

Table 5.1-1: mapping of functionality to entities for CSI compression use case

|  | Data Source entity | Model Training entity | Model Storage entity | Model Inference entity | Model Management entity |
| --- | --- | --- | --- | --- | --- |
| Type1 | Input: UE Label: UE | gNB | gNB | UE (Encoder) gNB (Decoder) | gNB |
| Type3 | Input: UE Label: UE | gNB (Decoder) UE (Encoder) | N/A | UE (Encoder) gNB (Decoder) | gNB |

For CSI compression use case, the overall procedure for LCM is shown in the following figure:

Figure 5.1-1: signaling procedures of CSI compression

Step 1. The UE indicates its AI/ML-related capability to the RAN node.

Step 2. The RAN node sends the CSI RS resource and reporting configuration to the UE. The UE performs CSI measurement and feedback CSI report, e.g., PMI or raw channel.

Step 3. The RAN node performs model training based on the acquired report.

Step 4. The UE indicates the stored model to the RAN node by model identification.

Step 5. The RAN node may send the meta-data during the model transfer.

Step 6. The UE may activate the model based on the model configuration autonomously or with explicit activation indication from the RAN node.

Step 7. The RAN node may send model performance monitoring configuration to the UE.

Step 8. The UE performs CSI measurement and obtains the data required for model inference as CSI generation.

Step 9. The UE reports the compressed CSI report and the RAN node performs subsequent model inference as CSI reconstruction.

Step 10-11. The UE reports the performance feedback to the RAN node. The report can be one-shot, event-triggered, or periodical. The RAN node monitors the performance and makes decisions about model activation/deactivation/ updating/switching.

Potential standard impact

AI/ML-based CSI compression may introduce the following potential Uu interface impact:

- AI-related Capability for CSI compression

- For model training, configuration and report of CSI, e.g., PMI or raw channel.

- Model identification/ transfer for CSI compression, e.g., Model ID, Meta-data;

- For model inference, configuration and report of compressed CSI report.

- For model monitoring, configuration and report of performance monitoring parameters, e.g., SGCS of model input and output.

- 5.2 Functionality Mapping and Procedure of BM

Based on RAN1’s agreements, we understand that the mapping of functionality to entities for beam prediction use case can be as in the following table:

Table 5.2-1: mapping of functionality to entities for beam prediction use case

|  | Data Source entity | Model Training entity | Model Storage entity | Model Inference entity | Model Management entity |
| --- | --- | --- | --- | --- | --- |
| UE-sided model without model transfer (Alt.2) | Input: UE Label: UE | UE | N/A | UE | gNB |
| UE-sided model with model transfer (Alt.3) | Input: UE Label: UE | gNB | gNB | UE | gNB |
| gNB-sided model (Alt.1) | Input: UE Label: UE | gNB | N/A | gNB | gNB |

For beam prediction use case, the overall procedure for LCM is shown in the following figure:

Figure 5.2-1: signaling procedures of Beam management

Step 1. The UE sends its AI/ML-related capability, indicating supporting beam prediction use case, to the RAN node.

Step 2. The RAN node sends the beam-sweeping resource and reporting configuration to the UE. The UE performs L1-RSRP measurement and sends report of all L1-RSRP and Rx beam angles to the RAN node.

Step 3. The RAN node performs model training based on the acquired report.

Step 4. The UE indicates the stored model to the RAN node by model identification.

Step 5. The RAN node may send the meta-data during the model transfer.

Step 6. The UE may activate the model based on the model configuration autonomously or with explicit activation indication from the RAN node.

Step 7. The RAN node may send model performance monitoring configuration to the UE.

Step 8. The UE performs L1-RSRP measurement and obtains the input data required for model inference.

Step 9. The UE performs model inference and reports the Top-k predicted L1-RSRP and Tx beam indexes to the RAN node.

Step 10-11. The RAN node monitors the performance and makes decisions about model activation/deactivation/ updating/switching.

Potential standard impact

AI/ML-based Beam management may introduce the following potential Uu interface impact:

- AI-related Capability for beam prediction

- For model training, configuration and report of L1-RSRP and Rx beam angle;

- Model identification/ transfer for beam prediction, e.g., Model ID, Meta-data;

- For model inference, configuration and report of Top-k predicted L1-RSRP and Tx beam indexes to the RAN node;

- Model monitoring, including configuration and report of performance metric, model activation/deactivation

- 5.3 Functionality Mapping and Procedure of positioning

Based on RAN1’s agreements, we undertand that the mapping of functionality to entities for positioning use case can be as in the following table:

Table 5.2-1: mapping of functionality to entities for positioning use case

|  | Data Source entity | Model Training entity | Model Storage entity | Model Inference entity | Model Management entity |
| --- | --- | --- | --- | --- | --- |
| Case1/2a without model transfer | Input: UE Label: UE/PRU | UE | N/A | UE | LMF |
| Case1/2a with model transfer | Input: UE Label: PRU/LMF | LMF | LMF | UE | LMF |
| Case2b | Input: UE Label: PRU/LMF | LMF | N/A | LMF | LMF |
| Case3a | Input: gNB Label: gNB/PRU/LMF | LMF | LMF | gNB | LMF |
| Case3b | Input: gNB Label: PRU/LMF | LMF | N/A | LMF | LMF |

For positioning use case, the overall procedure for positioning is shown in the following figure:

Figure 5.3-1: signaling procedures of Beam management

Step 1. The UE sends its AI/ML-related capability, indicating supporting positioning use case, to the LMF.

Step 2. The LMF sends the PRS resource and reports configuration to the UE. The UE performs PRS measurement and feedback with the CIR, RSRP, or ToA, which may be associated with the ground-truth location of the LMF.

Step 3. The LMF performs model training based on the acquired report.

Step 4. The UE indicates the stored model to the LMF by model identification.

Step 5. The LMF may send the meta-data during the model transfer.

Step 6. The UE may activate the model based on the model configuration autonomously or with explicit activation indication from the LMF.

Step 7. The LMF may send model performance monitoring configuration to the UE.

Step 8. The UE performs PRS measurement and obtains the data required for model inference.

Step 9. The UE performs model inference and reports the measurement/location estimate to the LMF.

Step 10-11. The LMF monitors the performance and makes decisions about model activation/deactivation/ updating/switching.

Potential standard impact

AI/ML-based Beam management may introduce the following potential LPP impact:

- AI-related Capability for positioning

- For model training, new or enhanced PRS configuration; configuration and report of CIR report;

- Model identification/ transfer for beam prediction, e.g., Model ID, Meta-data;

- For model inference, configuration and report of predicted location estimate, TOA, LOS/NLOS identification;

- Model monitoring, including configuration and report of performance metric, model activation/deactivation