3GPP TSG RAN WG2 Meeting #122                                                           R2-2305491

# Incheon, Korea, May 22nd – 26th 2023

Agenda item:	7.18.2

Source:	Intel Corporation

Title:	MT SDT mechanism (including configuration, paging, resume and capabilities)

Document for:	Discussion and decision

# Introduction

This document continues discussion on remaining open items to enable MT SDT mechanism focusing on control plane topics associated with configuration, paging, resume and capabilities.

# Discussion

## Configuration for MT-SDT

RAN2 agreements relevant for the discussion of MT-SDT related configurations:

R2#121

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

  - FFS 3a: Check for DVT (if UL data becomes available in UL)

  - 3b: Check for SDT RSRP threshold

  - 3c: Check for TA validation before selecting CG (if applicable)

  - 3d: Check for SSB level RSRP threshold for CG resource (if applicable)

- RBs configured for SDT are common for MO-SDT and MT-SDT

- When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

R2#120

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.

- MT-SDT is data that belongs to bearers that are configured for SDT. The network can only trigger MT-SDT if the data belongs to those bearers.  FFS whether the configuration is MO-SDT or MT-SDT specific

### Cell specific configuration for MT-SDT

Firstly, RAN2 needs to discuss the applicability/usage of sdt-ConfigCommon-r17 which is defined in SIB1 as follows:

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range    OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024,

sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400,

byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000,

byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600,

ms1000, ms2000,ms3000, ms4000, spare7, spare6,

spare5, spare4, spare3, spare2, spare1}

}

Considering previous RAN2 agreements and the MO-SDT and MT-SDT do not need to have different requirements on SDT RSRP threshold, duration of T319a or delay timer for SR, the values provided by Rel-17 MO-SDT could still be applicable. However, there are two potential concerns: 1) how the network can allow the initiation of SDT usage due to MT-SDT only (i.e., without allowing MO-SDT initiated) and 2) SDT data threshold is defined as mandatory present if SDT-ConfigCommonSIB-r17 is signaled. RAN2 only agreed that network should be able to configure only MT-SDT (without MO-SDT RA resources and/or CG-SDT), however scenario described by 1) was not explicitly discussed. One design approach is that when SDT is allowed, both MO-SDT and MT-SDT initiated are possible (dependent on whether UE was configured by network or not). This approach might be too restricted from network and operator point of view. Therefore, it seems preferable that from specification point of view, network has the flexibility to allow resume initiated by both or either of them. The simpler approach seems to define a MT-SDT specific common configuration and later discuss whether ASN.1 should be optimized to only use it when MT-SDT is only allowed, i.e. if MO-SDT and MT-SDT are both allowed, UE could use the common and applicable parameters from SDT-ConfigCommonSIB-r17.

In Rel-17 MO-SDT, a UE configured with SDT can initiate MO-SDT if SIB1 is included in SDT-ConfigCommonSIB-r17 and other SDT specific conditions are met. If SDT-ConfigCommonSIB-r17 is used for MT-SDT, it would be a problem if network only wants to allow resume due to MT-SDT (i.e., not MO-SDT).

- Considering that RAN2 already agreed that “network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT”, to confirm that this agreement applies to both UE-specific and cell-specific SDT configuration. If so, to clarify that for the cell-specific SDT configuration, this would mean that network should be able to allow UEs to initiate resume for MT-SDT only (i.e., without allowing the initiation of resume for MO-SDT).

- A new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) is defined including at least the following IEs from SDT-ConfigCommonSIB-r17: sdt-RSRP-Threshold, sdt-LogicalChannelSR-DelayTimer and t319a. The following describes the expected operation to be defined:

  - When network allows initiation of resume due to MT-SDT and MO-SDT, mt-SDT-ConfigCommonSIB-r18 is signalled but does not include IEs already included in SDT-ConfigCommonSIB-r17 to avoid unnecessary signaling. I.e., UE initiating resume due to MT-SDT also applies the applicable IEs signal in SDT-ConfigCommonSIB-r17).

  - When network only allows initiation of resume due to MT-SDT (and not MO-SDT), network does not signal SDT-ConfigCommonSIB-r17 and all required common IEs are provided via the new mt-SDT-ConfigCommonSIB-r18.

### UE specific configuration for MT-SDT

Legacy define SDT configuration for MO-SDT provided via RRCRelease message as follows:

SDT-ConfigSIB-r17 ::=          SEQUENCE {

sdt-DRB-List-r17          SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL,  -- Need M

sdt-SRB2-Indication-r17   ENUMERATED {allowed}                         OPTIONAL,  -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}   OPTIONAL,  -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }           OPTIONAL   -- Need S

}

#### RB configuration for MT-SDT

Some form of MT-SDT specific configuration needs to be defined in RRCRelease message, considering the agreements previously explained. The following approaches could be discussed on how network indicates which RBs are configured for MT-SDT:

- New MT-SDT specific RB configuration. Which RB is configured for MT-SDT could be enabled as follow by providing different configuration specific for DRBs and SRB2 similarly as it was done in Rel-18. One open question would be whether we also need to have an indication specific to indicate whether ROHC can be continued per cell or RNA as it was done for Rel-17 SDT, as shown in highlighted line.

mt-SDT-Config-rXY    SetupRelease { MT-SDT-Config-rXY }  OPTIONAL		-- Need M

MT-SDT-Config-rXY ::=        SEQUENCE {

mt-SDT-DRB-List-rXY   SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL, -- Need R

mt-SDT-SRB2-Indication-rXY    ENUMERATED {allowed}              OPTIONAL     -- Need R

}

- MO-SDT specific RB configuration is re-used for MT-SDT. Different levels of flexibility could be allowed depending on how the ASN.1 is defined. The following options summarizes key ones:

- Flag indication. This flag could indicate whether any RB configured for MO-SDT can also be used for MT-SDT (e.g., option 1.a) shown below), or whether any DRB or SRB2 configured for MO-SDT are used for MT-SDT (e.g., option 1.b) shown below).

Option 1.a) General MT-SDT flag

mt-SDT-rXY          ENUMERATED {allowed}                        OPTIONAL,

Option 1.b) Separate MT-SDT flag for DRB and SRB2

mt-SDT-DRB-rXY   ENUMERATED {allowed}           OPTIONAL,

mt-SDT-SRB2-rXY  ENUMERATED {allowed}           OPTIONAL

- Sub-set of RBs. This indicates which actual RB configured for MO-SDT is applicable for MT-SDT which would allow finer granularity than option 1. This could be enable e.g., as follows:

mt-SDT-Config-rXY    SetupRelease { MT-SDT-Config-rXY }  OPTIONAL	-- Need M

MT-SDT-Config-rXY ::=       SEQUENCE {

sdt-DRB-List-rXY         SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL,

sdt-SRB2-Indication-rXY  ENUMERATED {allowed}                        OPTIONAL

}

- Definition of Rel-17 SDT specific RB configuration is changed to allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation). This approach requires that a Rel-18 supporting MT-SDT have slightly different understanding of Rel-17 SDT configuration. A new indication would indicate for each RBs whether this UE specific configuration is applicable only for MT-SDT or for both MT-SDT and MO-SDT. If no new indication is added, the default behaviour is aligned to legacy, i.e. applicable only to MO-SDT. Similar options as in previous approach 2) could be considered as shown below:

- Flag indication.

Option 1.a) General MT-SDT flag.

SDT-rXY          ENUMERATED {mt-SDT, mo-mt-SDT}          OPTIONAL,

Option 1.b) Separate MT-SDT flag for DRB and SRB2 (understanding that UE might have different capability support for SDT over DRB(s) and SRB2 due to the corresponding complexity associated and therefore different indication might be preferable.

SDT-DRB-rXY   ENUMERATED {mt-SDT, mo-mt-SDT}           OPTIONAL,

SDT-SRB2-rXY  ENUMERATED {mt-SDT, mo-mt-SDT}           OPTIONAL

- Indication of a sub-set of RBs.

mt-SDT-Config-rXY       SetupRelease {MT-SDT-Config-rXY}  OPTIONAL	-- Need M

mo-mt-SDT-Config-rXY    SetupRelease {MO-MT-SDT-Config-rXY}  OPTIONAL	-- Need M

MT-SDT-Config-rXY ::=       SEQUENCE {

sdt-DRB-List-rXY         SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL,

sdt-SRB2-Indication-rXY  ENUMERATED {allowed}                         OPTIONAL

}

MO-MT-SDT-Config-rXY ::=       SEQUENCE {

sdt-DRB-List-rXY         SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL,

sdt-SRB2-Indication-rXY  ENUMERATED {allowed}                         OPTIONAL

}

In addition, legacy definition of sdt-Config would require some update, e.g. as follows:

sdt-DRB-List Indicates the ID(s) of the DRB(s) that are configured for SDT. If size of the sequence is zero, then the UE assumes that none of the DRBs are configured for SDT. The network only configures MN terminated MCG bearers for SDT. If UE support MT-SDT, the configuration of XX indicates whether the DRB is configured only for MT-SDT or for both MO-SDT and MT-SDT; and absent of configuration of XX indicates that the DRB is configured only for MO-SDT.

sdt-SRB2-Indication Indiates whether SRB2 is configured for SDT or not. If UE support MT-SDT, the configuration of XX indicates whether the SRB2 is configured only for MT-SDT or for both MO-SDT and MT-SDT; and absent of configuration of XX indicates that the SRB2 is configured only for MO-SDT.

Considering that network should be able to configure only MT-SDT, and it is not possible to configure a RB only for MT-SDT, approach 1) or approach 3) would be preferable. Another option is to enable both approach 1) and approach 2), however this seems unnecessary. From signaling point of view, approach 3) seems simpler even though the definition of legacy UE-specific SDT configuration need to be updated. As this is the first time discussing the ASN.1, we suggest for RAN2 to discuss all the possible options. In addition, it might be good to also check on companies preference on how RBs are configured when MO-SDT and MT-SDT are both enabled to a given UE. RAN2 agreed so far that UE shall be able to only be configured with one of them, and if it is configured with both, all SDT RBs should be resumed during any SDT operation (regardless on whether are configured for MO-SDT or MT-SDT. However, RAN2 has not discussed whether the RB list for MO and MT SDT will the same or different when both features are configured.

Re-using MO-SDT specific RB configuration for MT-SDT (as explained in approach 2)) does not seem ideal as it would also require the definition of MT-SDT specific RB configuration (as explained in approach 1)).

Adding MT-SDT specific RB configuration (as explained in approach 1)) is feasible although there might be redundant information with the MO-SDT specific one which could be avoid if the definition of Rel-17 SDT specific RB configuration is changed to allow MT-SDT only (as explained in approach 3). However, approach 3) requires changes on legacy MO-SDT definition when UE support MT-SDT.

- When network configures a UE with both Rel-17 MO-SDT and Rel-18 MT-SDT, to discuss whether RB(s) should have a common configuration: option (1) any RB configured SDT can perform both MO-SDT and MT-SDT, or option (2) RBs can be configured differently for MO-SDT and MT-SDT (e.g., one RB can be configured for MT-SDT and MO-SDT, while other RB is only configured for MT-SDT (or MO-SDT)).

- For the UE specific MT-SDT configuration provided in RRCRelease message, to discuss whether to define approach (1) a new/independent MT-SDT specific RB configuration, approach (2) new signaling indicate the MO-SDT specific RB configuration to be re-used for MT-SDT or approach (3) extend the definition of Rel-17 SDT specific RB configurations to also allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation).

## Paging for MT-SDT

RAN2 agreements relevant paging with MT-SDT indication:

R2#121

- Include a one-bit indication in paging to trigger MT-SDT.  We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

R2#120

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed. FFS what the indication will be called.  FFS signalling details.

The new MT-SDT indication could be defined as part of legacy Paging message as follows:

- Part of legacy paging record, for example, as shown below.

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList    OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING        OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs    OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700   OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17      OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}         OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...,

[[mt-SDT-r18                         ENUMERATED {true}       OPTIONAL    -- Need N]]

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

- New pagingRecordList defined as a non-critical, for example, as shown below by the text in green.

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList    OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING        OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs    OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700   OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17      OPTIONAL, -- Need N

nonCriticalExtension                Paging-v1800-IEs         OPTIONAL

}

Paging-v1800-IEs ::=                SEQUENCE {

pagingRecordList-v1800              PagingRecordList-v1800   OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}              OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingRecordList-v1800 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700 PagingRecord-v1800 ::=              SEQUENCE {

pagingCause-r18                     ENUMERATED {mt-sdt}      OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

We understand that option 2) shown above might be preferable to reduce signaling overhead (as option 1 would introduce additional 24bits which does not seem justifiable for paging). Moreover option 2) would also align with legacy approach used to convey “voice” paging cause reason.

- MT-SDT indication in paging is defined as a non-critical extension of a new paging record list, similarly as it is done for voice paging Cause (as shown in above ASN.1).

## Resume due to MT-SDT

The latest RAN2 agreements associated to the initiation of resume due to MT-SDT trigger:

R2#121

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

- When UE resumes for MT-SDT, UE resumes all RBs configured for SDT. RBs configured for SDT are common for MO-SDT and MT-SDT

- To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

### Resume cause value

The following figure summarizes our view on which resume cause value should be used when MT-SDT conditions are not met. Considering the agreements already taken, the only open question is which cause value is set when UE initiates resume due to MT-SDT trigger and MT-SDT conditions are not met.

Figure 1. Resume cause value to use when detecting MT-SDT indication for a given UE

In our understanding, network should know that this UE cannot perform SDT (understanding that during an SDT session, UE would not provide any measurement updates). Therefore, resume cause should avoid any ambiguity from network point of view. On summary, when MT-SDT conditions are not met when initiating SDT for MT-SDT, the resumeCause should be set to MT-Access as if UE responded to any legacy paging.

- When resume is initiated due to MT-SDT indication and UE does not meet MT-SDT conditions, resume cause is set to MT-Access so network can treat this as a legacy resume (instead than an SDT procedure).

## Capabilities for MT-SDT

A new radio capability per UE will be defined to identify UEs supporting MT-SDT, however this new feature could be defined in different ways:

- MT-SDT capability is defined as independent/standalone feature to be used with any RB

- MT-SDT capability is defined as independent/standalone feature to be used with any DRB and another MT-SDT-SRB capability is defined when MT SDT is to be sent over SRB.

- MT-SDT capability is defined to be dependent to the support and/or configuration of Rel-17 SDT features, i.e. RA-SDT, CG-SDT and/or SRB-SDT

For sure, UE needs to inform the network when it supports MT-SDT feature. The open questions are:

- Q1) how to define a capability to support MT-SDT: (1) for any RB or (2) for DRB vs for SRB

- Q2) What is the relation between Rel-18 MT-SDT and Rel-17 SDT features (RA-SDT, CG-SDT and/or SRB-SDT), if any

When UE initiates MT-SDT, UL/DL SDT is allowed during an ongoing SDT session. Therefore, it seems simpler to define the following UE behaviour:

- If UE support Rel-18 MT-SDT for DRB, the UE shall also support Rel-17 RA-SDT

- If UE support Rel-18 MT-SDT for SRB2, the UE shall also support Rel-17 RA-SDT and Rel-17 SRB-SDT

It is important to keep in mind that it would be up to network decision whether to configure UE’s usage of resume initiated via MO-SDT or MT-SDT.

- A new optional radio capability signaling is defined to indicate UE’s support of Rel-18 MT-SDT.

  - This MT-SDT radio capability is defined separately for DRB vs SRB2.

    - If a UE supports Rel-18 MT-SDT for DRB, this UE shall also support Rel-17 RA-SDT.

    - If a UE supports Rel-18 MT-SDT for SRB2, this UE shall also support Rel-17 RA-SDT and Rel-17 SRB-SDT.

# Conclusion

The observations captured are the following:

Observation 1.	In Rel-17 MO-SDT, a UE configured with SDT can initiate MO-SDT if SIB1 is included in SDT-ConfigCommonSIB-r17 and other SDT specific conditions are met. If SDT-ConfigCommonSIB-r17 is used for MT-SDT, it would be a problem if network only wants to allow resume due to MT-SDT (i.e., not MO-SDT).

Observation 2.	Re-using MO-SDT specific RB configuration for MT-SDT (as explained in approach 2)) does not seem ideal as it would also require the definition of MT-SDT specific RB configuration (as explained in approach 1)).

Observation 3.	Adding MT-SDT specific RB configuration (as explained in approach 1)) is feasible although there might be redundant information with the MO-SDT specific one which could be avoid if the definition of Rel-17 SDT specific RB configuration is changed to allow MT-SDT only (as explained in approach 3). However, approach 3) requires changes on legacy MO-SDT definition when UE support MT-SDT.

The proposals captured are the following:

Proposal 1.	Considering that RAN2 already agreed that “network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT”, to confirm that this agreement applies to both UE-specific and cell-specific SDT configuration. If so, to clarify that for the cell-specific SDT configuration, this would mean that network should be able to allow UEs to initiate resume for MT-SDT only (i.e., without allowing the initiation of resume for MO-SDT).

Proposal 2.	A new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) is defined including at least the following IEs from SDT-ConfigCommonSIB-r17: sdt-RSRP-Threshold, sdt-LogicalChannelSR-DelayTimer and t319a. The following describes the expected operation to be defined:

Proposal 2.1.	When network allows initiation of resume due to MT-SDT and MO-SDT, mt-SDT-ConfigCommonSIB-r18 is signalled but does not include IEs already included in SDT-ConfigCommonSIB-r17 to avoid unnecessary signaling. I.e., UE initiating resume due to MT-SDT also applies the applicable IEs signal in SDT-ConfigCommonSIB-r17).

Proposal 2.2.	When network only allows initiation of resume due to MT-SDT (and not MO-SDT), network does not signal SDT-ConfigCommonSIB-r17 and all required common IEs are provided via the new mt-SDT-ConfigCommonSIB-r18.

Proposal 3.	When network configures a UE with both Rel-17 MO-SDT and Rel-18 MT-SDT, to discuss whether RB(s) should have a common configuration: option (1) any RB configured SDT can perform both MO-SDT and MT-SDT, or option (2) RBs can be configured differently for MO-SDT and MT-SDT (e.g., one RB can be configured for MT-SDT and MO-SDT, while other RB is only configured for MT-SDT (or MO-SDT)).

Proposal 4.	For the UE specific MT-SDT configuration provided in RRCRelease message, to discuss whether to define approach (1) a new/independent MT-SDT specific RB configuration, approach (2) new signaling indicate the MO-SDT specific RB configuration to be re-used for MT-SDT or approach (3) extend the definition of Rel-17 SDT specific RB configurations to also allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation).

Proposal 5.	MT-SDT indication in paging is defined as a non-critical extension of a new paging record list, similarly as it is done for voice paging Cause (as shown in above ASN.1).

Proposal 6.	When resume is initiated due to MT-SDT indication and UE does not meet MT-SDT conditions, resume cause is set to MT-Access so network can treat this as a legacy resume (instead than an SDT procedure).

Proposal 7.	A new optional radio capability signaling is defined to indicate UE’s support of Rel-18 MT-SDT.

Proposal 7.1.	This MT-SDT radio capability is defined separately for DRB vs SRB2.

Proposal 7.1.1.	If a UE supports Rel-18 MT-SDT for DRB, this UE shall also support Rel-17 RA-SDT.

Proposal 7.1.2.	If a UE supports Rel-18 MT-SDT for SRB2, this UE shall also support Rel-17 RA-SDT and Rel-17 SRB-SDT.