3GPP TSG-RAN WG2 #122	R2-2305549

Incheon, Korea, 22nd May – 26th May 2023

Agenda Item:	7.9.3

Source:	Ericsson

Title:	Discussion on Inter-gNB Service Continuity

Document for:	Discussion, Decision

# 1	Introduction

The following agreements were made in the RAN3#119 meeting for the decision on the target U2N relay UE in a d2i and i2i path switch scenario:

During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message.

At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message.

In addition, regarding the lossless delivery issue, the following agreements were made:

Agreements:

For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305.  Other solutions are not pursued.

For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

We discuss here further details on the above proposals and express our views accordingly in this contribution

# 2	Discussion

## 2.1 Source vs Target gNB decision on Target U2N relay UE

The decision on the target U2N relay UE is applicable to the d2i and i2i scenarios.

During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message.

Based on the above agreement, RAN3 has concluded that the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO request. Hence, it would be reasonable for RAN2 to confirm the agreement and proceed further.

- For d2i and i2i scenarios, RAN2 to acknowledge RAN3’s agreement that the source gNB sends a list of candidate relay UEs belonging to the same target cell to the target gNB with the target gNB making the final decision.

Further, the Rel-17 reporting mechanism can be reused and no enhancements are necessary.

- Reuse the Rel-17 measurement reporting framework i.e., report consists of candidate U2N relay UE IDs, candidate U2N relay UE’s serving cell ID and the sidelink measurement quantity information.

## 2.2 Triggering the U2N Relay UE to CONN state

The triggering of the U2N relay UE into the CONN state is relevant for the d2i and i2i scenarios.

The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state should also be applicable to the Rel-18 inter/intra-gNB scenarios as a baseline for single-path relay.  Other mechanisms are not excluded if an issue is found with the baseline.

- The above agreement was made in the last meeting with the addition of the highlighted text. There was also some discussion about using a paging-based solution in addition to the trigger from the remote UE to transit the U2N relay UE in IDLE/INACTIVE to the CONN state.

- Although we foresee some potential benefits from the paging-based solution especially for inter-gNB path switch scenarios, in terms of reducing the latency of the overall path switch procedure by potentially transitioning the target U2N relay UE to the CONN state before remote UE initiates the PC5 communication. The following is our analysis:

- For the target U2N relay UE in the INACTIVE state, the paging-based solution is already feasible based on Rel-17 specifications and with no changes required to the legacy paging mechanism. That is, if the gNB can retrieve the target U2N relay UE’s context, it can page (using RAN-paging) the target U2N relay UE to initiate the random-access procedure. This can also be done before the path switch command is sent to the remote UE and as a result, potentially, the target U2N relay UE is already in the CONNECTED state when the remote UE initiates PC5 communication for relaying.

- For the target U2N relay UE in the IDLE state, additional specification work is required to support this state, and this would involve cross-WG interactions (SA2/RAN3). Given the current workload, such solutions should be down prioritized.

- For inter-gNB d2i and i2i scenarios, the following should be agreed about the paging-based mechanism to transit the target U2N relay UE in IDLE/INACTIVE state to the CONNECTED state:

  - In RRC_INACTIVE state, RAN2 to confirm that it is up to gNB implementation to page the target U2N relay UE before the path switch command is sent to the remote UE.

  - In RRC_IDLE state, RAN2 to not pursue the enhancements required for the paging solution.

## 2.3 UL/DL Lossless Delivery

Agreements:

For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305.  Other solutions are not pursued.

For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

The above agreement was made in the previous meeting with respect to the UL/DL lossless delivery. This is applicable to i2d or i2i scenarios. The solutions for UL/DL in the agreement are from the summary document [1]

The different solutions for downlink and uplink are discussed separately. Given the workload of this WI, in general, we prefer solutions which have minimum specification impact and the following analysis takes into account the same.

### 2.3.1 DL Lossless Delivery

The following solutions are being considered for DL based on [xx]

| Solution | Details | Spec Impact |
| --- | --- | --- |
| D4: Enhanced Data Forwarding from source gNB to Target gNB per Target gNB request (legacy PDCP status report based) | Target gNB relies on PDCP status report sent from the remote UE after path switch. Based on this information, the target gNB can request the source gNB to forward the missing packets (not forwarded before). | Cross WG interaction i.e., with RAN3 and would impact RAN3 specification. High workload |
| D3: A PDCP status report sent from Remote UE to the source gNB | Source gNB triggers the remote UE to send a PDCP status report to the source gNB before the source gNB performs SN status transfer to the target gNB. The timeline for triggering PDCP status report: - Upon receiving the path switch command - An explicit trigger before path switch command - Measurement reporting event triggers status report | Would require update to the PDCP specification to include a new trigger for PDCP SR in the UL. Medium workload |
| D5: Proactive Data Forwarding from Source gNB to Target gNB | Allow source gNB to forward all the buffered data to the target gNB without receiving the request from the target gNB | No specification impact, left to source/target gNB implementation. |

Solution D3 is a reasonable approach to make the source gNB aware of the DL status and provide lossless data delivery. However, the options as listed to trigger the PDCP status report in the UL will not be sufficient:

- Upon receiving the path switch command

The general understanding is that the UE/gNB will stop all CP/UP transmissions after receiving and transmitting the path switch command respectively. Hence, the PDCP status report cannot be triggered. In addition, the deteriorating PC5 channel conditions may result in the remote UE being unable to transmit the PDCP SR in the UL.

- Explicit trigger before the path switch command

It is unclear how this trigger would work. One option is when the remote UE sends to the measurement report, the gNB will first trigger the status report in the UL then, having received it, will send the path switch command. In which case, this might delay the transmission of the path switch command resulting in a path switch failure because of the deteriorating PC5 channel conditions.

- Measurement reporting event triggers a status report

With this trigger, the lossless issue persists because from the time of sending the PDCP SR and receiving the path switch command, the remote UE can continue to receive UP transmissions. Further, if CP transmissions are prioritized at the relay UE, some packets might still be leftover at the relay UE, but the (already sent) PDCP SR might not reflect this information. In addition, the lossless issue is only valid for an indirect path and if the PDCP SR is tied to event-based measurement reporting, it would also be triggered on the direct path which is unnecessary.

Hence, RAN2 should further study new triggers for sending the PDCP SR to the source gNB. Solution D3 will have specification impact as the new trigger condition would need to be captured in the PDCP spec.

Solution D4 is an optimization with the additional overhead of a request from the target gNB to the source gNB and believe that solution D5 would be sufficient to address the issue of lossless delivery in the DL by handling from an implementation standpoint. The arguments from companies against solution D5 is that the gNB would need to store a lot of packets for a longer time i.e., increased buffer requirement. However, in Solution D4, the packets would have to be stored for even longer (time) because the request from the target gNB can be triggered only after the successful path switch and reception of the PDCP SR in the UL.

Furthermore, Solution D5 can be achieved by implementation without any specification impact whereas Solution D4 would require interaction with RAN3. Keeping in mind the WI timeline, it would be best to work on solutions with minimum cross-WG (i.e., RAN3) impact.

Therefore, RAN2 can down select between Solution D3 and Solution D5 and try to find a suitable trigger for the PDCP SR in D3.

- For DL lossless delivery, down select between:

Solution D3: the PDCP SR can be sent to the source gNB and it can perform the required data forwarding to the target gNB. FFS the trigger condition for the PDCP SR to the source gNB and,

Solution D5: Proactive Data Forwarding between source gNB and target gNB with no specification impact.

### 2.3.2 UL Lossless Delivery

| Solution | Details | Spec Impact |
| --- | --- | --- |
| U3: Remote UE’s PDCP retransmission based on DL PDCP Status Report from target gNB | Target gNB sends the PDCP SR to the remote UE to trigger the retransmission of packets that were not received at the target gNB. | No specification impact foreseen. Target gNB sends the PDCP SR based on implementation and the remote UE stores the packets in the buffer based on the discardtimer value |
| U5: Source Relay UE continues to transmit UL data to source gNB | Keep the source relay UEs configuration long enough such that the relay UE can continue to transmit the remote UE’s packets | No spec impact foreseen, up to gNB implementation |

Both U3 and U5 are feasible and can achieve the UL lossless delivery.

The transmission of the PDCP SR in the DL is up to network implementation. Hence, for Solution U3, the target gNB can send the PDCP SR to the remote UE after the successful path switch. The remote UE based on the PDCP SR can know which packets are missing and can perform the retransmission of those packets. In terms of how much data would need to be buffered by the UE, this would depend on setting of the discardtimer value at the PDCP-layer.

For Solution U5, although feasible, we there might be an issue with interaction between the source and the target gNB. In addition, different implementations can have different impacts on the performance with some leading to lossy delivery of UL packets. In addition, this approach also keeps the resources of the source gNB for longer than necessary. Hence, we believe Solution U3 is a more robust solution.

- Support Solution U3 (i.e., remote UEs PDCP retransmission is based on DL PDCP SR from the target gNB) for UL lossless data delivery. No specification impact is foreseen

# 3 Conclusion

Based on the discussion in the previous sections we propose the following:

- For d2i and i2i scenarios, RAN2 to acknowledge RAN3’s agreement that the source gNB sends a list of candidate relay UEs belonging to the same target cell to the target gNB with the target gNB making the final decision.

- Reuse the Rel-17 measurement reporting framework i.e., report consists of candidate U2N relay UE IDs, candidate U2N relay UE’s serving cell ID and the sidelink measurement quantity information

- For inter-gNB d2i and i2i scenarios, the following should be agreed about the paging-based mechanism to transit the target U2N relay UE in IDLE/INACTIVE state to the CONNECTED state:

- In RRC_INACTIVE state, RAN2 to confirm that it is up to gNB implementation to page the target U2N relay UE before the path switch command is sent to the remote UE.

- In RRC_IDLE state, RAN2 to not pursue the enhancements required for the paging solution.

- For DL lossless delivery, down select between:

Solution D3: the PDCP SR can be sent to the source gNB and it can perform the required data forwarding to the target gNB. FFS the trigger condition for the PDCP SR to the source gNB and,

Solution D5: Proactive Data Forwarding between source gNB and target gNB with no specification impact.

- Support Solution U3 (i.e., remote UEs PDCP retransmission is based on DL PDCP SR from the target gNB) for UL lossless data delivery. No specification impact is foreseen

# 4 References

[1] R2-2304305, Summary of [AT121bis-e][432] Candidate Solutions for Lossless Delivery, NEC