3GPP TSG-RAN WG2 #122	R2-2305550

Incheon, Korea, 22nd May – 26th May 2023

Agenda Item:	7.9.4

Source:	Ericsson

Title:	Discussion on Multipath Relays

Document for:	Discussion, Decision

# 1 Introduction

The following list of proposals could not be treated in the last (RAN2#121bis-e) meeting due to a lack of time or consensus.

Agreements:

WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE

Proposals:

FFS if there are cases where the configuration of non-split SRBs over indirect path is useful

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case

In this contribution, we discuss the proposals above highlighted in yellow and present our views.

# 2 Discussion

We discuss the above proposals relating to the two scenarios in different sections.

## 2.1 Proposals for Scenario-1

Control Plane Aspects:

FFS if there are cases where the configuration of non-split SRBs over indirect path is useful

For the FFS, some companies argue that this can be left to network implementation, and we need not capture any restrictions on how to configure SRB1/SRB2. From the network’s point of view, following the principles of MR-DC, CP signaling over SRB is usually configured on the most robust and reliable path. In addition, the network can deliberate at any point of time which of the two paths (i.e., direct, and indirect path) is more reliable. As a result, the network will configure all the SRBs to be transmitted on that path either the direct or indirect path. Configuring the SRB1/SRB2 on different paths with one being more reliable than the other is not a sensible configuration and there is no scenario in which such a configuration is required.

- SRB1 and SRB2 are always configured on the same path.

On which bearer type should be configured for SRBs, we believe that the non-split SRBs should always be configured on the direct path. This is because the direct path will always be more robust and reliable as opposed to the indirect path. However, there are situations where the indirect path can be better for e.g., in cell-edge cases. In those scenarios, a split SRB with duplication can be configured for the UE.

- Non-split SRB1/2 can only be configured over the direct path.

- Split bearer SRB1/2 can only be configured with duplication on a per-SRB basis.

WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE

For the above WA, with the understanding that the network up on deciding to move the relay UE to the IDLE/INACTIVE state, can then release the multipath configuration at the remote UE, the above WA can be confirmed.

- Confirm the WA that for a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/INACTIVE. No spec impact is foreseen.

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case

For the above FFS on if a Rel-17 relay UE is supported for use in a multipath connection, RAN2 should wait for SA2’s reply on this issue.

- For the FFS on if a Rel-17 relay UE is supported for use in a multipath connection, RAN2 should wait for SA2’s reply to the LS.

One of the basic aspects related to the setup of a multipath configuration i.e., addition of either an indirect path (on top of a direct path) or a direct path (on top of an indirect path) that is yet to be discussed are the events/conditions under which the remote UE can request for a multipath configuration. For example, for measurement events, in an indirect path addition, the remote UE can be configured with the Rel-17 measurement configurations used in the case of a path switch scenario i.e., Event Y2 the candidate relay UE becomes better than a threshold. Similar measurement events can be configured for the addition of the direct path.

- RAN2 to discuss the measurement events/conditions under which a multipath connection can be setup i.e., indirect/direct path addition.

User Plane Aspects:

For a multipath UE, unlike legacy procedures for UL/SL prioritization, the data carried over the UL and SL is all related to the same DRB either in the form of duplication or data split. In mode-1 scheduling, the multipath UE can request for SL resources and at the same time, the UE would also need to trigger the BSR to the gNB for request for UL resources. Although, legacy procedures for UL/SL prioritization are defined, these would lead to additional delays, issues with reordering and possible expiry of timers. Thereby leading to unnecessary retransmissions and even loss of packets. As a result, possible solutions should be studied to handle such prioritizations.

- RAN2 to study the issue of UL/SL BSR prioritization with mode-1 scheduling in a multipath scenario.

Similarly, the legacy procedures for UL/SL prioritization might not be valid here for the reasons as described above. Hence, it should also be considered for discussion.

- RAN2 to study the UL/SL prioritization procedure for UP transmission in a multipath scenario.

# 3 Conclusion

Based on the discussion in the previous sections we propose the following:

- SRB1 and SRB2 are always configured on the same path.

- Non-split SRB1/2 can only be configured over the direct path.

- Split bearer SRB1/2 can only be configured with duplication on a per-SRB basis.

- Confirm the WA that for a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/INACTIVE. No spec impact is foreseen.

- For the FFS on if a Rel-17 relay UE is supported for use in a multipath connection, RAN2 should wait for SA2’s reply to the LS.

- RAN2 to discuss the measurement events/conditions under which a multipath connection can be setup i.e., indirect/direct path addition.

- RAN2 to study the issue of UL/SL BSR prioritization with mode-1 scheduling in a multipath scenario.

- RAN2 to study the UL/SL prioritization procedure for UP transmission in a multipath scenario.

# 4 References