3GPP RAN WG2 Meeting #122	R2-2305642

# Incheon, Korea, May 22-26, 2023

Agenda Item:	7.2.3

Source:	InterDigital Inc.

Title:	Discussion on RAT dependent integrity

Document for:	Discussion

# 1. Introduction

This contribution discusses RAT dependent integrity related open issues.

# 2. Discussion

## 2.1 Error distributions bounding techniques

In the LS reply from RAN1 [1], details related to error distributions are described. In the LS reply, the discussion is limited to the error sources that follow the Gaussian distribution since it is not clear whether the following agreement made in RAN2#112 is applicable to Uniform distribution.

Agreements:

RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

As indicated in the Table 6.1.1-2 in TR 38.859, error sources such as TRP/ARP location error, inter-TRP synchronization error can follow either Gaussian or Uniform distribution. It is not clear whether a single-Gaussian based bound can bound the Uniform distribution. RAN2 should study whether paired overbounding [2, 3] can be used to bound the Uniform distribution. As explained in [2], paired overbounding technique can be characterized by a Gaussian distribution with the standard deviation σ and its mean is shifted by ±μ to “bound” the unknown distribution. It is typically used for bounding the distribution which cannot be sufficiently bounded by a single Gaussian distribution. If the use of paired overbounding is clarified, RAN2 can proceed with defining the value range for the parameters of the bound. Thus the following proposal is made:

Proposal 1: RAN2 studies whether paired overbounding can bound the Unfirom distribution for error sources that follow Uniform distribution for error sources TRP/ARP location error and inter-TRP synchronization error.

## 2.2 Value range

In the LS reply from RAN1 [1], RAN1 shares views on on mean and standard deviation value if a Gaussian distribution is used as the bound for the error sources which follow the Gaussian distribution.

At least zero can be considered as the mean value for the Gaussian distribution based bound. It is not clear whether the error source can have non-zero mean.

From RAN1 perspective, the value ranges of existing fields corresponding to quality information (e.g., nr-TimingQuality, rtd-Quality-r16) and uncertainty information (e.g., LocationUncertainty-r16) can be reused as a reference to derive the value ranges for the parameters (e.g., standard deviation) for the overbound Gaussian distribution for the error sources listed in Table 6.1.1-2 in TR 38.859.

Following the information given by RAN1, RAN2 should be able to study the value range for the error sources as RAN2 has experience defining the value range for the parameters of the bounds derived for GNSS integrity during Rel. 17. Thus, the following proposal is made:

Proposal 2: RAN2 studies the value range for the Gaussian distribution based bound

## 2.2 On feasibility of computation of integrity

During RAN2#121bis-e, there was a discussion on whether intregrity can be computed for some positioning methods due to the lack of error sources related to the positioning methods [4]. It should be noted that as shown in Table 6.1.1-1 in TR 38.859 [5], shown in Table 1, error sources are identified for all UE based or UE assisted positioning methods. Thus, uncertainties for each positioning methods can be identified which makes computation of integrity feasible for the positioning methods. Thus, the following proposal is made.

Proposal 3: Based on error sources identified during the Rel-18 study, support positioning integrity for all positioning methods (DL-TDOA, UL-TDOA, multi-RTT, UL-AoA, DL-AoD)

Table 1 “Table 6.1.1-1: Error sources for LMF-based and UE-based positioning integrity modes” from TR 38.859

| Positioning Integrity Mode | DL TDOA | UL TDOA | Multi-RTT | UL AoA | DL AoD |
| --- | --- | --- | --- | --- | --- |
| LMF-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - RSTD measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - RTOA measurement - TRP location - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time.) | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement - TRP location | - Angle of arrival measurement - TRP location - ARP location (e.g., ARPLocationInformation in TS 38.455 [17]) | - TRP location - DL-PRS RSRPP of the first path or RSRP |
| UE-based (as defined in Table 9.4.1.1.1 in TR 38.857 [2]) | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) - Inter-TRP synchronization (e.g., NR-RTD-Info in TS 37.355 [16]) |  |  |  | - TRP location (e.g., NR-TRP-LocationInfo in TS 37.355 [16]) |

## 2.2 Residual Risk, IRallocation and Correlation Times

During RAN2#121bis-e, residual risk, IRallocation and correlation times were identified as additional integrity parameters that may require studies for all error sources [6]. RAN2 should study whether the parameters are applicable for the error sources identified during the Rel-18 study. For example, it is not clear whether residual risk and/or integrity corelation time can be evaluated for TRP location error. The following proposals are made.

Proposal 4: RAN2 to discuss whether to introduce Residual Risk and the IRallocation for RAT-dependent integrity.

Proposal 5: RAN2 to discuss whether to introduce the Integrity Correlation Times for RAT-dependent integrity.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: RAN2 studies whether paired overbounding can bound the Unfirom distribution for error sources that follow Uniform distribution for error sources TRP/ARP location error and inter-TRP synchronization error.

Proposal 2: RAN2 studies the value range for the Gaussian distribution based bound

Proposal 3: Based on error sources identified during the Rel-18 study, support positioning integrity for all positioning methods (DL-TDOA, UL-TDOA, multi-RTT, UL-AoA, DL-AoD)

Proposal 4: RAN2 to discuss whether to introduce Residual Risk and the IRallocation for RAT-dependent integrity.

Proposal 5: RAN2 to discuss whether to introduce the Integrity Correlation Times for RAT-dependent integrity.

# 4. References

[1] R2-2304614, “Reply LS to RAN2 on error source distributions, ” RAN2#122, Incheon, Korea, May, 2023.

[2] R2-2105985, “Guiding framework on integrity concepts for A-GNSS positioning, ” ESA, RAN2#114-e, May 2021.

[3] R2-2108474, “Discussion on GNSS Integrity Assistance Data,” Swift Navigation, Ericsson, Mitsubishi Electric Corporation , RAN2#115-e, Aug. 2021.

[4] R2-2304460, “Draft LS to RAN1 on RAT-dependent positioning integrity,” RAN2#121b-e, April. 2023.

[5] TR 38.859, “Study on expanded and improved NR positioning, ” TR 38.859, v 18.0.0., Dec. 2022.

[6] R2-2304193, “Summary of AI 7.2.3 on RAT-dependent integrity,” vivo, RAN2#121b-e, April. 2023.