3GPP TSG-RAN WG2 #122	R2-2305829

May 22 – 26, 2023

Agenda Item:	7.5.4.2

Source:	Ericsson

Title:	Discussion on PDU Discard

Document for:	Discussion, Decision

# Introduction

In Rel-18 XR WID [1] it was decided to specify discarding related XR capacity enhancements:

Specify the enhancements related to capacity: - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

At RAN2#121 the following agreement on PDU Discard and PDU Set Importance (PSI) was made:

- RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion. FFS for other cases.

At RAN2#121bis-e the following agreement on PDU Discard was made:

- 2: PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

In this contribution we share our view on PDU Discard.

# Discussion on PDU Set Discard

## 2.1 PDU Set timer-based discard

To initiate the discard discussion it should be noted that PDCP SDU timer-based discarding is always an option to avoid building up a queue of outdated data in the UE. An extension to this is PDU Set timer-based discard which can be a useful tool to reduce and limit the amount of PDU sets delivered to the receiver when the PDU Set delay budget has already been consumed. By avoiding those transmissions of PDU Sets that are not useful for the receiver the network can potentially save network resources and increase the overall capacity in the network. As described in [2], proactive discarding based on PDU set size, PDU set delay budget and bit rate information is a far more useful tool to achieve those capacity gains (see also figure 1). However, timer-based solutions may achieve a similar performance if the timer values can be quickly adapted depending on the NW circumstances. Utilizing timer adjustments together with the PDU Set Importance (PSI) can offer a broad flexibility to adapt to the different network situations as well as considering the different priority levels that the PDU sets may have.

Figure 1 Satisfied fraction of UEs when two different PDU Set dropping mechanisms are used: dropping based on timers (configured at PSDB value) or prediction.

The purpose of PDU Set discard is to reduce network wasting resources and can be achieved with a fast and flexible PDU Set timer-based discard solution

In the same way as with proactive discarding there are several factors that the network may consider when configuring the PDU Set discarding and discard timers. Those factors could for example be, network congestion, UE bit rate and traffic requirements such as PSDB and PSI. Another possible factor is the PSIHI, which has been defined by SA2 as information that indicate if an application can use the PDU Set even if it has not been fully transmitted. The PSIHI is information that can give an indication of how large impact lost data will have on the perceived quality but it must be emphasized that lost data is always unwanted from the application perspective. Thus the PSIHI is not a requirement for doing PDU Set discarding and independent of the value of the PSIHI the network can decide to do PDU Set discarding based on the other previously mentioned factors with the goal to reduce network resources wastage and increase the overall XR capacity in the network.

Network can take into account several factors when deciding to do PDU Set discarding and set the discard timer values, e.g. network congestion, UE bit rate, PSDB, PSI and PSIHI.

UE PDU Set discard is used by the UE if it is configured by the network.

When PDU Set discard is configured all the PDCP SDUs related to one PDU Set are discarded if one PDCP SDU discard timer in that PDU Set expires.

When PSIs are associated to the PDU sets each PSI level may be considered differently when in congestion. This could be done by associating the PSI level to one or more PDU set discard timer values. This approach can achieve all the benefits of PSI based discarding, i.e. to avoid wasting resources on data not preferred to transmit. Other discarding approaches, such as trying to couple discarding between different PDU Sets based on their PSI value, is not wanted for several reasons. First SA4 has said that there will be complicated and uncertain dependencies in future encodings. Second, as it has been shown in earlier evaluation results [2] and [3] considering dependencies between PDU Sets is complex and can even be detrimental to the capacity.

With a simple PSI discard timer solution there could, for example, be one value used in “normal” network conditions and a second value used when the network becomes congested. The network should then indicate which value to use at different times, e.g. when congestion happens the network could decide to configure a longer timer for higher PSI level and a shorter timer for lower PSI level. To apply new PDU set discard timer values the simplest option would be to perform RRC reconfiguration.

RRC can configure different discard timer values for each PSI level

However, the cases in which values are likely to be modified is when the network is congested or there are, in overall, capacity problems. In these cases, sending RRC signaling is not always the preferred option as not only the congestion level may be high, but also the interference and the probability of transmission failures. Thus, it is preferable that the network already provides in advance, through RRC, all the configurations related to PDU set discard timer for different scenarios. That means that the network provides the PDU set discard timer values to be used in the different congestion scenarios that may happen.

RRC can configure the PDU set discard timer with at least 2 timer values which the UE will apply when no PSI levels are configured by the network.

If the UE cannot identify the PSI level of the PDU Sets, the UE applies the same timer values as when no PSI levels are configured by the NW, if the timer value was configured.

If the UE supports PSI identification and can identify the PSI levels, the NW may configure PSI levels so that:

      - Each PSI level may be associated with at least 2 different PDU set discard timer values.

      - PSI levels are defined in order of importance: the first PSI level is the most important while the last defined PSI is the least important.

As indicated by SA2, PSI levels may be useful as an input for discarding PDU sets when the network is, for instance, congested. Obviously, in no congestion situations, the PDU set discard timer may have some values, while in congestion situations, the network may choose another value. The UE cannot know when the network is congested and, therefore, it needs to be the network indicating to the UE when a certain PSI level should change the PDU Set discard timer value.

As indicated above, RRC is not the preferred method to update a configuration when the network is having capacity limitations. If the network provides a complete configuration from the start, it is a matter of activating and selecting the parameters. This can be done with Control Elements/PDUs. Since PDU Set discard timer is implemented in the PDCP layer, the best option is to use PDCP Control PDUs for this purpose instead of DCIs or MAC CEs.

PDCP CEs are used to indicate which PDU Set discard timer value is applied.

PDCP CE may, in addition, also indicate the PSI and PDU set discard timer value when PSIs are used

## 2.2 Lower layer discarding and receiver stalling

In existing PDCP SDU discard procedures it is specified that PDCP signals to lower layers if discarding has taken place. This is done to enable lower layers to discard any data that has not yet started to be transmitted, e.g. RLC SDUs where no related RLC PDU has started to be transmitted. However, any data that has started to be transmitted will not be discarded. There is no reason to change this behavior as the lower layer data that has started transmission is generally only a small fraction of the total PDU Set data. As has been shown in simulations (see figure 1) the gain in XR capacity comes from discarding the major part of the PDU Set. The benefit of discarding only a fraction of a RLC PDU is so low that it is not worth the cost and complexity of introducing such.

New mechanism to discard packets in lower layer is not introduced

It has been claimed that signaling needs to be introduced from the transmitter to the receiver which indicates discarding has been done, to avoid potential stalling because of SN gaps. However as we have already explained above there should be no lower layer discarding of packets in flight and thus this is not a problem that will happen or need to be solved. Already in legacy discarding there is a working solution where the transmitter is rebuilding the SN space when doing discarding. There will thus be no SN gaps on the receiver side.

Discard signaling from transmitter to receiver is not introduced

# 4 Conclusion

In the previous sections we made the following observations:

Observation 1	The purpose of PDU Set discard is to reduce network wasting resources and can be achieved with a fast and flexible PDU Set timer-based discard solution

Observation 2	Network can take into account several factors when deciding to do PDU Set discarding and set the discard timer values, e.g. network congestion, UE bit rate, PSDB, PSI and PSIHI.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE PDU Set discard is used by the UE if it is configured by the network.

Proposal 2	When PDU Set discard is configured all the PDCP SDUs related to one PDU Set are discarded if one PDCP SDU discard timer in that PDU Set expires.

Proposal 3	RRC can configure different discard timer values for each PSI level

Proposal 4	RRC can configure the PDU set discard timer with at least 2 timer values which the UE will apply when no PSI levels are configured by the network.

Proposal 5	If the UE cannot identify the PSI level of the PDU Sets, the UE applies the same timer values as when no PSI levels are configured by the NW, if the timer value was configured.

Proposal 6	If the UE supports PSI identification and can identify the PSI levels, the NW may configure PSI levels so that:

1.	Each PSI level may be associated with at least 2 different PDU set discard timer values.

2.	PSI levels are defined in order of importance: the first PSI level is the most important while the last defined PSI is the least important.

Proposal 7	PDCP CEs are used to indicate which PDU Set discard timer value is applied.

Proposal 8	PDCP CE may, in addition, also indicate the PSI and PDU set discard timer value when PSIs are used

Proposal 9	New mechanism to discard packets in lower layer is not introduced

Proposal 10	Discard signaling from transmitter to receiver is not introduced

# 5 References

3GPP RP-230786, “Updated WID on XR Enhancements for NR”, Mar. 2023.

R2-2212887, Discussion on PDU Set Discard, Ericsson, RAN2#120, Toulouse, November 2022

R2-2212888, Discussion on PDU Prioritization, Ericsson, RAN2#120, Toulouse, November 2022