zs3GPP TSG-RAN WG2 Meeting #122	R2-2305931

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On the applicability of enhanced LCP

WID/SID:	NR_SL_enh2 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss aspects related to the applicability of enhanced LCP.

# 2	Discussion

## 2.1	On LCP enhancements due to COT sharing

During the discussions in RAN2#121bis, the chair has identified the following cases to analyse the impact of COT sharing on the LCP procedure:

- Case 1a - PDU generated before COT arrival, and the PDU does not satisfy COT requirement (either not to the initiating UE, or CAPC value is higher);

- Case 1b - PDU generated before COT arrival, and PDU does satisfy the COT requirement;

- Case 2a - PDU not generated before COT arrival, and no data in RLC buffer satisfying the COT requirement (either no data to the initiating UE, or although there is data to the initiating UE, yet the CAPC value is higher);

- Case 2b - PDU not generated before COT arrival, and there is data in RLC buffer satisfying the COT requirement.

For the case 1a, it is clear that the PDU will have to be transmitted using LBT Type 1. While for the case 2a, after the PDU has been generated it will also have to be transmitted using LBT Type 1.

Proposal 1: When the PDU, associated with a resource within a shared COT, does not meet the shared COT requirements (i.e. no data to the initiating UE or the CAPC associated with the PDU is higher than the CAPC of the shared COT) then the UE performs type-1 LBT.

For the case 1b, if the generated PDU satisfies the COT requirements and the UE has a valid resource associated with the PDU within the shared COT, then the UE should be allowed to apply type-2 LBT.

Proposal 2: When the PDU, associated with a resource within a shared COT, meets the shared COT requirements (i.e. both data to the initiating UE and the CAPC associated with the PDU is equal or lower than the CAPC of the shared COT) then the UE performs type-2 LBT.

For the case 2b, the following agreement was made in RAN2#121bis:

Agreement: If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

According with the discussions during RAN2#121bis, the LCP enhancements that compose the enhanced LCP are:

- Enhanced destination selection step to prioritize initiating UE;

- Enhanced LCH selection step to prioritize LCH associated with the shared COT.

We note that even though there is data in buffer satisfying COT requirement, the legacy LCP should still be applied and there should not be any prioritization of logical channels due to the presence of a shared COT. Otherwise, this can be used by a malicious initiating device to impact the traffic prioritization at a responding device and therefore can lead to degradation of other services not related to the shared COT.

Observation 1: The enhanced LCP can lead to performance degradation of services not related to the shared COT.

However, if a responding UE upon receiving a shared COT applies legacy-LCP, then in case of none of the available PDUs meets the requirement of the shared COT, then (at least to that responding UE) the established COT has been wasted. This is the case since the responding UE may have higher priority data that does not meet the shared COT requirements. Taking unicast as an example, if the higher priority traffic at the responding UE is for UEs other than the COT sharing UE, then the legacy-LCP will prioritize traffic that does not meet the COT requirements and therefore the responding UE will not be able to utilize the shared COT.

Observation 2: The legacy-LCP can prioritize traffic that does not meet the COT requirements and therefore the responding UE will not be able to utilize the shared COT.

However, if the initiating device is made aware of the characteristics of the traffic available at the responding UE and when this traffic will be available, then the initiating device can use this information to decide when to acquire the COT and share it with the responding UE. Furthermore, the use of a MAC CE for the exchange of this assistance information will be more suitable than RRC based assistance information, since then it be applicable to both unicast and groupcast.

Proposal 3: RAN2 to support MAC CE based transfer of assistance information from responding UE to initiating UE with the aim of making the shared COT usable by the responding UE.

The assistance information will be more suitable for periodic traffic than to aperiodic single shot traffic, since for the latter case the overhead related to performing LBT Type 1 will be lower than exchanging the assistance information.

Proposal 4: RAN2 to study the content of the assistance information and conditions for triggering the sharing of the assistance information.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: When the PDU, associated with a resource within a shared COT, does not meet the shared COT requirements (i.e. no data to the initiating UE or the CAPC associated with the PDU is higher than the CAPC of the shared COT) then the UE performs type-1 LBT.

Proposal 2: When the PDU, associated with a resource within a shared COT, meets the shared COT requirements (i.e. both data to the initiating UE and the CAPC associated with the PDU is equal or lower than the CAPC of the shared COT) then the UE performs type-2 LBT.

Observation 1: The enhanced LCP can lead to performance degradation of services not related to the shared COT.

Observation 2: The legacy-LCP can prioritize traffic that does not meet the COT requirements and therefore the responding UE will not be able to utilize the shared COT.

Proposal 3: RAN2 to support MAC CE based transfer of assistance information from responding UE to initiating UE with the aim of making the shared COT usable by the responding UE.

Proposal 4: RAN2 to study the content of the assistance information and conditions for triggering the sharing of the assistance information.