3GPP TSG RAN WG2 #122                                                                              R2-2305957

Incheon, Korea, May 22th – May 26th, 2023                                 Revision of R2-2302820

Agenda Item:	7.6.2.2

Source:	ZTE Corporation, Sanechips

Title:	Further discussion on GNSS reacquisition

Document for:	Discussion and Decision

Introduction

For IoT-NTN Performance Enhancements in Rel-18, some remaining issues from Rel-17 will continue to be discussed in Rel-18. As mentioned in the latest WID of IoT NTN enhancements RP-223519 [1], the GNSS enhancements related objective is listed below:

-	Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

- NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In this contributions, based on the progress in RAN1 and RAN2, we will mainly discuss the procedure for GNSS reacquisition during connected mode and give our suggestions.

Background

In R17 IoT NTN, acquisition/reacquisition of the GNSS position in the scenario of short, sporadic connections has been specified. That is, the UE may need to acquire/reacquire the GNSS position before establishing the connection to avoid interruption during the connection. However, if the GNSS position becomes out-of-date while UE is in RRC_CONNECTED, the UE shall enter idle mode to reacquire GNSS position.

For R18 IoT NTN, reacquisition of the GNSS position during the long connections needs to be studied. It’s expected that, for UE in connected mode, when GNSS position becomes invalid, UE could reacquire GNSS and then continue the service transmission.

RAN1 has had much discussion on this topic, the progress in latest RAN1#112bise meeting (2023.04, emeeting) is copied below for reference:

Agreement For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB. - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB. Agreement On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc Agreement Draft LS in R1-2304125 is endorsed. Final LS is approved in R1-2304126. Agreement UE reports one GNSS position fix time duration for GNSS measurement via a N-bit field at least including [1,2] seconds as component values. - FFS: value of N, other component value(s) of GNSS position fix time duration (e.g. N=3, with value in [3,7,13,19,25, X] seconds, and X is FFS). FFS: whether RAN4 input is needed.

RAN2#121 meeting (2023.02, Athens) and RAN2#121bise meeting (2023.04, emeeting) have had some discussion on UE report of the GNSS position fix time duration and GNSS validation duration, and also some discussion on the procedure of GNSS reacquisition. The following agreements have been achieved:

RAN2#121 agreements: - For UE to report GNSS position fix time duration for measurement during the initial access, at least the following Msg5 message can be used: RRCConnectionSetupComplete, RRCConnectionSetupComplete-NB, RRCConnectionResumeComplete, RRCConnectionResumeComplete-NB, FFS for RRCreestablishmentComplete and RRCConnectionReconfigurationComplete. FFS for Msg3. - FFS whether the UE can stay in RRC_CONNECTED state when current GNSS position becoming out-of-date if the UE has initiated a new measurement. - The value range {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} introduced in R17 is reused for connected UE GNSS validation duration report, unless modified by RAN1. - UE reports GNSS validity duration after GNSS measurement. FFS whether the UE reports every time or only if the validity duration changes. FFS if the duration is the remaining validity duration or the whole duration. RAN2#121bise agreements: - There is no need for UE to provide GNSS position fix time duration in Msg3. - RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS - UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry - For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap - For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE - RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting Working Assumption: - GNSS validity duration UE reported after GNSS measurement is the remaining validity duration.

In this contribution, we will further discuss the details of normal GNSS reacquisition procedure and also the GNSS reacquisition during C-DRX.

Discussion

Procedures for normal GNSS reacquisition in connected mode

In previous RAN1 meetings (RAN1#112 and RAN1#112bise), the main agreements related to procedures for normal GNSS reacquisition in connected mode are as below:

- For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap. FFS details of gap configuration.

- The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement. FFS based on configured timing

- For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB.

  - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB

- On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot

  - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc

From RAN2 perspective, we have the following understanding on the whole procedure:

- UE can either re-acquire GNSS when it receives the explicit trigger from eNB or autonomously re-acquire GNSS (when configured by the network), e.g., when the current GNSS position fix becomes out-of-date.

  - If UE receives the explicit trigger from eNB, no matter whether the trigger includes the explicit gap, UE would start the GNSS measurement at the time point of n + X, where n is the end of MAC CE receiving subframe/slot, X would be a predefined value or configured value.

  - If an explicit gap is included in the trigger, UE should complete the GNSS measurement during this explicit measurement gap. Otherwise (no explicit gap is included in the trigger), UE should take the latest reported GNSS position fix time duration as an implicit measurement gap and complete the GNSS measurement within it.

- RAN2 has agreed that UE can report GNSS position fix time duration for measurement in Msg5 during the initial access. This duration can be seen as the required time length for GNSS measurement which can be used by eNB as reference when it wants to configure the measurement gap. Even the measurement gap can be flexibly configured by eNB, RAN1 has also made an agreement that this configured gap should be equal to or larger than the latest UE reported GNSS position fix time duration.

The above procedure can be found in the following Figure 1:

As IoT UE generally cannot simultaneously perform service transmission in cellular network and GNSS operation, it’s easy to understand, when UE starts to reacquire the GNSS upon expiration of GNSS validity duration, the eNB/UE would stop the UL/DL transmission. Therefore, GNSS reacquisition would inevitably cause service transmission interruption. Therefore, in order try to avoid reduce the service transmission interruption, even on one hand, UE and eNB should have consistent understanding on the start of GNSS measurement in order that eNB can stop the service scheduling at the suitable timing. On the other hand, UE and eNB should have consistent understanding on the end of GNSS measurement in order that eNB can resume the service scheduling timely. There are two alternatives to keep such consistent understanding between UE and eNB:

- Alt 1: due to that more companies support Alt1, Alt 1 has been made working assumption in last meeting. The basic assumption for this Alt1 is that UE can finish GNSS reacquisition at any time point within the configured or implicit measurement gap. Therefore, after each time UE in connected mode finishes GNSS reacquisition, UE needs to report remaining validity duration. Upon reception of this report, eNB can know that the UE has successfully reacquire the GNSS and then can resume the service scheduling for the UE. At the same time, eNB can start a GNSS validity timer in its side. Based on this timer, eNB can deduce the next time the UE side GNSS will become invalid. Before this next time of GNSS expiration, eNB can choose to send an MAC CE with or without measurement gap configuration to explicitly trigger UE to perform GNSS measurement.

  - The main concern of Alt 1 is that much UE power consumption caused by UL MAC CE transmission for GNSS remaining validity duration report. During the previous meeting discussion, some companies mentioned that for some times UE finishes GNSS reacquisition, UE can skip the remaining GNSS validity duration report if the value keeps same as the last reported one (it’s possible as the granularity of remaining validity duration report is coarse). However, we understand this is infeasible. Even in this time the absolute value of GNSS remaining validity duration can be same as the last reported one (e.g., 4s), as eNB has no any idea when UE in connected mode finishes GNSS reacquisition for this time, eNB still cannot know to use this remaining GNSS validity duration. In other word, even eNB can know the time length of the GNSS validity timer in its side, it cannot know when to start this timer.

Observation 1: With working assumption that GNSS validity duration UE reported after GNSS measurement is the remaining validity duration, as eNB has no any idea when UE in connected mode finishes GNSS reacquisition, it’s mandatory for UE to report the remaining time after each time UE finishes its GNSS measurement, regardless of whether the remaining time is changed or not compared with last time GNSS reacquisition. If no such report, eNB cannot know when to start the corresponding GNSS validity duration time in its side.

  - Another issue of Alt 1 is the granularity of remaining GNSS validity duration report. In previous meeting, RAN2 has agreed that the value range {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity} introduced in R17 is reused for connected UE GNSS validation duration report, unless modified by RAN1. On top of Alt1, that means the granularity of remaining GNSS validity duration report would be very coarse. During previous discussion, more companies support Alt1 also indicate “Aligned with R17” as the main reason. However, we think the situation for GNSS reacquisition in connected mode is very different from that in R17. For R17 IoT NTN, the remaining GNSS validity duration report is mainly used to assist eNB to gracefully release the RRC connection before the expiration of GNSS, in order to avoid the resources hangs. However, for GNSS reacquisition in connected mode, one of the remaining GNSS validity duration report is to align the service scheduling/monitoring between UE and eNB. Since the granularity of service scheduling is with unis of ms, the current remaining GNSS validity duration report would be too coarse that will cause a big deviation between the scheduling of the eNB and the monitoring of UE, e.g., several milliseconds.

Observation 2: With working assumption that GNSS validity duration UE reported after GNSS measurement is the remaining validity duration and the previous agreement on value range of connected UE GNSS validation duration report, the remaining GNSS validity duration report would be too coarse that will cause a big deviation between the scheduling of the eNB and the monitoring of UE, e.g., several milliseconds.

- Alt 2: In this alternative, UE can report the whole GNSS validity duration only once, e.g., during initial access and can update it later if it changes (seldom). The related assumption for Alt 2 is that, UE and eNB would autonomously starts the new GNSS validity timer on their own sides on the common time point with the same timer length (the whole validity duration). Here the common time point could be the end of explicit or implicit measurement gap.

  - One of the concern of Alt 2 is that, it may be possible that the UE finish its GNSS measurement at a time point earlier than the end of the explicit or implicit measurement gap. But based on the understanding of the concept of GNSS position fix duration and also the reason why we introduce it, we assume this would be the rare case and the earlier amount will be relatively small. So it’s still reasonable for UE to start the new GNSS validity timer at the end of the explicit or implicit measurement gap (e.g., a bit later than the real end of GNSS reacquisition). By this way, UE don’t need to report the whole GNSS validity duration many times which is signaling efficient. Moreover, as the reported whole GNSS validity duration is mainly used to provide a fixed pattern in both UE and eNB to perform GNSS reacquisition, its granularity would not be sensitive.

Based on the above analysis, we still think the Alt2, e.g., the GNSS validity duration UE reported after GNSS measurement is the whole validity duration would be the better one. So we give the following proposal:

Proposal 1: RAN2 doesn’t confirm the working consumption about GNSS validity duration report in RAN2#121bise meeting. RAN2 agree that the GNSS validity duration UE reported after GNSS measurement is the whole validity duration.

If Proposal 1a cannot be agreed, we have the further agreements as below:

Proposal 2a: RAN2 confirms the working consumption that GNSS validity duration UE reported after GNSS measurement is the remaining validity duration.

Proposal 2b: UE will always report the remaining GNSS validity duration after each time UE finishes GNSS reacquisition.

Proposal 2c: RAN2 needs to introduce finer values, e.g., in unit of milliseconds, for the value range of UE remaining GNSS validation duration report.

GNSS reacquisition during inactive state of Connected DRX

In RAN1 previous meetings, companies has discussed the possibility of reacquiring GNSS position fix during inactive state of Connected DRX. In latest RAN1#112bise meeting, RAN1 has made conclusion as below:

RAN1#112bise Conclusion From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2 Send an LS to RAN2 for the conclusion.

Accordingly, in RAN2#121bise meeting, RAN2 agreed to discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting.

When UE is configured with C-DRX, an ideal case is that the expiration of GNSS validity duration is just within the inactive state of C-DRX and the time length of inactive state is also long enough for UE to complete the GNSS position fix reacquisition. In such case, there would be no service interruption. And there is no issue for NW as NW can also know this GNSS reacquisition and see it as a normal one.

But it’s easy to understand that since C-DRX is flexibly configured, this ideal situation would not always happen. More commonly, even if the expiration of GNSS validity duration can be within the inactive state of C-DRX, the GNSS reacquisition of UE may last till after end of inactive state, or, the expiration of GNSS validity duration occurs even after end of inactive state. In these cases, the service interruption is still unavoidable. So a possible optimization is, instead of UE starting to reacquire the GNSS when the GNSS validity duration timer expires (e.g., at T2 time point), if C-DRX is configured and UE determines that it’s possible to complete GNSS reacquisition during an inactive state of C-DRX, UE can deliberately stop the GNSS validity duration timer and start the GNSS reacquisition early (e.g., at T1, which is earlier than T2) during inactive state of C-DRX. This is shown in Figure 3.

Figure 2: Eealy start of GNSS reacquisition during inactive state of C-DRX (1)

UE can decide when to start GNSS reacquisition as long as it is guaranteed to complete before end of inactive state of C-DRX. But in order that the reacquired GNSS can cover a period of time as long as possible, it’s better for UE to start GNSS reacquisition as late as possible.

Proposal 3a: If C-DRX is configured and UE determines that it’s possible to complete GNSS reacquisition during an inactive state of C-DRX, UE is allowed to deliberately stop the GNSS validity duration timer and start the GNSS reacquisition early during an inactive state of C-DRX, e.g., earlier than the expiration of GNSS validity duration timer.

Since what we are talking is an early GNSS reacquisition, it means that the current GNSS is still valid. Even if there is an opportunity to make use of the inactive state of C-DRX, if the validity period of current GNSS is still relatively long, there is no need to autonomously trigger GNSS acquisition too early. Therefore, in order to avoid unnecessary GNSS reacquisition, certain conditions can be set to control UE to trigger GNSS reacquisition only when appropriate. One possible condition is that, only when the remaining validity time period of the UE’s current GNSS is less than a threshold, the UE can autonomously trigger GNSS reacquisition in the inactive state of C-DRX. This threshold can be predefined or configured by NW.

Proposal 3b: Only when the remaining validity time period of the UE’s current GNSS is less than a predefined or configured threshold, the UE can autonomously trigger GNSS reacquisition in an inactive state of C-DRX.

Moreover, if UE is allowed to start the GNSS reacquisition early, e.g., earlier than the expiration of GNSS validity duration timer with the intention to align with the inactive state of C-DRX, the consistent understanding between UE and NW about the start of each time GNSS reacquisition would be broken. The NW cannot be aware of this and the NW will still begin to stop the scheduling for the UE at T1, which is completely unnecessary. In order to avoid such issue, UE needs to report kind of indication to inform NW about this early or temporarily GNSS reacquisition. In the following, three possible alternatives are discussed:

- One possible alternative can be that, before the UE starts to perform GNSS reacquisition, UE is required to report to the NW this early GNSS reacquisition (Alt1). Based on UE’s report, NW can also stop its GNSS validity duration timer accordingly and restart it after GNSS reacquisition. However, due to that the NW would not schedule DL or UL for the UE during the inactive state of C-DRX, it may be kind of difficult for UE to report to NW just before its GNSS reacquisition, e.g., UE may need to initiate a connected mode PRACH procedure. This may be undesired. An alternative way could be that, if UE can determine the T1 in advance, UE can inform the network about this expected early GNSS reacquisition and also the start time in future, e.g., T1, before the start of the inactive state of C-DRX. That means UE can send this report during the active state of C-DRX, for example, along with other service transmission, via RRC singling, MAC CE or L1 signaling etc. This Alt1 can be found in above Figure 3.

- There is another alternative that, UE can inform the NW after the GNSS reacquisition is finished (Alt2). However, if UE sends this report as soon as the GNSS reacquisition is finished, as it’s highly possible that UE is still in inactive state, UE needs to trigger PRACH procedure. In order to avoid PRACH procedure during inactive state, UE also can pending this report and send it after the end of the inactive state. One additional benefit is that if the GNSS validity duration changes, the updated GNSS validity duration can also be reported to the eNB, e.g., along with the indication about this early GNSS reacquisition. But the disadvantage is that, when NW receives this indication from the UE, the GNSS validity duration timer on the NW side may have already expired and the NW have stopped DL/UL scheduling for the UE for a while. As it can be assumed that the interval between the end of GNSS reacquisition and the end of the inactive state could be very short, we think this disadvantage could be small. This Alt2 is shown in Figure 4.

Figure 3: Eealy start of GNSS reacquisition during inactive state of C-DRX (2)

Based on the above discussion, we prefer Alt1.

Proposal 3c: UE can inform the network about the expected early GNSS reacquisition and also the start time in future, e.g., T1, before the start of the inactive state of C-DRX. In other word, UE can send this report during the active state of C-DRX along with other service transmission, via RRC singling, MAC CE or L1 signaling etc.

Conclusion

In previous sections, the following observations and proposals were made:

Proposal 1: RAN2 doesn’t confirm the working consumption about GNSS validity duration report in RAN2#121bise meeting. RAN2 agree that the GNSS validity duration UE reported after GNSS measurement is the whole validity duration.

If Proposal 1a cannot be agreed, we have the further agreements as below:

Proposal 2a: RAN2 confirms the working consumption that GNSS validity duration UE reported after GNSS measurement is the remaining validity duration.

Proposal 2b: UE will always report the remaining GNSS validity duration after each time UE finishes GNSS reacquisition.

Proposal 2c: RAN2 needs to introduce finer values, e.g., in unit of milliseconds, for the value range of UE remaining GNSS validation duration report.

Proposal 3a: If C-DRX is configured and UE determines that it’s possible to complete GNSS reacquisition during an inactive state of C-DRX, UE is allowed to deliberately stop the GNSS validity duration timer and start the GNSS reacquisition early during an inactive state of C-DRX, e.g., earlier than the expiration of GNSS validity duration timer.

Proposal 3b: Only when the remaining validity time period of the UE’s current GNSS is less than a predefined or configured threshold, the UE can autonomously trigger GNSS reacquisition in an inactive state of C-DRX.

Proposal 3c: UE can inform the network about the expected early GNSS reacquisition and also the start time in future, e.g., T1, before the start of the inactive state of C-DRX. In other word, UE can send this report during the active state of C-DRX along with other service transmission, via RRC singling, MAC CE or L1 signaling etc.

References

- RP-223519, Revised WID: IoT (Internet of Things) NTN (non-terrestrial network) enhancements, MediaTek Inc., December 2022