3GPP TSG RAN WG2 #122                                                                         R2-2305963

Incheon, Korea, May 22-26, 2023                                          Revision of R2-2302825

Agenda Item:	7.19.3

Source:	ZTE Corporation, Sanechips

Title:	Early indication and access restriction for eRedCap UE

Document for:	Discussion and Decision

Introduction

In WID RP-223544[1], the objective of additional early indication for Rel-18 eRedCap UE is as below:

Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2]

In RAN1 #111 [2], the following agreements were reached:

Agreement For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs Conclusion There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”. Conclusion For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR. Agreement For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X. Agreement Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Agreement For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Working Assumption - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In RAN2 #121 meeting [3], the following agreement was reached.

RAN2 confirms the enhanced INACTIVE eDRX can be applied to all R18 UEs. FFS if it can only be supported by UEs which support R17 eDRX. We will wait for RAN1 progress to see if there is a need for a Msg1 early indication for eRedCap. The NR MIB “cellBarred” bit applies to all UEs (Normal UEs, Redcap UEs and eRedcap UEs).

In RAN2 #121bis meeting [4], the following agreement was reached.

SIB1 should be able to indicate whether the cell enables access for eRedCap UEs or not (assuming that eRedCap UE is not allowed to access to the legacy cell nor the cell not supporting eRedCap). FFS on the relationship and granularity with the access control/cell barring purpose indication. Introduce R18 eRedCap UE specific IFRI in SIB1. The new R18 eRedCap UE specific IFRI functionality works as follows: - Controls cell selection/reselection to intra-frequency cells for eRedCap UEs when this cell is considered barred by the eRedCap UE, as specified in TS 38.304 [20]. - Working assumption (pending check in running CRs): If not present, an eRedCap UE treats the cell as barred, i.e., the UE considers that the cell does not support eRedCap. Introduce eRedcapAccessAllowed-r18 in interFreqCarrierFreqList in SIB4, about the frequency of neighbour cell supporting eRedCap, similar to R17. From RAN2 perspective, there is no need to introduce eRedCap UE specific initial BWP configuration (i.e. no R18 new field and at most one specific initial UL/DL BWP can be configured). If the R17 RedCap specific initial BWP is configured, eRedCap UEs always use it as its specific initial BWP (assuming no eRedCap UE specific initial BWP configuration field introduced). Working assumption: Use two new LCID values to support Msg3 early identification for eRedCap UE (can be revised and discussed together with other R18 WIs, if R18 WIs may occupy relatively many LCIDs).

In this contribution, based on the RAN1 and RAN2 progresses, we will further discuss how to support early indication, access restriction and provide our suggestions.

Discussions

2.1 Early Indication

In RAN1 #111 meeting [2], the following agreements for Msg3 transmission resource allocation were reached:

Agreement For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

According to the above agreement, for Rel-18 eRedCap UE, the scheduled Msg3 PUSCH resource will be restricted within 5MHz. Meanwhile, for legacy UEs supporting RA based small data transmission, it is very possible to schedule Msg3 PUSCH resource for RA based SDT exceeding the above restriction. Therefore, there is a need for NW to differentiate the R18 eRedCap UE from the legacy UE before the transmission of RAR. Then it is beneficial to support Msg1/MSGA based early indication for UE to inform NW that it is a Rel-18 eRedCap UE.

Proposal 1: Msg1/MSGA based early indication could be supported in order that NW can differentiate the R18 eRedCap UE and meet its 5MHz bandwidth restriction for scheduling Msg3 PUSCH resources.

RACH partitioning can be used to provide Msg1/MSGA based early indication. In Rel-17, feature combination in RACH partitioning is introduced as below:

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                    OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                    OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                        OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                    OPTIONAL,  -- Need R

spare4                     ENUMERATED {true}                    OPTIONAL,  -- Need R

spare3                     ENUMERATED {true}                    OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                    OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                    OPTIONAL   -- Need R

}

Then for Rel-18 eRedCap, it is straightforward to define a new feature of eRedCap by using the reserved spare bit in FeatureCombination-r17.

However, it’s known that RACH partitioning is easy to cause waste of random access resources. Moreover, if there are not enough RACH resources, it would be difficult to provide RACH partitioning for so many functions at the same time. Therefore, we think Msg1/MSGA based early indication for Rel-18 eRedCap can only be optionally supported, e.g., rely on the status of RACH resources.

Once there are enough RACH resources, the NW can configure the new feature of eRedCap in FeatureCombination-r17. The eRedCap UE can make use of Msg1/MSGA to indicate to the NW that it is a Rel-18 eRedCap UE. This can facilitate the NW to exactly schedule Msg3 PUSCH resource within 5MHz for this UE. However, if RACH resources is not enough, the NW would not configure the new feature of eRedCap in FeatureCombination-r17. If the NW still follow the legacy scheduling scheme, it may schedule more than 5M Msg3 PUSCH resource for any UEs. This may further cause the eRedCap UE fails to transmit Msg3. This is obviously undesired.

In order try to avoid such RACH failure, NW implementation can make a restriction, i.e. the scheduled Msg3 PUSCH resource is mandatory to be within 5MHz. To limit the restriction scope, if the eRadCap RACH resource is not configured but the redCap RACH resource is configured, the eRadCap UE can use the redCap RACH resource, in which case the NW should only limit the Msg3 PUSCH resource within 5Mhz for UEs using redCap RACH resource in the cell that supports eRedCap. As we assume the UEs that need to be scheduled with more than 5M Msg3 PUSCH resources are still a minority, the impacts of such restriction can also be small.

Proposal 2a: RAN2 to define a new feature of eRedCap with the reserved spare bit in FeatureCombination-r17. This is used as Msg1/MSGA based early indication for indicating Rel-18 eRedCap UE.

Proposal 2b: If the eRadCap RACH resource is not configured but the redCap RACH resource is configured, the eRadCap UE use the redCap RACH resource.

2.2 Access restriction

Based on the following agreement

SIB1 should be able to indicate whether the cell enables access for eRedCap UEs or not (assuming that eRedCap UE is not allowed to access to the legacy cell nor the cell not supporting eRedCap). FFS on the relationship and granularity with the access control/cell barring purpose indication.

In the eRedCap WID, it has described that “aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction”, one cellBarred bit seems enough for eRedCap UEs (e.g. to indicates whether the cell allows the UEs supporting UE peak data rate reduction and 5 MHz BB bandwidth only for PDSCH and PUSCH to access). Whether finer granularity with the access control/cell barring purpose indication for eRedCap UE can wait for RAN1 decision on the eRedCap UE type definition.

Proposal 3: A cellBarredEnhRedCap bit is introduced in SIB1 for eRedCap UE. Whether finer granularity with the access control/cell barring purpose indication for eRedCap UE can wait for RAN1 decision on the eRedCap UE type definition.

Since Rel-18 eRedCap feature is a further enhanced feature on the basis of Rel-17 RedCap, e.g. to further reduce UE complexity in FR1 at BB and UE peak data rate. The Rel-18 eRedCap feature may use some of the Rel-17 RedCap enhancement (e.g. halfDuplex operation, 1 Rx feature and NCD-SSB etc). Thus, if the NW allows Rel-18 eRedCap UE to access, it should also allow Rel-17 RedCap UE to access (e.g. support some feature and configuration).

Proposal 4: If the NW allows Rel-18 eRedCap UE to access, it should also allow Rel-17 RedCap UE to access.

Conclusions

This contribution presents some of our views on early indication and access restriction for Rel-18 eRedCap UEs. We have the following proposals:

Proposal 1: Msg1/MSGA based early indication could be supported in order that NW can differentiate the R18 eRedCap UE and meet its 5MHz bandwidth restriction for scheduling Msg3 PUSCH resources.

Proposal 2a: RAN2 to define a new feature of eRedCap with the reserved spare bit in FeatureCombination-r17. This is used as Msg1/MSGA based early indication for indicating Rel-18 eRedCap UE.

Proposal 2b: If the eRadCap RACH resource is not configured but the redCap RACH resource is configured, the eRadCap UE use the redCap RACH resource.

Proposal 3: A cellBarredEnhRedCap bit is introduced in SIB1 for eRedCap UE. Whether finer granularity with the access control/cell barring purpose indication for eRedCap UE can wait for RAN1 decision on the eRedCap UE type definition.

Proposal 4: If the NW allows Rel-18 eRedCap UE to access, it should also allow Rel-17 RedCap UE to access.

References

- RP-223544 Revised WID on Enhanced support of reduced capability NR devices.

- Draft_Minutes_report_RAN1#111_v010.

- R2-2301910 Report from eRedCap breakout session, RAN2#121, February, 2023

- Draft_R2_121bis_meeting_report_v2