3GPP TSG-RAN WG2 Meeting #122		 R2-2305970

Incheon, Korea: 22nd May – 26th May 2023

Source:	Qualcomm Incorporated

Title:	Discussion on Data Collection Requirements/Constraints for Different LCM Purposes

Agenda Item:	7.16.2.2 Data Collection

Document for:	Discussion and Decision

# Introduction & Background

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1], with the main goal of exploring the benefits of augmenting the air interface with features enabling improved support of AI/ML-based algorithms for enhanced performance and/or reduced complexity/overhead.

Through studying a few carefully selected use cases, the goal is to identify a common AI/ML framework, including functional requirements of AI/ML architecture, which could be used in subsequent projects. The study should also identify areas where AI/ML could improve the performance of air-interface functions.

The study will serve to identify what is required for an adequate AI/ML model characterization and description establishing pertinent notation for discussions and subsequent evaluations. Various levels of collaboration between the gNB and UE are identified and considered. Specification impact will be assessed to improve the overall understanding of what would be required to enable AI/ML techniques for the air interface.

The SI consists of studying individual use cases as well as deriving a general framework for AI/ML. Below we summarize the goal of the study as shown in [1,2] relevant to the general framework:

AI/ML model, terminology, and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting separate or joint ML operations.

- Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures, and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

The SI further defines responsibility for different WGs for accessing potential specification impacts [1,2], whereas the RAN2 study access protocols aspects of the potential specification impacts, as mentioned below:

- […]

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input

    - Collaboration level-specific specification impact per use case

In this contribution, we will discuss different aspects of data collection.

# Protocols design aspects

As described in the SID [1, 2], RAN2 should access protocol-related specification impact considering the progress in RAN1 as the reference. In this contribution paper, we will take RAN1 agreements (in RAN1#109-e [3], RAN1#110 [4], RAN1#110bis [5], RAN1#111 [7], RAN1#112 [9], and RAN1#112-bis [15]) and RAN2 agreements (in RAN2#119bis-e [6], RAN2#120 [8], RAN2#121 [10], and RAN2#121bis-e [16]) meeting agreements to discuss different aspects of data collection for AI/ML life Cycle Managements (LCMs).

## 2.1	Data Collection Requirements/Constraints for Different LCM Purposes

In RAN1#110bis-emeeting [5], RAN1 concluded that data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

Observation 1: In RAN1#110bis-emeeting [5], RAN1 concluded that data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc.

In the RAN2#121 meeting [10], RAN2 endorsed the following,

- RAN2 loosely endorsed proposal outcomes of the post-RAN2 #120 email discussion in R2-2301440

- RAN2 endorsed the table in R2-2302286 as the starting point.

During the email discussion [11] and RAN2#121 [10], companies showed interest in separately analyzing data collection requirements and solutions for different LCM purposes.

Observation 2: In the RAN2#121 meeting, RAN2 agreed to separately analyze the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted (proposal 3 [10]).

Observation 3: Chair: There is significant support to aim for evaluating the data collection methods per LCM purpose.

In the RAN2#121bis-emeeting, RAN2 further agreed to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. RAN2 agreed to study the applicability (and limitations) of each identified data collection framework for each identified LCM purpose, i.e., inference, monitoring, and (offline) training.

Observation 4: In the RAN2#121bis-emeeting, RAN2 agreed to understand/determine/capture requirements of data collection for the LCM functionalities and document the results.

Observation 5: In the RAN2#121bis-emeeting, RAN2 agreed to study the applicability (and limitations) of each identified data collection framework for each identified LCM purpose, i.e., inference, monitoring, and (offline) training.

### 2.1.1	Data Collection Requirements/Constraints for UE-side Models’ offline training

In this section, we focus on data collection requirements and constraints for UE-side models’ offline training. Note that for different UE types, e.g., low, mid, and top-tier UEs, the UE vendors may be interested in developing different models with varying model performance. For example, the UE vendors may develop models with extremely high expected performance for top-tier UEs, and develop models with acceptable performance for low-tier UEs. Similarly, UE vendors may develop different models for different energy and computation status at the UE. Theoretically, the network can develop and perform offline training for the UE-side model, however, such differentiation (e.g., per UE type, power status, etc.) cannot be achieved through UE-side model training at the network because of the following reasons,

- Vendor information is not available on the network

- A temporary identity is available at the NG-RAN; therefore NG-RAN cannot identify UE uniquely across different RRC states

      - On UE-side models’ offline training by network

As discussed earlier, the network side may not be a suitable place for training UE side models, as different models may need to be developed for different tier UEs and/or for different UE internal conditions. As the NG-RAN does not have vendor information and maintains a temporary identity for the UE, the network cannot develop and train a UE-specific or UE-vendor-specific model.

Observation 6: Different models may need to be developed for different tier UEs, for different UE internal conditions, and others.

Observation 7: NG-RAN cannot develop UE-specific or UE-vendor-specific models due to the following reasons

- Vendor information is not available on the NG-RAN

- A temporary identity is available at the NG-RAN; therefore NG-RAN cannot identify UE uniquely across different RRC states

Furthermore, note that for model training a large amount of training data will be required. They need to be stored and processed for the model training. Training data storage and processing, and model training require significantly large storage and computation power. Furthermore, to avoid frequent switching of the UE-side models during inference, the model may need to be generalized across gNBs. Training of the UE-side models at NG-RAN may fail to achieve a good model generalization, which will require frequent model transfer during handovers.

Observation 8: For model training a large amount of training data will be required. They need to be stored and processed for the model training.

Observation 9: Training data storage, training data processing, and model training require significantly large storage and computation power. Therefore, model training cannot be supported at the NG-RAN.

Observation 10: To avoid frequent switching of the UE-side models during inference, the model may need to be generalized across gNBs. Training of the UE-side models at NG-RAN may fail to achieve a good model generalization, which will require frequent model transfer/delivery during handovers.

Note that the existing control plane-based data collection frameworks such as MDT, UE assistance information, early idle/inactive measurements, RRM measurement reports, and CSI reporting framework, support the collection of standardized data only after the feature is deployed. However, for an AI/ML model or feature to be deployed in the network, the first step is to collect data for the development of models. During the development of the model for an AI/ML feature using offline training, the required input, their normalization, and other properties of the data may not be well defined.

Observation 11: Existing Data collection for the Rel-18/Rel-19 use cases cannot be supported until the feature is deployed, which is a significant constraint on data collection for offline training of UE-side models.

#### 2.1.1.2 On UE-side models’ offline training by the UE vendors

As discussed earlier, the UE vendors may want to develop different models for different UE types, UE internal conditions, etc for each use case and sub-se cases. Therefore, although for each sub-use case, RAN1 may define data collection requirements based on nominal inputs and outputs, the UE side may take other information into account for model development and training. Note that the actual inputs/outputs to/from the models used at the device may be different from the nominal inputs and outputs. For example, a model at UE may take auxiliary inputs such as SNR, Doppler, sensor measurements, etc. that do not need to be standardized. As another example, a model at UE may use pre-processing and post-processing that may make the actual input/output to/from the model different from the nominal ones. What should be the actual inputs and outputs of a model is an implementation-specific decision per device/chipset development and cannot be pre-determined/standardized.

Observation 12: For offline training of the UE-side models, e.g., for training data labeling and model training, UE-side may take auxiliary inputs such as SNR, Doppler, sensor measurements, etc. that do not need to be standardized.

#### 2.1.1.3 Evaluation of data collection framework for UE-side models

Considering the above discussions in sections 2.1.1.1 and 2.1.1.2 for offline training of UE side models, we can categorize the data collection requirement/constraints as the following

- Data collection for model training cannot be supported until the feature is deployed

- Collection of auxiliary information for training data labeling and/or model training

- Data collection for UE-specific model training

- Offline model training requires a large amount of training data requiring training data storage

- Offline model training requires significant computational resources for training data processing and training

- Achieving good model generalizations

Proposal 1: The following constraints/requirements should be considered for the data collection for the UE side model’s offline training

- Data collection for model training cannot be supported until the feature is deployed

- Collection of auxiliary information for training data labeling and/or model training

- Data collection for UE-specific model training

- Offline model training requires a large amount of training data requiring training data storage

- Offline model training requires significant computational resources for training data processing and training

- Achieving good model generalizations

Let us evaluate identified data collection frameworks considering the constraints/requirements outlined in Proposal 1 in Table Ι.

Proposal 2: Consider Table Ι for evaluation of identified data collection framework for the UE-side model’s offline training.

Table Ι: Evaluation of identified data collection framework for UE-side model’s offline training

| LCM | LCM requirements/constraints | Identified data collection framework | Why the existing framework may/may not work. |
| --- | --- | --- | --- |
| Model training | - Data collection for model training cannot be supported until the feature is deployed - Collection of auxiliary information for training data labeling and/or model training - Data collection for UE-specific model training - Offline model training requires a large amount of training data requiring training data storage - Offline model training requires significant computational resources for training data processing and training - Achieving good model generalizations | Logged MDT | - Supports data collection in IDLE/INACTIVE states. - Cannot collect the data until the corresponding use case is deployed - Cannot support the collection of non-standardized data - Hard to uniquely identify the UE at RAN, in particular, for management-based MDT - Limited storage at the UE |
|  |  | Immediate MDT | - Cannot collect the data until the corresponding use case is deployed - Cannot support the collection of non-standardized data - Hard to uniquely identify the UE at RAN, in particular, for management-based MDT - Limited payload size |
|  |  | L3 measurements | - Cannot collect the data until the corresponding use case is deployed - Cannot support the collection of non-standardized data - Hard to uniquely identify the UE at RAN - Limited payload size - Limited storage and computational power at NG-RAN - Model generalization cannot be achieved |
|  |  | L1 measurements | - Cannot collect the data until the corresponding use case is deployed - Cannot support the collection of non-standardized data - Hard to uniquely identify the UE at RAN - Limited payload size - Limited storage and computational power at NG-RAN - Model generalization cannot be achieved |
|  |  | UAI | - Cannot collect the data until the corresponding use case is deployed - Cannot support the collection of non-standardized data - Hard to uniquely identify the UE at RAN - Limited payload size - Limited storage and computational power at NG-RAN - Model generalization cannot be achieved |
|  |  | Early measurements | - Supports data collection in IDLE/INACTIVE states. - Cannot collect the data until the corresponding use case is deployed - Cannot support the collection of non-standardized data - Hard to uniquely identify the UE at RAN - Limited payload size - Limited storage and computational power at NG-RAN - Model generalization cannot be achieved |
|  |  | LPP | - May be okay for the positioning use case. Not suitable for CSI use cases and BM use cases. |
|  |  | EVEX | See section 2.1.1.4 for a detailed evaluation. |

Proposal 3: Existing control plane-based methods are not suitable for data collection for offline training of UE-side models.

#### 2.1.1.4	Evaluating EVEX for data collection for offline model training

In the RAN2#121bis-emeeting, EVEX was discussed as the data collection framework for offline model training. Based on the online discussions in RAN2#121bis-emeeting, we believe that companies' concerns can be categorized into the following

- Whether/how offline training of RAN-centric use case is supported in EVEX

- Whether/how security is ensured in the EVEX

- Whether/how user privacy is ensured in the EVEX

- Whether/how fine granular control is supported in EVEX

Observation 13: Based on the online discussions in RAN2#121bis-emeeting, we believe that companies' concerns can be categorized into the following

- Whether/how offline training of RAN-centric use case is supported in EVEX

- Whether/how security is ensured in the EVEX

- Whether/how user privacy is ensured in the EVEX

- Whether/how fine granular control is supported in EVEX

Let us discuss each of these concerns one by one.

##### 2.1.1.4.1    Whether/how offline training of RAN-centric use case is supported in EVEX

As discussed in Proposal 1, for supporting offline training, significantly large data storage space and computation power may be required at the data collection terminating node (or node performing offline model training). Therefore, even if the use cases are RAN-centric, the training may not be performed at the NG-RAN. Furthermore, to achieve a good generalization performance, the training data should be collected across different NG-RANs.

Observation 14:  For supporting offline model training, significantly large data storage space and computation power may be required at the data collection terminating node (or node performing offline model training). Therefore, even if the use cases are RAN-centric, the training may not be performed at the NG-RAN.

Observation 15: To achieve a good generalization performance, the training data should be collected across different NG-RANs.

Consider the below figure from TS 26.531 [14] for EVEX. In the EVEX framework, the training data may be made available to the interested training entities, for example, ASPs, NWDAF, NRF, trusted AS, External AS, RAN (via AMF), and others.

Proposal 4: In the EVEX framework, the training data may be made available to the interested training entities, for example, ASPs, NWDAF, NRF, trusted AS, External AS, RAN (via AMF), and others.

##### 2.1.1.4.2    Whether/how security is ensured in the EVEX

The EVEX framework supports the information security model, where the following security models are supported [14]

- Transport security: An encrypted data transfer protocol shall be employed at reference point R2 to protect the secrecy and integrity of collected UE data in transit between the Direct Data Collection Client and the Data Collection AF.

- Data exposer restriction model: The Provisioning AF restricts the exposure of UE data over reference points R5 and R6 by configuring a set of Data Access Profiles for each Event ID to be exposed.

- Authentication of data collection clients by the Data Collection AF: a data collection client shall supply authentication information to the Data Collection AF

Observation 16: The EVEX framework supports the following security models

- Transport security

- Data exposure restriction model

- Authentication of data collection client by the Data Collection AF

Proposal 5: The EVEX framework supports the required security models for data collection.

##### 2.1.1.4.3    Whether/how user privacy is ensured in the EVEX

In the EVEX framework, via Service Level Agreements (SLA) with the ASP, operators can configure the DCAF with instructions that control the procedures of permissible UE data collection and permissible event exposure by the DCAF of its collected UE data to event consumers (e.g., NWDAF or ASP). Note that operators can ensure that only those data can be collected from the UE for which relevant user consents are available.

Observation 17: In the EVEX framework, via Service Level Agreements (SLA) with the ASP, operators can configure the DCAF with instructions that control the procedures of permissible UE data collection and permissible event exposure by the DCAF of its collected UE data to event consumers (e.g., NWDAF or ASP).

Observation 18:  Operators can ensure that only those data can be collected from the UE for which relevant user consents are available.

Proposal 6: The EVEX framework ensures user privacy through SLA and user consent, by enabling restrictions that UE can share the information with AF only if appropriate user consent is available.

##### 2.1.1.4.4    Whether/how fine granular control is supported in the EVEX

The EVEX framework supports different Data Access Profile which defines restrictions along the time, user, and location dimensions. The finest granularity allows access to events as they take place in time. The coarsest level of access aggregates all event data along the time axis to produce a single aggregated value.

Observation 19: The EVEX framework supports different Data Access Profile which defines restrictions along the time, user, and location dimensions.

Proposal 7: EVEX supports different Data Access Profile defines restrictions along the time, user, and location dimensions to provide fine granular control for data collection.

Proposal 8: Consider EVEX as one of the options for data collection for offline model training.

### 2.1.2	Data Collection Requirements/Constraints for model inference

As the existing RAN1 use cases may have tight timelines. Note that during the model inference, not only do the measurements need to be collected and reported but they may need to be further processed and validated. For example, the UE may need to collect and report the measurement; thereafter the NG-RAN may need to process and validate before using for inference. Therefore, in our understanding, a data collection framework for model inference should have low latency. Also, note that the data required for model inference may have low data volume.

Proposal 9: The following should be considered when considering the data collection for model inference

- Terminates at NG-RAN

- Low data volume

- Low latency

Let us evaluate identified data collection methods considering the constraints/requirements outlined in Proposal 9 in Table ΙΙ.

Proposal 10: Consider Table Ι for evaluation of identified data collection framework for model inference.

Table ΙΙ: Evaluation of identified data collection framework for offline model training

| LCM | LCM requirements/constraints | Identified data collection framework | Why the existing framework may/may not work. |
| --- | --- | --- | --- |
| Model Inference | - Terminates at NG-RAN - Low data volume - Low latency | Logged MDT | - Supports data collection in RRC IDLE/INACTIVE states - Terminates at TCE/OAM - No issue with low data volume - Does not support low latency |
|  |  | Immediate MDT | - Terminates at TCE/OAM - No issue with low data volume - High latency, may not be suitable for RAN1 use cases |
|  |  | L3 measurements | - Terminates at NG-RAN - No issue with low data volume - Can support low latency for BM and positioning. May not be suitable for CSI use cases. |
|  |  | L1 measurements | - Terminates at NG-RAN - Suitable for carrying low data volume - Can support low latency |
|  |  | UAI | - May not be suitable for carrying inference output - Can be used for requesting assistance information from the network |
|  |  | Early measurements | - Supports data collection in IDLE/INACTIVE states. |
|  |  | LPP | - Suitable for carrying positioning (for both assistance info and inference measurements) |
|  |  | EVEX | - Does not achieve the low latency requires for model inference |

### 2.1.3	Data Collection Requirements/Constraints for model monitoring

Based on RAN1 discussions, the monitoring can be performed for the following purposes

- Fast-timescale monitoring for model activation, deactivation, fallback, and switching

- Slow-timescale monitoring for model selection, activation, deactivation, fallback, and switching

- Slow-timescale monitoring for model updates and retraining

Observation 20: Based on RAN1 discussions, the monitoring can be performed for the following purposes

- Fast-timescale monitoring for model activation, deactivation, fallback, and switching

- Slow-timescale monitoring for model selection, activation, deactivation, fallback, and switching

- Slow-timescale monitoring for model updates and retraining

RAN1 is still evaluating the need and feasibility of fast-timescale monitoring. Note that slow-scale monitoring not only increases the overhead over the uU interface but also increases the processing requirements at both UE and network sides. This is not desirable as one of the objectives for this study item is to reduce overhead over uU interface.

Proposal 11: Wait for RAN1 progress on which flavors of model monitoring methods should be supported.

# Conclusion

Observation 1: In RAN1#110bis-emeeting [5], RAN1 concluded that data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc.

Observation 2: In the RAN2#121 meeting, RAN2 agreed to separately analyze the data collection requirements and solutions for the different LCM purposes. FFS if general frameworks/solutions could be adopted (proposal 3 [10]).

Observation 3: Chair: There is significant support to aim for evaluating the data collection methods per LCM purpose.

Observation 4: In the RAN2#121bis-emeeting, RAN2 agreed to understand/determine/capture requirements of data collection for the LCM functionalities and document the results.

Observation 5: In the RAN2#121bis-emeeting, RAN2 agreed to study the applicability (and limitations) of each identified data collection framework for each identified LCM purpose, i.e., inference, monitoring, and (offline) training.

Observation 6: Different models may need to be developed for different tier UEs, for different UE internal conditions, and others.

Observation 7: NG-RAN cannot develop UE-specific or UE-vendor-specific models due to the following reasons

- Vendor information is not available on the NG-RAN

- A temporary identity is available at the NG-RAN, therefore NG-RAN cannot identify UE uniquely across different RRC states

Observation 8: For model training a large amount of training data will be required. They need to be stored and processed for the model training.

Observation 9: Training data storage, training data processing, and model training require significantly large storage and computation power. Therefore, model training cannot be supported at the NG-RAN.

Observation 10: To avoid frequent switching of the UE-side models during inference, the model may need to be generalized across gNBs. Training of the UE-side models at NG-RAN may fail to achieve a good model generalization, which will require frequent model transfer/delivery during handovers.

Observation 11: Existing Data collection for the Rel-18/Rel-19 use cases cannot be supported until the feature is deployed, which is a significant constraint on data collection for offline training of UE-side models.

Observation 12: For offline training of the UE-side models, e.g., for training data labeling and model training, UE-side may take auxiliary inputs such as SNR, Doppler, sensor measurements, etc. that do not need to be standardized.

Proposal 1: The following should be considered when considering the data collection for the UE side model’s offline training

- Data collection for model training cannot be supported until the feature is deployed

- Collection of auxiliary information for training data labeling and/or model training

- Data collection for UE-specific model training

- Offline model training requires a large amount of training data requiring training data storage

- Offline model training requires significant computational resources for training data processing and training

- Achieving good model generalizations

Proposal 2: Consider Table Ι for evaluation of identified data collection framework for the UE-side model’s offline training.

Proposal 3: Existing control plane-based methods are not suitable for data collection for offline training of UE-side models.

Observation 13: Based on the online discussions in RAN2#121bis-emeeting, we believe that companies' concerns can be categorized into the following

- Whether/how offline training of RAN-centric use case is supported in EVEX

- Whether/how security is ensured in the EVEX

- Whether/how user privacy is ensured in the EVEX

- Whether/how fine granular control is supported in EVEX

Observation 14:  For supporting offline model training, significantly large data storage space and computation power may be required at the data collection terminating node (or node performing offline model training). Therefore, even if the use cases are RAN-centric, the training may not be performed at the NG-RAN.

Observation 15: To achieve a good generalization performance, the training data should be collected across different NG-RANs.

Proposal 4: In the EVEX framework, the training data may be made available to the interested training entities, for example, ASPs, NWDAF, NRF, trusted AS, External AS, RAN (via AMF), and others.

Observation 16: The EVEX framework supports the following security models

- Transport security

- Data exposure restriction model

- Authentication of data collection client by the Data Collection AF

Proposal 5: The EVEX framework supports the required security models for data collection.

Observation 17: In the EVEX framework, via Service Level Agreements (SLA) with the ASP, operators can configure the DCAF with instructions that control the procedures of permissible UE data collection and permissible event exposure by the DCAF of its collected UE data to event consumers (e.g., NWDAF or ASP).

Observation 18:  Operators can ensure that only those data can be collected from the UE for which relevant user consents are available.

Proposal 6: The EVEX framework ensures user privacy through SLA and user consent, by enabling restrictions that UE can share the information with AF only if appropriate user consent is available.

Observation 19: The EVEX framework supports different Data Access Profile which defines restrictions along the time, user, and location dimensions.

Proposal 7: EVEX supports different Data Access Profile defines restrictions along the time, user, and location dimensions to provide fine granular control for data collection.

Proposal 8: Consider EVEX as one of the options for data collection for offline model training.

Proposal 9: The following should be considered when considering the data collection for model inference

- Terminates at NG-RAN

- Low data volume

- Low latency

Proposal 10: Consider Table Ι for evaluation of identified data collection framework for model inference.

Observation 20: Based on RAN1 discussions, the monitoring can be performed for the following purposes

- Fast-timescale monitoring for model activation, deactivation, fallback, and switching

- Slow-timescale monitoring for model selection, activation, deactivation, fallback, and switching

- Slow-timescale monitoring for model updates and retraining

Proposal 11: Wait for RAN1 progress on which flavors of model monitoring methods should be supported.

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

- RAN1#109-emeeting report

- RAN1#110 meeting report

- RAN1#110bis-meeting report

- R2-2211101, “Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online”, RAN2#119bis-e, Oct 2022.

- Final_Minutes_report_RAN1#111_v100, “Final Report of 3GPP TSG RAN WG1 #111 v1.0.0  (Toulouse, France, 14th – 18th November 2022)”, Nov 2022.

- R2-2300002, “Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France”, RAN2#120, Nov 2022.

- Draft_Minutes_report_RAN1#112_v040, “Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0  (Athens, Greece, 27th February – 3rd March 2023)” Mar 2023.

- Draft_R2_121_meeting_report_v1, “Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece”, Ran2#121, Feb 2023.

- R2-2301440, Outcome of [Post120][054][AIML18] Data Collection, Ericsson, vivo

- R2-2302286, Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)

- TS 23.288, “Architecture enhancements for 5G System (5GS) to support network data analytics services”, Dec. 2022

- TS 26.531, “Data Collection and Reporting; General Description and Architecture”

- Draft_Minutes_report_RAN1#112b-e_v010, “Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0,” April 2023.

- Draft_R2_121bis_meeting_report_v2, “Draft Report of 3GPP TSG RAN WG2 meeting #121bis-e Online,” April 2023.