3GPP TSG-RAN WG2 Meeting #122	R2-2306012

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	7.4.2.2

Source:	Ericsson

Title:	L1 measurements aspects for LTM

Document for:	Discussion, Decision

# 1	Introduction

In RAN WG1 #112, RAN1 sent an LS in [1] with agreements regarding the time alignment establishment and the CSI Measurement configuration for LTM, asking RAN2 to taken them into account.

In RAN2#121 bis the following agreements were made regarding CSI measurement configuration:

The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS.

RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration).

RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

RAN2 assumes that whether filtering, hysteresis, and time-to-trigger are needed for LTM specific L1 measurements is up to RAN1.

FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status.

Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

In this contribution, we propose some further progress to the CSI measurement configuration for LTM.

# 2	Discussion on L1 measurements for LTM

RAN1 has agreed that the measurements to support LTM decisions at the S-DU are like the measurements supporting beam management decisions, such as L1-RSRP for configured LTM candidate cell(s) (and not only for serving cells, as in legacy beam management operation). RAN1 has also agreed that the reports are like in beam management, based on Uplink Channel Control Information (UCI), and that both intra-frequency and inter-frequency measurements are supported.

### 2.1	L1 reporting configuration

Considering that RAN1 has agreed on reporting configurations as in beam management/ UCI (semi-persistent report on PUSCH, and aperiodic report on PUSCH), we proposed RAN2 to consider that an IE like CSI-ReportConfig is the IE in which CSI reports for LTM are configured. A new IE could be created for that, but most of the configuration would have to be repeated, so we see no strong reason for that (at least not yet). Consequently, the CSI-ReportConfig instance configured for an LTM report is included in the ServingCellConfig of the serving cell in which the UE needs to transmit that CSI report for LTM, which is in light with the following agreement from RAN2#121bis:

RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

Based on that, we propose the following:

CSI reports for LTM candidates are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted.

In inter-DU scenarios, a consequence of these RAN1 agreements is that when the CU requests a C-DU to configure an LTM candidate (e.g. cell A in frequency f0), the UE needs to be configure to measure at least the SpCell in the LTM candidate cell configuration i.e., the ServingCellConfig (e.g. for the UE’s current SpCell) needs to be updated to include the a CSI-ReportConfig instance within CSI-MeasConfig for reporting that LTM candidate cell, which would require an additional step between the CU and the S-DU.

When CU receives the UE Context Setup/ Modification Response, in response to a request to add/modify of LTM candidate cell configuration(s), the CU triggers a UE context modification with the S-DU which needs to include further LTM related information, so that the S-DU updates the UE’s current ServingCellConfig, including an instance of the CSI-ReportConfig for LTM within the UE’s CSI-MeasConfig, within the updated CellGroupConfig which is included in the UE Context Modification Response.

When CU receives the UE Context Setup/Modification in response to a request to add/modify of LTM candidate cell configuration(s), the CU triggers a UE context modification with the S-DU so the S-DU updates the UE’s current ServingCellConfig, including an instance of the CSI-ReportConfig for LTM.

It is FFS what further information is included in the UE Context Modification Request so the S-DU generates the CSI-ReportConfig instance(s) for LTM, to be included in the UE Context Modification Response.

Figure 1 UE context modification is needed to update the UE’s current CSI-MeasConfig to include reporting configuration(s) for LTM

Some companies may be tempted to say that this additional step between the CU and the S-DU could be avoided by allowing the CU to configure the reporting configuration for LTM. To keep the existing architecture principles, that would mean a reporting configuration outside CellGroupConfig. However, when we look at the CSI-ReportConfig IE and the agreements in RAN1, it seems the parameters are quite closely tied to the physical layer (which is not surprising), so it would not make sense to let the CU set that without support from the S-DU i.e., at the end, regardless which node configures it, there would need to be some interaction between the S-DU and the CU. Thus, we still believe that because RAN1 has agreed on a reporting configuration relying on UCI, the S-DU is the one configuring it. In addition to it, the RAN2 agreement saying that the reporting configuration for LTM is included within ServingCellConfig seems to exclude other exotic proposals.

In the next section, we discuss what is included on each CSI-ReportConfig instance configured for LTM, and what the S-DU needs to be aware to generate that CSI-ReportConfig instance configured for LTM.

### 2.1	Resource configuration and RS configuration

Now that we discussed how the UE figures out how to report, we need to discuss two issues:

- Issue 1: How the UE determines what to measure e.g. cells, frequencies, SSB(s) and the association between an instance of a CSI-ReportConfig for LTM and what is to be measured and possibly included in that report?

- Issue 2: How the UE obtains the properties of what needs to be measured, which is what RAN1 is calling RS configuration (e.g. PCI or logical ID, SMTC location, frequency location, and SCS)?

### 2.1.1	Issue 1: Resource configuration

In legacy beam management, a CSI report includes one or more SSB(s) or CSI-RS resource(s) of one of the configured serving cell(s). The way that is configured is as follows: an instance of the legacy CSI-ReportConfig IE includes a CSI-ResourceConfigId, which indicates resource set(s) associated to that instance of the CSI-ReportConfig. When a CSI-ResourceConfigId points to a set of SSB(s) of one of the configured serving cell(s), indicated by the parameter carrier, CSI reports associated to that CSI-ReportConfig instance include these SSB(s) pointed by CSI-ResourceConfigId. It may be argued that such a resource associated to a CSI report instance is some sort of measurement object for beam management.

Similarly, a CSI-ReportConfig instance for LTM also needs to point to a resource configuration which reflects what may be possibly included in an instance of a CSI report. Regarding that, in RAN1#111 the following has been agreed for LTM CSI reports:

RAN1#111 (Nov 2022) Agreement - For gNB scheduled L1 measurement report for Rel-18 LTM, report as UCI is supported - Semi-persistent report on PUSCH, and aperiodic report on PUSCH are supported - FFS: periodic and semi-persistent PUCCH In a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included.

If a single CSI report instance includes candidate cell(s) for intra-frequency and/or inter-frequency, a resource configuration pointed within a CSI-ReportConfig needs to include a pointer to LTM candidate cell(s) which may either be intra-frequency LTM candidate cell(s) and/or inter-frequency LTM candidate cell(s). An intra-frequency candidate is a candidate whose SSB frequency and subcarrier spacing (SCS) of the candidate SpCell are the same as the SSB frequency and SCS of the UE’s current SpCell.

The CSI-ReportConfig for LTM needs to include a pointer (e.g., ltm-ResourceConfigId) to intra-frequency LTM candidate cell(s) and/or inter-frequency LTM candidate cell(s).

There are at least three alternatives for defining such a pointer within a CSI-ReportConfig for LTM:

- Alternative a) LTM flag

- Alternative b) List of candidate frequencies

- Alternative c) List of LTM candidate cell(s)

Alternative a) is quite simple, which is one of the main benefits: the CSI-ReportConfig for LTM includes an LTM flag which indicates that the CSI-ReportConfig instance is for LTM. Based on that the UE assumes that all configured LTM candidate cell(s) and their SSBs may be associated to that report. At some point, a maximum number of LTM candidate cells and SSB(s) per LTM candidate cell to be included per CSI report instance would need to be defined, so that the fact that the resource configuration points to all LTM candidate cells of course does not meant the UE would necessarily measure all LTM candidate cells all the time and include all measured SSB(s) of all LTM candidate cell(s) in each CSI report instance for LTM.

In alternative b), each CSI-ReportConfig instance is associated to one or multiple frequencies, so that a CSI report contains measurements on the SSB(s) of the LTM candidate cell(s) whose frequencies are indicated in the CSI-ReportConfig instance. Such an alternative has some similarities with the way L3 measurement reports are configured, except that a ReportConfigNR instance is associated to a single MeasObjectNR, i.e., a single frequency, while in alternative 2, that would need to point to multiple frequencies (if we stick to the full flexibility indicated by the RAN1 agreement).

In alternative c), each CSI-ReportConfig instance is associated to sets of LTM candidate cells, so that a CSI report contains measurements on the SSB(s) of the LTM candidate cell(s) whose set has been indicated in the CSI-ReportConfig instance. Such an alternative allows the full flexibility indicated by the RAN1 agreement in which the CSI report for LTM include measurements on intra-frequency and inter-frequency LTM candidate cells.

The CSI-ReportConfig for LTM includes a pointer (e.g., ltm-ResourceConfigId) that can be modelled according to one of the following alternative to be discussed by RAN2:

  - LTM flag

  - List of candidate frequencies

  - List of candidate cells

### 2.1.2	Issue 2: RS configuration

Assuming now that the UE is aware that a given CSI-ReportConfig instance for LTM is associated to one or more LTM candidate cell(s), e.g., by an indication of frequencies and/or the actual candidate cell(s), the UE needs to know how to find and measure these cells i.e., the RS configuration(s) for these cells e.g. PCI, SSB properties, SMTC, SSB frequency, etc.

In legacy, a CSI-ReportConfig instance points to a serving cell (parameter ‘carrier’) to indicate in which ServingCellConfig the UE finds (within CSI-MeasConfig) the associated resource config ID (CSI-ResourceConfigId included in the CSI-ReportConfig).

CSI-ReportConfig ::=                SEQUENCE {

[…]

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

[…]

Further details on how to find the SSB(s) associated to the indicated serving cell, which is what RAN1 calls RS configuration, could be found by the UE in the ServingCellConfigCommon of the indicated serving cell. However, RAN2 agreed the following:

The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS.

Thus, even though the required properties are available in the ServingCellConfigCommon of each LTM candidate cell configuration, RAN2 has agreed that the same information should be repeated to the UE per LTM candidate cell outside the UE’s current ServingCellConfig and outside the ServingCellConfig of each LTM’s candidate cell configuration.

As these are properties may be different for each LTM candidate cell, they should be provided per cell. And, as they should not be within he UE’s current ServingCellConfig(s) or within the ServingCellConfig of LTM candidate(s), there may be either placed in the UE’s current CellGroupConfig (outside ServingCellConfig) or outside the UE’s current CellGroupConfig, e.g. in the LTM-Config IE in which configuration(s) per candidate are included. That is left FFS based on the stage-2 discussions for the inter-node signalling.

The RS configuration is provided per LTM candidate cell (including PCI or logical ID, SMTC location, frequency location, and SCS).

FFS if the RS configuration is included in the UE’s current CellGroupConfig or in the LTM-Config IE (outside UE’s current CellGroupConfig).

### 2.2	Inter-DU specs for LTM measurement configuration and further details

In the case LTM candidate cells are of a Candidate DU (C-DU) different from the S-DU, one needs to define the signalling to support the LTM measurement configuration discussed above.

Figure 2 inter-node signalling for a CSI measurement configuration for LTM

(Steps 1/2/3) To assist the network to configure LTM, the UE sends a MeasurementReport message (L3 measurement result FFS) to the S-DU containing measurements of neighbouring cells. The S-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the CU. Based on that, the CU determines to initiate L1/L2 triggered mobility configuration for a given UE (e.g. Cells A, B, C, D, E, F) and the associated C-DU. Then, the CU transmits a UE CONTEXT SETUP REQUEST (or a UE CONTEXT MODIFICATION REQUEST) to a C-DU associated to the cells requested to be LTM candidate cell.

(Step 4) If the C-DU accepts the request of LTM configuration, it responds to the CU with a UE CONTEXT SETUP RESPONSE (or a UE CONTEXT MODIFICATION RESPONSE) message including the generated lower layer RRC configuration for the accepted target candidate cell(s), i.e., a CellGroupConfig per accepted LTM candidate. In addition, the C-DU also includes, outside each lower layer RRC configuration for the accepted target candidate cell(s), e.g. in an RRC container, the “RS configuration” for each accepted LTM candidate cell which the UE needs to measure, including per SpCell candidate the Physical Cell Identity (PCI), the SMTC, a frequency location, SCS.

The RS configuration is included by the C-DU to the CU in the response to the request to configure LTM e.g., in the UE CONTEXT MODIFICATION/ SETUP RESPONSE, within an RRC container (together with the lower layer configuration of the LTM candidate cell).

(Step 4a) As discussed in section 2.1, the CU needs to inform the S-DU about LTM, so the S-DU generates an updated version of the UE’s current CellGroupConfig including the CSI-ReportConfig instances for LTM. As discussed in section 2.1, each CSI-ReportConfig instance needs to include an indication of LTM candidate cells to be associated to a report instance, such as an a) LTM flag, b) List of candidate frequencies and/or c) List of candidate cells.

In any of these cases, the S-DU needs to know that LTM is being configured, and the association between LTM candidate cells and LTM configuration ID(s) (denoted ltm-CandidateId in the running CR for RRC), to be later provided to the UE. In addition, the S-DU may also need to be aware of the RS configuration per LTM candidate cell, which may need to be taken into account when the S-DU update the UE’s current CellGroupConfig e.g., adjustment of measurement gaps depending on the frequencies of the LTM candidate cell(s), DRX settings, etc.

The CU transmits to the S-DU the RS configuration per LTM candidate cell and the associated LTM configuration ID (when the CU receives LTM candidate configuration(s) from the C-DU). FFS if this is within a container in a UE CONTEXT MODIFICATION REQUEST (to be decided by RAN3).

An example of the information included in the container is shown below, for the case the C-DU accepts LTM candidate cells A, B, C, D:

LTM candidate ID=1  RS Config [PCI A*, SSB frequency, SMTC]

LTM candidate ID=2  RS Config [PCI B*, SSB frequency, SMTC]

LTM candidate ID=3  RS Config [PCI C*, SSB frequency, SMTC]

LTM candidate ID=4  RS Config [PCI D*, SSB frequency, SMTC]

(Step 4b) Another open issue related to that step between the CU and the S-DU is whether the resource configuration should be determined by the CU or by the S-DU. In other words, as discussed in 2.1, what is associated to a given instance of CSI-ReportConfig for LTM. In the case RAN2 adopts solution a), the solution is rather simple, as the S-DU simply needs to add the LTM flag in the CSI-ReportConfig instances to be configured for LTM, without the need to group LTM candidate cells and/or frequencies. However, in case b) or c) are adopted, either the S-DU or the CU needs to determine a group of cells and/or frequencies to be associated per CSI-ReportConfig instance.

FFS if the S-DU or the CU determines the resource configuration(s) i.e., group of LTM candidate cell or frequencies per CSI-ReportConfig instance.

In the solution based on the S-DU decision, for example, the S-DU receives the RS configuration per LTM candidate cell and the associated LTM candidate ID(s) per candidate, generates the resource configuration in the UE’s current CellGroupConfig (ltm-CSI-ResourceConfigToAddModList) and associates each configuration to a resource configuration ID (LTM-CSI-ResourceConfigId).

For example, assuming solution b), the S-DU could generate the following resource configuration:

LTM-CSI-ResourceConfigId ID=1  LTM candidate ID=1, LTM candidate ID=2;

LTM-CSI-ResourceConfigId ID=2  LTM candidate ID=3, LTM candidate ID=4;

Then, associate resource configuration(s) to instances of the CSI-ReportConfig for LTM, as follows:

CSI-ReportConfig ID=1  LTM-CSI-ResourceConfigId ID=1;

CSI-ReportConfig ID=2  LTM-CSI-ResourceConfigId ID=2;

Another solution, in case b) is adopted, is to define the list of LTM candidate ID(s) in the CSI-ReportConfig instance for LTM. However, in our view, the solution relying on a resource configuration ID is more flexible for later introducing further enhancements e.g. CSI-RS measurements per LTM candidate cell.

In the solution based on the CU decision, for example, the CU needs to indicate to the S-DU the group(s) of cell(s) and/or frequencies, which serves as input for the S-DU to define the resource configuration(s) and the association to CSI-ReportConfig instances for LTM, for example, as follows:

Group 1  LTM candidate ID=1, LTM candidate ID=2;

Group 2  LTM candidate ID=3, LTM candidate ID=4;

Even in that case, the S-DU is still the node which generates the overall CSI measurement configuration.

S-DU transmits to the CU an updated version of the UE’s current CellGroupConfig including resource configuration(s) and CSI-ReportConfig(s) for LTM, each associated to its respective resource configuration.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	CSI reports for LTM candidates are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted.

Proposal 2	When CU receives the UE Context Setup/Modification in response to a request to add/modify of LTM candidate cell configuration(s), the CU triggers a UE context modification with the S-DU so the S-DU updates the UE’s current ServingCellConfig, including an instance of the CSI-ReportConfig for LTM.

Proposal 3	It is FFS what further information is included in the UE Context Modification Request so the S-DU generates the CSI-ReportConfig instance(s) for LTM, to be included in the UE Context Modification Response.

Proposal 4	The CSI-ReportConfig for LTM needs to include a pointer (e.g., ltm-ResourceConfigId) to intra-frequency LTM candidate cell(s) and/or inter-frequency LTM candidate cell(s).

Proposal 5	The CSI-ReportConfig for LTM includes a pointer (e.g., ltm-ResourceConfigId) that can be modelled according to one of the following alternative to be discussed by RAN2:

a.	LTM flag

b.	List of candidate frequencies

c.	List of candidate cells

Proposal 6	The RS configuration is provided per LTM candidate cell (including PCI or logical ID, SMTC location, frequency location, and SCS).

Proposal 7	FFS if the RS configuration is included in the UE’s current CellGroupConfig or in the LTM-Config IE (outside UE’s current CellGroupConfig).

Proposal 8	The RS configuration is included by the C-DU to the CU in the response to the request to configure LTM e.g., in the UE CONTEXT MODIFICATION/ SETUP RESPONSE, within an RRC container (together with the lower layer configuration of the LTM candidate cell).

Proposal 9	The CU transmits to the S-DU the RS configuration per LTM candidate cell and the associated LTM configuration ID (when the CU receives LTM candidate configuration(s) from the C-DU). FFS if this is within a container in a UE CONTEXT MODIFICATION REQUEST (to be decided by RAN3).

Proposal 10	FFS if the S-DU or the CU determines the resource configuration(s) i.e., group of LTM candidate cell or frequencies per CSI-ReportConfig instance.

Proposal 11	S-DU transmits to the CU an updated version of the UE’s current CellGroupConfig including resource configuration(s) and CSI-ReportConfig(s) for LTM, each associated to its respective resource configuration.

# 4	References

R1-2302194, “LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM”, source: RAN1