3GPP TSG-RAN WG2 Meeting #122	R2-2306050

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item:	7.1.3

Source: 	Fujitsu

Title:	Discussion on wake-up timer solution

Document for:	Discussion and Decision

- Introduction

In the RAN2#121bis-e meeting, how to make the NCR-MT back to RRC_CONNECTED after releasing it to RRC_IDLE was discussed. There were two options, i.e., wake-up timer solution and OAM-based solution and no consensus was made.

In this document, we discuss how the NCR-MT should go back to RRC_CONNECTED.

- Discussions

2.1	Supporting wake-up timer solution

In the last RAN2 meeting, it was discussed whether RAN2 went with wake-up timer solution or OAM-based solution. The number of supporting companies for the two solutions were almost even and rapporteur’s proposal was not to define wake-up timer and use OAM-based solution if needed [1]. But no consensus was made.

As the rapporteur’s proposal, OAM-based solution will only rely on the implementation provided from outside of 3GPP network. Therefore, RAN2 does not need to discuss anything on OAM-based solution. This means the OAM-based solution cannot be precluded because it can work with the current specification.

However, the OAM-based solution does not work with the NCR-MT without DRB support. In order to cover this case, we propose to support wake-up timer solution optionally. In other words, if the NCR-MT supports wake-up timer, the NCR-MT indicates it in UE capability, and the network may send RRCRelease message including wake-up timer IE to the NCR-MT.

Proposal 1: The wake-up timer is optionally supported, i.e., if the NCR-MT supports wake-up timer, the NCR-MT indicates it in UE capability, and the network may send RRCRelease message including wake-up timer IE to the NCR-MT.

2.2	Open issues for wake-up timer solution

The wake-up timer solution has open issues which need to be solved. The open issues for the wake-up timer discussed in the last meeting [1] were:

- Whether NCR-MT’s AS layer or NAS layer will initiate RRC connection setup procedure upon timer expires

- Whether the NCR-MT stop the timer when it reselects a different cell

AS layer or NAS layer

RRC of the NCR MT cannot autonomously perform RRC connection establishment procedure. In order for RRC of the NCR-MT to perform RRC connection establishment procedure, the RRC need to be triggered by NAS of the NCR-MT, and NAS message (such as REGISTRATION REQUEST [2]) needs to be transmitted in RRCSetupComplete. The reason is not only the dedicatedNAS-Message is a mandatory IE in RRCSetupComplete message, but also a new security key has to be issued by the core network. As the NCR-MT releases all the UE contexts including security keys when going to RRC_IDLE, a new security key needs to be issued from the AMF by using NAS message when it goes back to RRC_CONNECTED.

Proposal 2: NAS layer of the NCR-MT triggers RRC connection establishment procedure. This is necessary at least for obtaining a new security key from the core network.

Therefore, RAN2 should discuss how NAS message from the NCR-MT will be transmitted at the expiry of the wake-up timer. The easiest way will be RRC of NCR-MT indicates a notification to NAS of NCR-MT at the timer expiry, and NAS of the NCR-MT provides a NAS message by receiving the notification.

Proposal 3: RRC of NCR-MT indicates a notification to NAS of NCR-MT at the timer expiry, and NAS of the NCR-MT provides a NAS message by receiving the notification.

If Proposal 3 is agreed, we propose to send an LS to CT1 to inform the decision as well as the past CN related agreements, such as IAB-MT skips UAC, and request CT1 to update their specifications.

Proposal 4: RAN2 sends an LS to CT1 to inform the decision as well as the past CN related agreements, and request CT1 to update their specifications.

Reselection of different cell

When the NCR-MT in RRC_IDLE running wake-up timer reselects a different cell, the similar approach with the NCR-MT in RRC_INACTIVE should be applied, i.e., the NCR-MT goes back to RRC_CONNECTED so that it can receive side-control configuration from the new gNB. Therefore, we propose if the NCR-MT in RRC_IDLE running wake-up timer reselects a different cell, stops the wake-up timer and considers it is expired.

Proposal 5: If the NCR-MT in RRC_IDLE running wake-up timer reselects a different cell, stops the wake-up timer and considers it is expired.

- Conclusion

Proposal 1: The wake-up timer is optionally supported, i.e., if the NCR-MT supports wake-up timer, the NCR-MT indicates it in UE capability, and the network may send RRCRelease message including wake-up timer IE to the NCR-MT.

Proposal 2: NAS layer of the NCR-MT triggers RRC connection establishment procedure. This is necessary at least for obtaining a new security key from the core network.

Proposal 3: RRC of NCR-MT indicates a notification to NAS of NCR-MT at the timer expiry, and NAS of the NCR-MT provides a NAS message by receiving the notification.

Proposal 4: RAN2 sends an LS to CT1 to inform the decision as well as the past CN related agreements, and request CT1 to update their specifications.

Proposal 5: If the NCR-MT in RRC_IDLE running wake-up timer reselects a different cell, stops the wake-up timer and considers it is expired.

# References

- R2-2304420, " Summary of [AT121bis-e][704][NCR] NCR RRC running CR (ZTE)"

- TS 24.501v18.1.0, “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”