3GPP TSG-RAN WG2 Meeting #122                                        R2-2306077

Inchon, Korea, May 22th – May 26th, 2023

Source:         ZTE Corporation

Title:            Discussion on BW aggregation and RedCap positioning

Agenda item:    7.2.5

Document for:	Discussion and Decision

- Introduction

WID[1] on Expanded and Improved NR Positioning was approved for Rel-18 where items to specify bandwidth aggregation techniques and RedCap positioning are as follows:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

In this contribution, we provide our views on the configuration of bandwidth aggregation and RedCap positioning from RAN2 perspective.

- Discussion

  - Bandwidth aggregation

    - DL-PRS aggregation for on-demand procedure

During RAN1#112-bis-e[2] meeting, the following conclusion is achieved by RAN1:

Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

When PRS bandwidth aggregation is supported from network side, the following agreement in RAN1#112[3] has been made:

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

For UE initiated on-demand PRS, Rel-17 supports preconfigured on-demand PRS where maximum 8 PRS configurations can be configured by LMF to UE as shown in the following box abstracted from 37.355. Then UE can further request the preferred configuration ID(s).

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ... } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... }

Alternatively, as shown in the following box abstracted from 37.355, UE can directly request the preferred PRS parameters for each PFL including the preferred PRS bandwidth, repetition factor, comb size, etc.

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-On-Demand-DL-PRS-PerFreqLayer-r17 NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE { dl-prs-FrequencyRangeReq-r17 ENUMERATED { fr1, fr2, ...}, dl-prs-ResourceSetPeriodicityReq-r17 ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40, p64, p80, p160, p320, p640, p1280, p2560, p5120, p10240, p20480, p40960, p81920, ...} OPTIONAL, dl-prs-ResourceBandwidthReq-r17 INTEGER (1..63) OPTIONAL, dl-prs-ResourceRepetitionFactorReq-r17 ENUMERATED {n2, n4, n6, n8, n16, n32, ...} OPTIONAL, dl-prs-NumSymbolsReq-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-CombSizeN-Req-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-QCL-InformationReqTRPlist-r17 DL-PRS-QCL-InformationReqTRPlist-r17 OPTIONAL, ... } dl-prs-ResourceBandwidthReq This field specifies the requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

Based on the agreement, in the case there is preconfigured on-demand PRS, LMF should inform UE whether/which PFLs are linked/aggregated. Since one On-Demand-DL-PRS-Configuration is associated with one PFL, that is, LMF should inform UE whether/which On-Demand-DL-PRS-Configuration IDs are linked/aggregated.

As shown Step 1 in Figure 1, the grouping information should be informed to UE together with the Rel-17 on-demand PRS configurations. In the figure, it is assumed four on-demand PRS configurations are divided into two groups where configuration#1 and #2 are linked, and #3 and #4 are linked for aggregation.

In step 2, after UE receives the on-demand PRS configurations and the grouping information, UE can request the preferred on-demand PRS configuration ID(s) if the UE does not request PRS aggregation measurement, or can request the preferred group ID(s) if the UE needs PRS aggregation measurement for higher positioning accuracy.

Alternatively, in step 2, UE can directly request the preferred PRS bandwidth value regardless of on-demand PRS configuration where the case of the requested bandwidth larger than the maximum value of the single PFL, e.g. >272 PRBs implies that UE requests PRS bandwidth aggregation measurement for higher positioning accuracy.

Figure 1. On-demand PRS for PRS bandwidth aggregation

For LMF-initiated on-demand PRS, it only supports LMF directly requests preferred PRS parameters including bandwidth. Similar as Option 2 in Figure 1, Rel-18 can be enhanced to support that the requested bandwidth is larger than the maximum value of the single PFL, which implies that LMF requests PRS bandwidth aggregation. Since it depends on RAN3’s design, we suggest to send LS to RAN3 on LMF-initiated on-demand PRS request.

Proposal 1: For UE-initiated on-demand PRS request for PRS bandwidth aggregation, support

- On-demand PRS configuration can include the group information where the linked on-demand PRS configuration IDs are in the same group for aggregation

- UE can request on-demand PRS configuration ID(s) or group ID(s)

- UE can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

Proposal 2: For LMF-initiated on-demand PRS request for PRS bandwidth aggregation, support

- LMF can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN3

    - UL-SRS aggregation in RRC_INACTIVE

In Rel-17, semi-persistent and periodic SRS for positioning are supported for UE in RRC inactive mode. The positioning SRS configured by RRCRelease signaling, and the details are shown in following according to 38.331, where SRS configuration for NUL or SUL can be within or outside the initial BWP. It is noted that the SRS is assumed within the same serving cell of the initial BWP.

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-changeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

For Rel-18, SRS carrier aggregation has been supported for UEs in both RRC_CONNECTE sate and RRC_INACTIVE state as shown in the following agreement.

Agreement Positioning SRS bandwidth aggregation is supported for UEs in RRC_CONNECTED. Positioning SRS bandwidth aggregation is supported for UEs in RRC_INACTIVE state. - For the details, Rel-17 positioning SRS configuration for UE in RRC_INACTIVE state outside initial UL BWP can be the starting point

To support SRS carrier aggregation in RRC_INACTIVE state, an additional SRS configuration in a serving cell other than the one of initial BWP should be introduced and informed by RRCRelease signaling. Because intra-band contiguous carrier aggregation is only supported among NUL carriers, the newly introduced SRS configuration only for NUL is sufficient.

Proposal 3: To support two or three carrier aggregation of positioning SRS transmission for UE in RRC inactive mode, introduce one or two NUL carriers with respective SRS configuration, where the newly introduced carrier(s) and the carrier of the initial BWP are intra-band NUL contiguous carriers.

Relationship with communication CA

In RAN1#112bis-e meeting, the following agreement was made.

Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA.

For UE capability, it may be easy to introduce a new BandCombinationList-PosCA-r18, where only one band for intra-band contiguous CA is included in each BandCombination. For RRC configuration of SRS in RRC_CONNECTED, we think the existing RRC structure can be reused.

Proposal 4: For positioning SRS bandwidth aggregation in RRC_CONNECTED state which can be decoupled from communication CA, support to introduce a new UE capability BandCombinationList-PosCA-r18.

  - RedCap positioning

    - DL-PRS hopping

RAN1#112 has confirmed to support frequency hopping of positioning PRS as follows:

Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

- RAN1 confirms that hopping is performed at receiver side. For PRS, the gNB sends PRS with a normal bandwidth, meanwhile at UE side, the frequency domain resource of receiving one PRS is divided into several parts, each part corresponds to a frequency hop, and several hops are received in different time periods, respectively. There are 2 cases of PRS repetition:

- If a PRS is inter-slot repeated with a repetition factor, UE should receive different frequency parts per slot per repetition instance;

- If a PRS is intra-slot configured and PRS symbol is larger than PRS comb size, the PRS is actually per comb-size-symbol(s) repeated within a slot, UE should receive different frequency parts per per comb-size-symbol(s) of a PRS resource within a slot.

- For the above 2 cases, RedCap UE can use on-demand PRS request to request hopping. Currently on-demand PRS request contains PRS bandwidth, PRS repetition factor, PRS symbol and PRS comb size. We think these IEs (and the value ranges of these IEs) are enough for RedCap UE to indicate NW of PRS reception hopping, for example, since NW knows the RedCap UE identity and the maximum bandwidth it supports (according to the reported UE capability), if UE requests a PRS bandwidth larger than its UE capability, it implies UE requests a Rx PRS hopping, the requested PRS repetition factor implies the number of inter-slot Rx hopping, and the requested PRS comb size and PRS symbols imply the number of intra-slot Rx hopping. If UE requests a PRS bandwidth equal or smaller than its UE capability, it implies UE does not request a Rx PRS hopping.

- Proposal 5: RedCap UE can use on-demand PRS procedure to request Rx PRS hopping without additional specification impact in RAN2, i,e., if UE requests a PRS bandwidth larger than its UE capability, it implies UE requests a Rx PRS hopping.

Since RAN1 agreed on the per-PRS resource Rx hopping, we suggest to introduce one or more of the following parameters per PRS resource in the DL-PRS configuration in the assistance data.

- Number of hops. This parameter will affect positioning accuracy. More hops will compose a larger bandwidth which brings a higher positioning accuracy.

- Number of overlapped RB. This parameter is used for phase adjustment between adjacent hops. It will affect positioning accuracy.

- Total bandwidth of all hops. Actually, this is equivalent to the number of hops (e.g., each hop has the same bandwidth). One of these two parameters can be finally selected.

Proposal 6: For Rx PRS hopping, support to introduce one or more of the following parameters per PRS resource in the DL-PRS configuration in the assistance data:

- Number of hops

- Number of overlapped RB between hops

- Total bandwidth of all hops

The DL-PRS configuration in the assistance data can be via broadcast or dedicate LPP signaling.

When PRS configuration is broadcasted with DL-PRS frequency hopping for all UEs, but the RedCap UE’s location accuracy requirement is not high, the RedCap UE does not have to perform hopping and combine the measurement of multiple hops. In this case, the RedCap UE should report which hop(s) the UE is used for the measurement.

When PRS configuration with DL-PRS frequency hopping is given to the RedCap UE via LPP dedicate signaling, the RedCap UE may not be able to perform hopping and combine the measurement of multiple hops all the time due to collision. Therefore, a fallback behaviour is needed to let UE report the corresponding hop indication for each measurement result.

Proposal 7: RedCap UE should report the corresponding hop indication for each measurement result, wherein the hop indication includes the frequency range.

    - UL-SRS hopping

For positioning SRS hopping, RAN1’s consensus is not to use BWP switching to perform hopping because of latency consideration. From signaling configuration view, the SRS for RedCap UE Tx hopping can be configured as a new SRS configuration within the legacy SRS-Config IE. The new SRS configuration should have the new bandwidth which is larger than the current BWP.

Proposal 8: Support to configure a new SRS configuration within the legacy SRS-Config IE to indicate SRS hopping for RedCap UE.

- Conclusion

- In this section we provide the following proposals:

- For bandwidth aggregation:

Proposal 1: For UE-initiated on-demand PRS request for PRS bandwidth aggregation, support

- On-demand PRS configuration can include the group information where the linked on-demand PRS configuration IDs are in the same group for aggregation

- UE can request on-demand PRS configuration ID(s) or group ID(s)

- UE can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

Proposal 2: For LMF-initiated on-demand PRS request for PRS bandwidth aggregation, support

- LMF can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN3

Proposal 3: To support two or three carrier aggregation of positioning SRS transmission for UE in RRC inactive mode, introduce one or two NUL carriers with respective SRS configuration, where the newly introduced carrier(s) and the carrier of the initial BWP are intra-band NUL contiguous carriers.

Proposal 4: For positioning SRS bandwidth aggregation in RRC_CONNECTED state which can be decoupled from communication CA, support to introduce a new UE capability BandCombinationList-PosCA-r18.

- For RedCap:

- Proposal 5: RedCap UE can use on-demand PRS procedure to request Rx PRS hopping without additional specification impact in RAN2, i,e., if UE requests a PRS bandwidth larger than its UE capability, it implies UE requests a Rx PRS hopping.

Proposal 6: For Rx PRS hopping, support to introduce one or more of the following parameters per PRS resource in the DL-PRS configuration in the assistance data:

- Number of hops

- Number of overlapped RB between hops

- Total bandwidth of all hops

Proposal 7: RedCap UE should report the corresponding hop indication for each measurement result, wherein the hop indication includes the frequency range.

Proposal 8: Support to configure a new SRS configuration within the legacy SRS-Config IE to indicate SRS hopping for RedCap UE.

- References

- RP-223549, New WID on Expanded and Improved NR Positioning, 2022-12.

- Report of 3GPP TSG RAN WG1 meeting #112-bis-e, 2023-04.

- Report of 3GPP TSG RAN WG1 meeting #112, 2023-03.