3GPP TSG-RAN WG2 Meeting #122          	          R2-2306095

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	7.16.2.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on model control and others

Document for:	Discussion and decision

Introduction

At RAN2#121 meeting, the SI AIML for air interface was discussed. Some agreements are achieved regarding the following aspects: data collection, model transfer/delivery, model ID & UE capability and general aspect (as shown in Annex 5.3 at the end of this paper). The discussion about data collection, model transfer/delivery and model ID will be provided in our other contribution papers dedicated for 7.16.2.1 – 7.16.2.3 sections in agenda. Therefore, in this discussion paper, we discuss model controls (beyond/other than model transfer/delivery) and model monitoring, following RAN2#121b-e agenda.

Discussion

Discussion on AI/ML Model control

The RAN2#121b-e meeting has made some agreements as below in terms of model control (beyond/other than model transfer/delivery):

For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

For model control, as the CSI compression and beam management use cases are listed in one category while positioning is listed separately, we organize our 2.1 section accordingly as below.

Model control for CSI/BM

For functionality-based LCM, we already have discussion in [1] which is dedicated for agenda item 7.16.2.1.

Considering UE-sided/UE part for two-sided models, for UE-initiated or gNB-initiated model control in CSI/BM use cases, we organize our discussion from the perspectives of (1) definition of UE-initiated and gNB-initiated model control, and (2) the difference among use cases regarding model control.

- Definitions

We think it is important to clarify the meaning of UE-initiated and gNB-initiated model control. Our views are as below:

gNB-initiated: gNB makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

UE-initiated: UE may detect the need for model control and request to the gNB. In this way, UE’s request for model control is reported to gNB, gNB makes a decision and indicate UE to execute accordingly.

For gNB-initiated way, we think it is aligned with the handling of lots of legacy features, and some model-control-related decision policies and/or configurations may be pre-configured within NW (maybe up to implementation), and when it fulfils, gNB could decide and trigger model control.

For UE-initiated way, this may need some configurations from network in advance, e.g. the performance threshold configuration, etc.

According to the definition given above, since the decision is made by network, there may be no RAN2 impacts for gNB-initiated model control. For UE-initiated model control, UE needs to request to NW and NW may send indication to UE for further model control, which may result in potential RAN2 impacts. Therefore, the following observation is made:

Proposal 1: For CSI/BM, RAN2 to agree on the definitions as below:

gNB-initiated: gNB makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

UE-initiated: UE may detect the need for model control and request to the gNB. In this way, UE’s request for model control is reported to gNB, gNB makes a decision and indicate UE to execute accordingly.

- Difference among use cases regarding model control

In this section, we give a brief discussion on how the different cases are different (e.g. applicability to UE-sided vs network sided model) for model control.

For CSI compression with two-sided model, the decoder is the gNB-sided model and the encoder is the UE-sided model. In a common case, model control would be the next-step after model monitoring. Therefore, if decoder needs to be selected/(de)activated/switched/fallback, gNB-initiated model control is applicable, while gNB may need to collect some information (e.g. measurement for monitoring) from UE side. It should be noted that, decoder and encoder need to be aligned to correctly proceed the AI-based CSI compression. Therefore, if decoder is, e.g. switched at gNB-side, gNB needs to send indication to UE so that UE can synchronize encoder in time, which may result in RAN2 impacts. This means gNB-initiated model control is also applicable for encoder. In a word, gNB-initiated model control is applicable for both gNB-sided and UE-sided model for CSI compression use case. On the contrary, for UE-initiated model control in CSI compression, if UE want to, e.g. switch encoder, then UE may request to NW and NW decides to switch models both for gNB-part and UE-part models in the same time. Therefore, for two-sided model, UE-initiated model control may be applicable, but it’s not clear how and when UE knows itself wants to switch/(de)activate models. The motivation for UE-initiated model control for CSI compression should be further discussed.

For beam management with single-sided model, similar to the discussion above, gNB-initiated model control is applicable for both gNB-sided model (no spec impacts) and UE-sided model (may have spec impacts, e.g. sending indication from gNB to UE). UE-initiated method may not be applicable for gNB-side model, e.g., the UE should not request the NW to switch models if the model works well at gNB side; but it’s applicable for UE-sided model since the UE may monitor the performance of models and know when to switch/(de)activate the model, then UE send requests to NW for model control.

To summarize the discussion above, we provide the Summary Table as below:

|  | UE-sided model | gNB-sided model | Two-sided model |
| --- | --- | --- | --- |
|  | BM | BM | CSI compression |
| gNB-initiated | Applicable | Applicable No spec impacts | Applicable |
| UE-initiated | Applicable | Not applicable | Applicable |

Proposal 2: For CSI compression and Beam management, gNB-initiated method is applicable for both uses cases.

Proposal 3: For CSI compression and Beam management, RAN2 to discuss the need of UE-initiated method.

Model control for Positioning

For functionality-based LCM, we already have discussion in [1] which is dedicated for agenda item 7.16.2.1.

Considering UE-sided models, for UE-initiated/gNB-initiated/LMF-initiated model control in Positioning use cases, the definitions are as below:

LMF-initiated: LMF makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

gNB-initiated: gNB makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

UE-initiated: UE may detect the need for model control and request to the gNB/LMF. In this way, UE’s request for model control is reported to gNB/LMF, gNB/LMF makes a decision and indicate UE to execute accordingly.

Proposal 4: For Positioning, RAN2 to agree on the definitions as below:

LMF-initiated: LMF makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

gNB-initiated: gNB makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

UE-initiated: UE may detect the need for model control and request to the gNB/LMF. In this way, UE’s request for model control is reported to gNB/LMF, gNB/LMF makes a decision and indicate UE to execute accordingly.

For UE-initiated model control, positioning Case1/2a with UE-sided model is applicable, since UE can monitor the performance of model and detect the need for switching/(de)activation/fallback in the first place. Then UE can request to NW for further model control indication. And for positioning Case3a with gNB-sided model and Case2b/3b with LMF-sided model, UE-initiated model control may not be applicable, UE may just provide some RS and/or measurements for the NW to initiate and proceed model control.

For gNB-initiated model control, positioning Case1/2a with UE-sided model is not applicable, since Case1/2a is UE-based/LMF-based positioning where the final positioning is done within UE/LMF. gNB may have no performance information of UE-side model during the inference phase, where gNB only provide PRS to UE, so gNB may not be appropriate to initiate model control. For positioning Case3a with gNB-sided model, it’s natural for gNB to initiate the model control since gNB can monitor the inference performance and decides whether to switch/(de)activate/fallback. UE may provide some SRS to gNB, while information about final positioning from LMF may be needed to proceed model control at gNB. And for positioning Case2b/3b with LMF-sided model, gNB-initiated method is not applicable. gNB may just report/transfer some measurements to LMF.

For LMF-initiated method, positioning Case1 is not applicable, but Case2a may be applicable since the model inference output would be sent to LMF for final positioning, where LMF may initiate model control if LMF somehow finds the positioning accuracy degrades. This is same for the positioning Case3a with gNB-sided model. And for positioning Case2b/3b with LMF-sided model, it’s natural for LMF to initiate the model control since LMF holds the model and monitor the performance.

To summarize the discussion above, we provide the Summary Table as below:

|  | UE-sided model (Case 1/2a) Direct/assisted positioning | gNB-sided model (Case 3a) assisted positioning | LMF-sided model (Case 2b/3b) Direct positioning |
| --- | --- | --- | --- |
| UE-initiated | Applicable | Not applicable | Not applicable |
| gNB-initiated | Not applicable | Applicable Maybe some interaction w/ LMF | Not applicable |
| LMF-initiated | Not applicable for Case1 May applicable for Case2a | May applicable | Applicable |

Proposal 5: For Positioning, gNB-/LMF-initiated method is applicable.

Proposal 6: For Positioning, RAN2 to discuss the need of UE-initiated method.

For the above discussions, we think NW should be responsible for the decisions, so all our proposals can be considered as NW-decided approach.

For NW-decided approach, as analysed in our contribution R2-2301577 from last RAN2#121 meeting, if NW sends indications (but not model information) to UE to operate model(s), we think at least RRC signalling should be supported, the reasons are listed as below:

- It is secure, and TS 38.331 has the following definition

  - Once AS security is activated, all RRC messages on SRB1, SRB2, SRB3 and SRB4, including those containing NAS messages, are integrity protected and ciphered by PDCP.

- For the transmission delay, we think one DL RRC message may take tens of milliseconds or even less, which should be enough for transmitting the indications

- It is flexible

Besides RRC signalling, MAC CE may be considered.

For the transmission of indications from the NW to the UE, we think it should at least include the following components:

- Transmission delay in Uu interface

- Processing delay, i.e. the time between when the UE gets the indications and when it really applies the model. Usually the processing delay is longer than the transmission delay

For transmission delay, we think MAC CE may take less delay than RRC signalling, e.g. by tens of milliseconds. However, for processing delay, we are not sure whether it will be much longer than transmission delay or not. If yes, we think the total delay will not make much differences between two options.

Additionally, DCI signalling could also be considered for model controlling indications that sent by NW to UE. For the transmission delay, DCI, as a L1-signalling, may take even less delay than MAC CE. However, if the security requirement is fulfilled or not for DCI to carry a model controlling indications is still not clear, for example, RRC signalling comes with integrity protection and encryption. And similar to the discussion above, we are not sure if the processing delay of DCI is much longer than transmission delay or not.

Observation 1: For transmission of indications from NW to the UE, the following components are included: transmission delay and processing delay.

Observation 2: Processing delay may dominate the whole delay for model control like model switching. Whether to use RRC signalling/MAC CE/DCI signalling depends on the ratio of processing delay in the whole delay, which might require RAN1 inputs. And it’s also related to the sending frequency of model control command, e.g. how frequently the model activated/deactivated.

Proposal 7: For NW sending indications to UE for model control purposes, e.g. model activation, model switching, RRC signalling should be supported.

Discussion on AI/ML Model monitoring

Model monitoring identifies the adaptiveness between AI/ML model and environment, which provides the main reference to activate/deactivate/select/switch/update model in time and avoid performance decline. Model monitoring requires to collect information that reflects the model status/effects, and thereby the corresponding measurement and report can be considered for potential spec impact. For the monitoring manner, depending on the execution node (e.g., gNB and UE), it can be classified as following three cases:

- Case1. UE-side Monitoring for AI/ML Model

  - UE collects inputs for monitoring and calculates monitoring KPI

  - UE makes the decision(s) of model selection/activation/deactivation/switching/fallback operation.

- Case2. NW-side Monitoring for AI/ML Model

  - NW (e.g. gNB) collects inputs for monitoring and calculates monitoring KPI

  - NW (e.g. gNB) makes the decision(s) of model selection/activation/deactivation/switching/fallback operation.

- Case3. Hybrid Monitoring for AI/ML Model

  - UE collects inputs for monitoring, calculates monitoring KPI and feedback the KPI to NW (e.g. gNB)

  - NW (e.g. gNB) makes the decision(s) of model selection/activation/deactivation/switching/fallback operation.

Case1 can be applied to monitor the UE-side model, with potentially requesting Network to send assistant signals (AI/ML-related RS, etc.) to facilitate the UE to obtain monitoring inputs. Since Network is responsible for the performance of entire cellular network (so it takes the obligation to guarantee the robust performance), Network can configure the threshold criterion (e.g. threshold for system throughput and/or intermediate KPI, etc.) to UE to facilitate the UE to make a proper decision. Additionally, after UE making decision, UE may need to report the decision to Network and Network can indicate the UE to execute the decision accordingly. Case2 is applicable to at least Network-side model and the two-sided model, in which Network can collect the monitoring inputs and calculate the KPI. The inputs can be the feedback from UE including ground-truth labels or instantaneous performance indicator (e.g., throughput, ACK/NACK, etc.). Case3 is applicable to Network-side model, UE-side model and Two-sided model.

As for the metrics for monitoring discussed in our previous companion contribution R1-2208428, there are two options for model monitoring: 1) inference accuracy and 2) system performance. For Metric1, taking CSI compression as an example, with UE sending the ground-truth CSI label to Network, Network can monitor the instantaneous inference accuracy of AI/ML-based CSI feedback. Also, for BM, UE can feedback the measured ground-truth optimal beam index obtained during monitoring window for monitoring of the Network-side BM. For positioning, PRU can feedback the instantaneous ground-truth location to Network for monitoring the positioning accuracy. For Metric2, it can be useful to reflect the average and overall performance of the AI/ML model as they can be statistics over a long period of time.

The spec impacts include the signalling for supporting the model monitoring. For example, the report of the monitoring metrics and/or monitoring decision, configuration of triggering-events and/or threshold, data collection-related signalling such as enhancement of existing RS (e.g. CSI-RS and/or SRS), etc. Therefore, the following proposal is made:

Proposal 8: It is proposed RAN2 to discuss case 2 and 3:

- Case2. NW-side Monitoring for AI/ML Model

  - Network collects data and makes decision, and then Network indicates the UE to execute the decision accordingly

- Case3. Hybrid Monitoring for AI/ML Model

  - Network may configure a threshold criterion to UE to facilitate UE to make decision.

  - UE may need to report the decision to Network and Network indicates the UE to execute the decision accordingly.

Proposal 9: It is proposed RAN2 to discuss the benefits and impacts for case 1 UE-side Monitoring for AI/ML Model.

According to the last RAN1#112bis-e meeting, there are some agreements have been achieved regarding the model monitoring, e.g. AIML based positioning. Whether to provide ground truth label (and associated label quality) via air interface may have RAN2 specification impact. In case of label-based monitoring, monitoring metric could be the statistics of the difference between model output and provided ground truth label. Otherwise, monitoring metric for label-free monitoring could be the statistics of measurements compared to the statistics associated with training data. Similar to the discussion above, whether to transfer these monitoring metrics via air interface for further model control decision and/or execution may have RAN2 specification impacts. What’s more, RAN1 has made some progress on the discussion of functional mapping of the entities, e.g. identified some entities to derive monitoring metrics. The workload for RAN2’s discussion of model monitoring procedures would be much less if RAN1 confirms some requirements for monitoring procedures and/or metrics. So based on the discussion above, we observed that:

Observation 3: For model monitoring, RAN1 has made some progress, e.g. identified some entities to derive monitoring metrics, which may lead to RAN2 spec impacts. If RAN1 can confirm some requirements, e.g. monitoring procedures and metrics, RAN2 can check possible RAN2 impacts.

Conclusion

For this paper, we mainly discuss AI/ML Model control (beyond/other than model transfer/delivery)and model monitoring. It is proposed:

For AI/ML model control

Observation 1: For transmission of indications from NW to the UE, the following components are included: transmission delay and processing delay.

Observation 2: Processing delay may dominate the whole delay for model control like model switching. Whether to use RRC signalling/MAC CE/DCI signalling depends on the ratio of processing delay in the whole delay, which might require RAN1 inputs. And it’s also related to the sending frequency of model control command, e.g. how frequently the model activated/deactivated.

Proposal 1: For CSI/BM, RAN2 to agree on the definitions as below:

gNB-initiated: gNB makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

UE-initiated: UE may detect the need for model control and request to the gNB. In this way, UE’s request for model control is reported to gNB, gNB makes a decision and indicate UE to execute accordingly.

Proposal 2: For CSI compression and Beam management, gNB-initiated method is applicable for both uses cases.

Proposal 3: For CSI compression and Beam management, RAN2 to discuss the need of UE-initiated method.

Proposal 4: For Positioning, RAN2 to agree on the definitions as below:

LMF-initiated: LMF makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

gNB-initiated: gNB makes a decision and then trigger model control (e.g. send indications to UE to do model selection/(de)activation/switching/fallback).

UE-initiated: UE may detect the need for model control and request to the gNB/LMF. In this way, UE’s request for model control is reported to gNB/LMF, gNB/LMF makes a decision and indicate UE to execute accordingly.

Proposal 5: For Positioning, gNB-/LMF-initiated method is applicable.

Proposal 6: For Positioning, RAN2 to discuss the need of UE-initiated method.

Proposal 7: For NW sending indications to UE for model control purposes, e.g. model activation, model switching, RRC signalling should be supported.

For AI/ML model monitoring

Observation 3: For model monitoring, RAN1 has made some progress, e.g. identified some entities to derive monitoring metrics, which may lead to RAN2 spec impacts. If RAN1 can confirm some requirements, e.g. monitoring procedures and metrics, RAN2 can check possible RAN2 impacts.

Proposal 8: It is proposed RAN2 to discuss case 2 and 3:

- Case2. NW-side Monitoring for AI/ML Model

  - Network collects data and makes decision, and then Network indicates the UE to execute the decision accordingly

- Case3. Hybrid Monitoring for AI/ML Model

  - Network may configure a threshold criterion to UE to facilitate UE to make decision.

  - UE may need to report the decision to Network and Network indicates the UE to execute the decision accordingly.

Proposal 9: It is proposed RAN2 to discuss the benefits and impacts for case 1 UE-side Monitoring for AI/ML Model.

References

- R2-2306092, Discussion on architecture and general, Huawei, HiSilicon