3GPP TSG-RAN WG2 Meeting #122 	R2-2306206

# Incheon, Korea, May 22-26, 2023

Source:	TCL Communication

Title:	Discussion on multiple-PUSCHs CG for XR

Agenda item:	7.5.4.3

Document for:	Discussion and decision

- Introduction

In RAN1 #112bis meeting, the multiple-PUSCHs CG for XR has been extensively discussed and concluded as following:

For TDRA design for multi-CG PUSCH, prioritize Alt-A1, Alt-B, and Alt-C2 for further downscoping and/or modification from corresponding agreement in RAN1#112. - FFS: How to address TDD configuration issue

- Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112) - FFS details

The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - FFS: Details including time duration/range Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

In this contribution we will discuss the multiple-PUSCHs CG for XR from RAN2 perspective.

- Discussion

  - multiple-PUSCHs CG configuration

- Non-integer periodicity

The periodicity configuration for the DRX multiple-PUSCHs CG need to be enhanced due the non-integer periodicity of the XR video traffic. One aspect is that some new integer value for periodicity should be introduced for the periodicity configuration; another aspect is how to adapt more than one periodicity with integer value to the the non-integer periodicity of the XR video traffic.

However the similar issue are being discussed for DRX configuration and the similar solution is preferred, so we can wait the conclusion about DRX configuration for the non-integer periodicity of the XR video traffic.

Proposal 1: the solution for multiple-PUSCHs CG configuration should be similar as that for DRX configuration which is being discussed

- Special parameters for multiple-PUSCHs CG

Based on the potential TDRA design, the number of CG PUSCH transmission occasions in a period and/or the interval between two adjacent CG PUSCH transmission occasions could be configured by high layer differently for different TDRA alternatives. Moreover, for different types of CG, the configuration could be also different. For the Type 1 CG, all the special parameter for multiple-PUSCHs CG should be configured by high layer while only part of the special parameters should be configured by high layer.

Proposal 2: the number of CG PUSCH transmission occasions and/or the interval between two adjacent CG PUSCH transmission occasions in a period could be configured by high layer differently for different Types of CG and TDRA alternatives

  - Unused CG PUSCH transmission occasions indication

Due to jitter and non-integer periods, there are four basic cases of Unused CG PUSCH transmission occasions in actual usage after configuring multiple-PUSCHs CG as illustrated in Figure 1:

Case 1: Multiple consecutive CG PUSCH transmission occasions at the beginning are not used.

Case 2: Several CG PUSCH transmission occasions in the middle are not used.

Case 3: Multiple CG PUSCH transmission occasions at the end are not used.

Case 4: Insufficient configured or remaining CG PUSCH transmission occasions.

Figure1

According to the agreed solution in RAN1 for UTO-UCI, in each used CG PUSCH transmission occasion, a UTO-UCI should be included with a bitmap to indicate whether each followed up CG PUSCH transmission occasion will be used or not, while no UTO-UCI transmission in the any unused CG PUSCH transmission occasions. Based on this principle, we analyse how the UTO-UCI will be transmitted for each case.

Case 1: Multiple consecutive CG PUSCH transmission occasions at the beginning are not used

In this case, since Multiple consecutive CG PUSCH transmission occasions at the beginning are not used, UE can’t transmit a UTO-UCI with the CG PUSCH transmission in any of those occasions. As a consequence, gNB could only assume that all those front CG PUSCH transmission occasions will be used and can’t not reuse them. This situation will continue until a real CG PUSCH transmission with XR traffic date occurs. Obviously, it degrades the efficiency of the radio resources and is not desirable and acceptable.50758738-37-39

As is well know, whether CG PUSCH transmission occasion is used or not is controlled by MAC. In order to transmit a UTO-UCI to indicate the following unused CG PUSCH transmission occasions, a special BSR could be generated in MAC in UE to trigger the first CG PUSCH transmission occasion to be used. Based on the agreed solution in RAN1, a UTO-UCI could be transmitted along with the special BSR in the first CG PUSCH transmission occasion even there is no XR traffic data transmission.

Proposal 3: a special BSR or other MAC CE could be generated in MAC to trigger the first CG PUSCH transmission with the UTO-UCI when no XR traffic data for the first CG PUSCH transmission occasion

Case 2 and case 3: Several CG PUSCH transmission occasions in the middle or at the end are not used

Based on the agreed solution in RAN1, In these two cases, the UTO-UCI could be transmitted in any of the used CG PUSCH transmission occasions before the first unused CG PUSCH transmission occasion.

Case 4: Insufficient configured or remaining CG PUSCH transmission occasions

There are some cases where the CG resources are insufficient for the uplink XR traffic transmission. One case is that CG is not sufficient to transmit the XR traffic in some periods. Another case is that the remained CG is not sufficient due to some unused CG resources even if CG resources are over-configured. In those case, according to the current mechanism, UE could request one or more supplementary dynamic grants by a BSR for the remained XR traffic data transmission followed up the CG PUSCH occasions.

One issue for the supplementary DG is the potential additional delay for the XR traffic transmission due to the Active time for DG is not followed up closely to the last CG PUSCH occasion as illustrated in Figure 2 and could be FFS.

Figure 2

Proposal 4: a BSR could be used to request supplementary DG in the case where CG resources are insufficient and the potential additional delay for the DG could be FFS

- Conclusion

In this contribution, we further discussed the multiple-PUSCHs CG for XR and provided some observations and our proposals as follows:

Proposal 1: the solution for multiple-PUSCHs CG configuration should be similar as that for DRX configuration which is being discussed

Proposal 2: the number of CG PUSCH transmission occasions and/or the interval between two adjacent CG PUSCH transmission occasions in a period could be configured by high layer differently for different Types of CG and TDRA alternatives

Proposal 3: a special BSR or other MAC CE could be generated in MAC to trigger the first CG PUSCH transmission with the UTO-UCI when no XR traffic data for the first CG PUSCH transmission occasion

Proposal 4: a BSR could be used to request supplementary DG in the case where CG resources are insufficient and the potential additional delay for the DG could be FFS

- References

- 3GPP TR 38.835: "Study on XR enhancements for NR".