3GPP TSG-RAN WG2 Meeting #122							R2-2306221

Incheon, Korea, 22th May – 26th May 2023

Agenda item:	7.24.2 (TEI18 by R2)

Source:	Samsung, Ericsson, Huawei, HiSilicon

Title:	Introduction of ‘multiple QoS’ class in positioning

Document for:	Discussion & Decision

- Introduction

SA2 WI 5G_eLCS_Ph2 WI has made in Release 17 the introduction of ‘Multiple QoS’ class between LCS client and LMF. In this paper, it is discussed how this new QoS class can be used in RAN for reducing LPP message transaction latency as well as signaling overhead.

- Discussion

  - Background of introduction of multiple QoS class

SA2 introduced ‘multiple QoS’ class (S2-2105125). The motivation is that there will be the case / area that the LCS client can work with a range of QoS levels from the target UE’s. For example, the LCS client would prefer a certain QoS level, but could also work with LCS reports up to a minimum QoS level and compensate the accuracy reduction by other means (e.g. through AI/ML based post-processing techniques). Therefore, LCS client gives some margin on required QoS so that the LMF can handle with more flexibility on the QoS requirement.

  - Current operation between LCS client and LMF with respect to QoS class

Now new ‘multiple QoS’ class was added, which results in total three types of QoS class can be indicated to LMF. As in below excerpt from Ts 23.273, ‘best effort’, ‘multiple QoS’, and ‘assured’ class has their own adherence level to the accompanied accuracy value(s).

## 4.1b LCS Quality of Service LCS Quality of Service is used to characterise the location request. It can either be determined by the operator or determined based on the negotiation with the LCS client or the AF. It is optional for LCS client or the AF to provide the LCS Quality of Service in the location request. LCS Quality of Service information is characterised by 3 key attributes: - LCS QoS Class as defined below. - Accuracy: i.e. Horizontal Accuracy (see clause 4.3.1 of TS 22.071 [2]) and Vertical Accuracy (see clause 4.3.2 of TS 22.071 [2]. - Response Time (e.g. no delay, low delay or delay tolerant as described in clause 4.3.3 of TS 22.071 [2]). NOTE 1: One or two QoS values for Horizontal Accuracy, Vertical Accuracy can be provided in the location request in addition to a preferred accuracy when LCS QoS Class is set to Multiple QoS Class. The LCS QoS Class defines the degree of adherence by the Location Service to another quality of service parameter (Accuracy), if requested. The 5G system shall attempt to satisfy the other quality of service parameter regardless of the use of QoS Class. There are 3 LCS QoS Classes: - Best Effort Class: This class defines the least stringent requirement on the QoS achieved for a location request. If a location estimate obtained does not fulfil the other QoS requirements, it should still be returned but with an appropriate indication that the requested QoS was not met. If no location estimate is obtained, an appropriate error cause is sent. - Multiple QoS Class: This class defines intermediate stringent requirements on the QoS achieved for a location request. If the obtained location estimate does not fulfil the most stringent (i.e. primary) other QoS requirements affected by the degree of adherence of the QoS class, then another location estimation may be triggered at LMF attempting less stringent other QoS requirements. The process may be iterated until the least stringent (i.e. minimum) other QoS requirements are attempted. If the least stringent other QoS requirements cannot be fulfilled by a location estimate, then the location estimate shall be discarded, and an appropriate error cause shall be sent. NOTE 2: An AF may provide a location request with Multiple QoS Class via NEF. For an LCS client to provide a location request with Multiple QoS Class an Le interface implementation supporting Multiple QoS Class may be required. NOTE 3: Multiple QoS Class can only be applied for Deferred 5GC-MT-LR Procedure in this release of the specification. - Assured Class: This class defines the most stringent requirement on the accuracy achieved for a location request. If a location estimate obtained does not fulfil the other QoS requirements, then it shall be discarded, and an appropriate error cause shall be sent. NOTE 4: How the LMF decides the positioning method is an implementation aspect not pre-determined by QoS criteria. For LCS client, it may indicate accuracy defined in TS 29.572 [12], tables 6.1.6.3.2-1 and 6.1.6.3.5-1. For AF, it may either indicate the accuracy defined in TS 29.572 [12], table 6.1.6.3.2-1, or indicate a particular value e.g. PLMN ID defined in TS 29.122 [35], table 5.3.2.4.7-1.

Each of class has Horizontal-/Vertical-accuracy value correspondingly (one pairs for assured and Best effort, while maximum 3 pairs for multiple QoS). One common response Time is indicated too. Once LCS client sends {QoS class, H-/V-accuracy values, response time} to LMF via GMLC/AMF, then LMF is responsible to return the location estimate with the indication of QoS fulfilment. All the others like to use which positioning method and how to configure the related reference signals to be measured at the target UE etc. are up to the LMF implementation. But LMF’s returning to the LCS client is as follows:

- If ‘assured’ class is indicated, LMF should return the location estimate to the LCS client which satisfies the accompanied accuracy value. If LMF cannot get the location estimate satisfying the accompanied accuracy value, it will discard the obtained location estimate and generate the error to the LCS client.

- If ‘best effort’ class is indicated, LMF can return the any obtained location estimate to the LCS client but with the indication whether the obtained location estimate fulfils the accompanied accuracy value or not. If LMF cannot obtained any location estimate result, then LMF can generate the error to the LCS client.

- If ‘multiple QoS’ class is indicated, (in this case, there would be max 3 of accuracy values i.e., the most preferred, intermediate and the least preferred can accompanied) LMF can try multiple location estimation triggering ‘without any interaction with LCS client’. LMF might further reconfigure the related configurations to target UE and TRPs related, and modify the positioning method per each triggering. At least, LMF should try the most stringent one at first, intermediate at second and the least preferred one in final.

Signaling details between LCS client and LMF:

Following is the excerpt from TS 29.572 on the data type in Nlmf_Location specific service message transmitted from NF service consumer (i.e., LCS client) to LMF.

##### 6.1.6.2.13	Type: LocationQoS

Table 6.1.6.2.13-1: Definition of type LocationQoS

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| hAccuracy | Accuracy | O | 0..1 | Horizontal accuracy |  |
| vAccuracy | Accuracy | O | 0..1 | Vertical accuracy |  |
| vertRequested | boolean | O | 0..1 | Vertical accuracy requested (yes/no) |  |
| responseTime | ResponseTime | O | 0..1 | No delay, Low delay or Delay tolerant |  |
| minorLocQoses | array(MinorLocationQoS) | C | 1..2 | If present, this IE shall contain a list of MinorLocationQoS in priority order. When the lcsQosClass sets to "MULTIPLE_QOS", this IE shall be present, and when lcsQosClass sets to either "BEST_EFFORT" or "ASSURED" this IE shall be absent. | MUTIQOS |
| lcsQosClass | LcsQosClass | C | 0..1 | LCS QoS Class, see clause 4.1b of 3GPP TS 23.273 [19]. This IE shall be absent if neither hAccuracy nor vAccuracy is included. |  |
| NOTE: If the lcsQosClass set to "MULTIPLE_QOS", at least one of attributes hAccuracy, vAccuracy shall be present. |  |  |  |  |  |

##### 6.1.6.2.43	Type: MinorLocationQoS

Table 6.1.6.2.43-1: Definition of type MinorLocationQoS

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hAccuracy | Accuracy | O | 0..1 | Horizontal accuracy |
| vAccuracy | Accuracy | O | 0..1 | Vertical accuracy |

Observation 1. If LCS client indicates to the LMF Multiple QoS class as a QoS class, then max of two more H-/V- accuracy values can be added in priority order to the original H-/V- accuracy values which is the preferred QoS value. (Therefore total 3 of H-/V- accuracy values with preference priority)

  - Current operation between LMF and target UE regarding QoS

Once LCS client indicates QoS attribute i.e., {QoS class, accuracy value, response time} to LMF, LMF will determine the positioning method, and related configurations to be used by the target UE, then request positioning measurement/estimate to the target UE via LPP. LPP RequestLocationInformation msg has the following attributes to be delivered to the target UE (excerpt from 37.355)

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]]

}

Once above information/request is received, UE will measure/ do the location estimate. If the result of measurement/location estimate fails to fulfil this indicated accuracy level, error msg will be transmitted to LMF. If succeed, corresponding measure result / location estimate will be transmitted to LMF. When failure happened to the given accuracy value, it is up to the LMF whether it tries again with different positioning method or change the Assistance data contents or just repeat this LPP procedure without modification.

For multiple QoS class case, it is also ok for LMF to request another positioning request with less stringent QoS value without change of positioning method or configuration of AD, because already LMF is said that the return value (location estimate) to LCS client is ok with that less stringent QoS values. Using current LPP procedure, there should be always a new Request/Provide Location Information and possible new Provide Assistance Data.

Observation 2. When multiple QoS class is indicated to the LMF, there could be two ways of LMF handling in current LPP procedure:

- Opt 1. LMF starts another LPP positioning request procedure with the same QoS value but with new positioning method in the case that the first (the most preferred) QoS value was not fulfilled.

- Opt 2. LMF try the multiple LPP positioning request procedures with different QoS values in parallel proactively.

Compared with these options, it is much better to give the multiple QoS values to the UE and allow the UE to evaluate the fulfillment of the indicated QoS values at once with the measured location estimate. Since there would be no extra time taken to evaluate the fulfillment of multiple QoS accuracies with a single measurement/location estimate, this method can reduce unnecessary sequential or parallel LPP procedures with new positioning method (Opt 1) or other less stringent QoS value (Opt 2). Opt 1 takes more LPP signaling including new Provide AD, new Request Location Information and new Provide Location Information, and processing time in-between. Opt 2 also has more signaling traversed in parallel.

In the above figure, if multiple QoS class is given to the LMF from the LCS client, LMF can handle them efficiently with proposed LPP procedure where target UE can autonomously operate based on the location information result. The detail explanation is following.

  - Proposed solution: including multipleQoS information in LPP

Our proposal is to introduce the ‘multipleQoS’ class in LPP domain too. LMF can indicate to the target UE with the multiple QoS information when it received LCS service request with multipleQoS class from LCS client. This multiple QoS information would be the preferred QoS H-/V- values, intermediate QoS H-/V- values, and minimum QoS H-/V- values. These accuracy values should be the same as what LMF received from LCS client.

Proposal 1. When LMF receives the service request with multipleQoS class from LCS client, multiple set of (H-/V-) accuracy values per QoS level same as LMF received from LCS client is indicated to LPP location information request procedure.

Once the P1 is agreed, we can further discuss on how the spec would be impacted. For UE operation, UE should consider the sequential evaluation on whether the measured result/ location estimate is fulfilling each accuracy levels indicated. This should be done in the priority order (i.e., from the most preferred accuracy to the least preferred accuracy). However, in other case, UE can just evaluate all the related accuracy at once. Whether UE does sequential evaluation separately or does evaluation for all the indicated accuracy values at once is up to UE implementation. Therefore, the specification impact can be as following proposal:

Proposal 2. If UE receives LPP Request Location Information including multiple QoS, UE should evaluate whether the obtained location estimate fulfils the accompanied accuracy requirements for all the given QoS.

Once the measured result/ location estimate fulfils any accuracy requirements among indicated ones, UE should report the result/location estimate with the indication of the highest preferred accuracy values among fulfilled ones.

Proposal 3. Once the measured result/ location estimate fulfils any accuracy requirements among indicated ones, UE should report the measurement result/location estimate with the indication of the highest preferred accuracy values among fulfilled ones.

Based on the above proposals, this operation is only applicable to the UE-based positioning where the location estimate is known by the target UE, so to evaluate the QoS fulfilment.

Proposal 4. This proposed operation is only applicable to the UE-based positioning.

The details of the specification impact is described in the Annex draft CR.

- Conclusion

In this contribution, we discussed on the introducing the multiple QoS level information to LPP and provide the following conclusion:

Observation 1. If LCS client indicates to the LMF Multiple QoS class as a QoS class, then max of two more H-/V- accuracy values can be added in priority order to the original H-/V- accuracy values which is the preferred QoS value. (Therefore total 3 of H-/V- accuracy values with preference priority)

Observation 2. When multiple QoS class is indicated to the LMF, there could be two ways of LMF handling in current LPP procedure:

- Opt 1. LMF starts another LPP positioning request procedure with the same QoS value but with new positioning method in the case that the first (the most preferred) QoS value was not fulfilled.

- Opt 2. LMF try the multiple LPP positioning request procedures with different QoS values in parallel proactively.

Proposal 1. When LMF receives the service request with multipleQoS class from LCS client, multiple set of (H-/V-) accuracy values per QoS level same as LMF received from LCS client is indicated to LPP location information request procedure.

Proposal 2. If UE receives LPP Request Location Information including multiple QoS, UE should evaluate whether the obtained location estimate fulfils the accompanied accuracy requirements for all the given QoS.

Proposal 3. Once the measured result/ location estimate fulfils any accuracy requirements among indicated ones, UE should report the measurement result/location estimate with the indication of the highest preferred accuracy values among fulfilled ones.

Proposal 4. This proposed operation is only applicable to the UE-based positioning.

- Annex

Proposed TP in 37.355

First change

#### –	CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationTime-r17

ScheduledLocationTime-r17	OPTIONAL,	-- Need ON

targetIntegrityRisk-r17

TargetIntegrityRisk-r17		OPTIONAL	-- Need ON

]]

}

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...

}

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

TriggeredReportingCriteria ::=		SEQUENCE {

cellChange							BOOLEAN,

reportingDuration					ReportingDuration,

...

}

ReportingDuration ::=				INTEGER (0..255)

AdditionalInformation ::= ENUMERATED {

onlyReturnInformationRequested,

mayReturnAditionalInformation,

...

}

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]],

[[	multiQoS-r18			MultiQoS-r18			OPTIONAL,	-- Cond UEB

]]

}

HorizontalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

VerticalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

HorizontalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

VerticalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

MultiQoS-r18 ::= SEQUENCE (SIZE (1..3)) OF QoS_param

QoS_param ::= SEQUENCE {

accuracyLevel-r18		INTEGER(0..2)

horizontalAccuracyExt-r18 		HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalCoordinateRequest-r18	BOOLEAN,

verticalAccuracyExt-r18		VerticalAccuracyExt-r15			OPTIONAL,	-- Need ON

}

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-v1700 }

OPTIONAL		-- Need ON

]]

}

ResponseTimeNB-r14 ::= SEQUENCE {

timeNB-r14							INTEGER (1..512),

responseTimeEarlyFixNB-r14			INTEGER (1..512)		OPTIONAL,		-- Need ON

...,

[[	unitNB-r15				ENUMERATED { ten-seconds, ... }	OPTIONAL		-- Need ON

]]

}

Environment ::= ENUMERATED {

badArea,

notBadArea,

mixedArea,

...

}

MessageSizeLimitNB-r14 ::= SEQUENCE {

measurementLimit-r14				INTEGER (1..512)		OPTIONAL,		-- Need ON

...

}

ScheduledLocationTime-r17 ::= SEQUENCE {

utcTime-r17			UTCTime											OPTIONAL,	-- Need ON

gnssTime-r17		SEQUENCE {

gnss-TOD-msec-r17		INTEGER (0..3599999),

gnss-TimeID-r17			GNSS-ID

}												OPTIONAL,	-- Need ON

networkTime-r17		CHOICE {

e-utraTime-r17			SEQUENCE {

lte-PhysCellId-r17			INTEGER (0..503),

lte-ArfcnEUTRA-r17			ARFCN-ValueEUTRA,

lte-CellGlobalId-r17		CellGlobalIdEUTRA-AndUTRA

OPTIONAL,	-- Need ON

lte-SystemFrameNumber-r17	INTEGER (0..1023)

},

nrTime-r17				SEQUENCE {

nr-PhysCellID-r17		NR-PhysCellID-r16,

nr-ARFCN-r17			ARFCN-ValueNR-r15,

nr-CellGlobalID-r17		NCGI-r15	OPTIONAL,		-- Need ON

nr-SFN-r17				INTEGER (0..1023),

nr-Slot-r17				CHOICE {

scs15-r17				INTEGER (0..9),

scs30-r17				INTEGER (0..19),

scs60-r17				INTEGER (0..39),

scs120-r17				INTEGER (0..79)

}										OPTIONAL	-- Need ON

},

...

}												OPTIONAL,	-- Need ON

relativeTime-r17	INTEGER (1..1024)								OPTIONAL	-- Need ON

}

TargetIntegrityRisk-r17 ::=		INTEGER (10..90)

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ECID | The field is optionally present, need ON, if E-CID or NR E-CID is requested. Otherwise it is not present. |
| UEB | The field is optionally present, need ON, if the locationInformationType is set to locationEstimateRequired, locationEstimatePreferred, or locationMeasurementsPreferred; oltherwise it is not present. |

CommonIEsRequestLocationInformation field descriptions

locationInformationType This IE indicates whether the server requires a location estimate or measurements. For 'locationEstimateRequired', the target device shall return a location estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', the target device shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', the target device shall return a location estimate if possible, but may also or instead return measurements for any requested position methods for which a location estimate is not possible. For 'locationMeasurementsPreferred', the target device shall return location measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible.

triggeredReporting This IE indicates that triggered reporting is requested and comprises the following subfields: - cellChange: If this field is set to TRUE, the target device provides requested location information each time the primary cell has changed. - reportingDuration: Maximum duration of triggered reporting in seconds. A value of zero is interpreted to mean an unlimited (i.e. "infinite") duration. The target device should continue triggered reporting for the reportingDuration or until an LPP Abort or LPP Error message is received. The triggeredReporting field should not be included by the location server and shall be ignored by the target device if the periodicalReporting IE or responseTime IE or responseTimeNB IE is included in CommonIEsRequestLocationInformation.

periodicalReporting This IE indicates that periodic reporting is requested and comprises the following subfields: - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 1, 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device shou-ld continue periodic reporting until an LPP Abort message is received. The value 'ra1' shall not be used by a sender. - reportingInterval indicates the interval between location information reports and the response time requirement for the first location information report. Enumerated values ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64 correspond to reporting intervals of 1, 2, 4, 8, 10, 16, 20, 32, and 64 seconds, respectively. Measurement reports containing no measurements or no location estimate are required when a reportingInterval expires before a target device is able to obtain new measurements or obtain a new location estimate. The value 'noPeriodicalReporting' shall not be used by a sender.

additionalInformation This IE indicates whether a target device is allowed to return additional information to that requested. If this IE indicates 'onlyReturnInformationRequested' then the target device shall not return any additional information to that requested by the server. If this IE indicates 'mayReturnAdditionalInformation' then the target device may return additional information to that requested by the server. If a location estimate is returned, any additional information is restricted to that associated with a location estimate (e.g. might include velocity if velocity was not requested but cannot include measurements). If measurements are returned, any additional information is restricted to additional measurements (e.g. might include E-CID measurements if A-GNSS measurements were requested but not E-CID measurements).

qos This IE indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the server from the measurements provided by the target device assuming that the measurements are the only sources of error. Fields are as follows: - horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE) - verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded uncertainty altitude as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - responseTime - time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. If the periodicalReporting IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - responseTimeEarlyFix indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFix IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the time IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFix IE. A server should set the responseTimeEarlyFix IE to a value less than that for the time IE. A target shall ignore the responseTimeEarlyFix IE if its value is not less than that for the time IE. - unit indicates the unit of the time and responseTimeEarlyFix fields. Enumerated value 'ten-seconds' corresponds to a resolution of 10 seconds. Enumerated value 'ten-milli-seconds' corresponds to a resolution of 0.01 seconds. If this field is absent, the unit/resolution is 1 second. Enumerated value 'ten-milli-seconds' is only applicable for NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning. If the enumerated value 'ten-milli-seconds' is included for methods others than NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning the target device shall ignore the unit field. - velocityRequest indicates whether velocity (or measurements related to velocity) is requested (TRUE) or not (FALSE). - responseTimeNB If the periodicalReporting IE or responseTime IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - timeNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. - responseTimeEarlyFixNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFixNB IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the timeNB IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFixNB IE. A server should set the responseTimeEarlyFixNB IE to a value less than that for the timeNB IE. A target shall ignore the responseTimeEarlyFixNB IE if its value is not less than that for the timeNB IE. - unitNB indicates the unit of the timeNB and responseTimeEarlyFixNB fields. Enumerated value 'ten-second' corresponds to a resolution of 10 seconds. If this field is absent, the unit/resolution is 1 second. - horizontalAccuracyExt indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the horizontalAccuracy field is included in QoS. - verticalAccuracyExt indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the verticalAccuracy field is included in QoS. - multiQoS indicates set of the QoS parameters per each QoS level when the target UE is allowed to fulfil multiple QoS levels for the obtained location estimate without further LPP signaling from LMF. When this field is indicated, the target UE can report the location estimate with the accuracyLevel of the highest QoS level fulfilled among the indicated multiple QoS levels. If this field is present, there should be no horizontalAccuray, verticalAccuray nor verticalCoordinateRequest. - QoS_param indicates the parameters per each QoS level. This field may have the following sub parameters, horizontalAccuracyExt, verticalAccuracyExt, verticalCoordinateRequest, and accuracyLevel. accuracyLevel indicates the identification of each QoS level to be fulfilled, and the order of the stringecy level, i.e., ‘0’ means the most preferred or stringent QoS level, and ‘1’ means the next or intermediate preferred/stringent QoS level, and so on. All QoS requirements shall be obtained by the target device to the degree possible but it is permitted to return a response that does not fulfill all QoS requirements if some were not attainable. The single exception is time and timeNB which shall always be fulfilled – even if that means not fulfilling other QoS requirements. A target device supporting NB-IoT access shall support the responseTimeNB IE. A target device supporting HA GNSS shall support the HorizontalAccuracyExt, VerticalAccuracyEx, and unit fields with enumerated value 'ten-seconds'. A target device supporting NB-IoT access and HA GNSS shall support the unitNB field.

Second change

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[[

segmentationInfo-r14	SegmentationInfo-r14	OPTIONAL		-- Cond Segmentation

]],

[[

integrityInfo-r17		IntegrityInfo-r17		OPTIONAL

]],

[[	achievedQoSLevel-r18	AchievedQoSLevel-r18	OPTIONAL		-- Cond MultipleQoS

]]

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint								Ellipsoid-Point,

ellipsoidPointWithUncertaintyCircle			Ellipsoid-PointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse		EllipsoidPointWithUncertaintyEllipse,

polygon										Polygon,

ellipsoidPointWithAltitude					EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc								EllipsoidArc,

...,

highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680																				HA-EllipsoidPointWithScalableUncertaintyEllipse-r16,

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16

}

Velocity ::= CHOICE {

horizontalVelocity							HorizontalVelocity,

horizontalWithVerticalVelocity				HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty			HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty

HorizontalWithVerticalVelocityAndUncertainty,

...

}

LocationError ::= SEQUENCE {

locationfailurecause			LocationFailureCause,

...

}

LocationFailureCause ::= ENUMERATED {

undefined,

requestedMethodNotSupported,

positionMethodFailure,

periodicLocationMeasurementsNotAvailable,

...

}

EarlyFixReport-r12 ::= ENUMERATED {

noMoreMessages,

moreMessagesOnTheWay

}

LocationSource-r13 ::= BIT STRING {	a-gnss				(0),

wlan				(1),

bt					(2),

tbs					(3),

sensor				(4),

ha-gnss-v1510		(5),

motion-sensor-v1550 (6),

dl-tdoa-r16			(7),

dl-aod-r16			(8) } (SIZE(1..16))

IntegrityInfo-r17 ::= SEQUENCE {

horizontalProtectionLevel-r17		INTEGER (0..50000),

verticalProtectionLevel-r17			INTEGER (0..50000)				OPTIONAL,

achievableTargetIntegrityRisk-r17	INTEGER (10..90)				OPTIONAL,

...

}

AchievedQoSLevel-r18 ::= INTEGER(0..2)

-- ASN1STOP