3GPP TSG-RAN WG2 Meeting #122	R2-2306256

Incheon, Korea, May 22nd – 26th, 2023

Source: 	vivo

Title:	Discussion on Multi-Consecutive Slots transmission

Agenda Item: 	7.15.3

Document for: Discussion and Decision

# Introduction

During RAN1#112bis-e meeting, an LS was discussed and sent to RAN2 [1]:

1. Overall Description: RAN1 has discussed the following approaches to implement/achieve MCSt for SL-U communication. RAN1 would like to seek RAN2’s opinion on the following questions. Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA). Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs) Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

In this contribution, we will analyze the 3 approaches and provide our opinion on the questions.

# Discussion

## Issues for Question 1

To conclude whether it is feasible for question 1, the following questions needed to be discussed and addressed first:

- Whether Approach 1/2 require the UE to be able to triggering the resource selection procedures for multiple SL processes at the same time?

Although Question 2 mentions that it is for approach 3, we observe that whether the resource selection procedures for multiple SL processes are triggered at the same time may also affect the detail design of Approach 1/2. For example:

- Alt. 1: If the UE repeatedly trigger resource selection for different processes, then for each process, the UE independently decide whether to select resources concatenating to other selected resources for other process with ‘best effort’;

- Alt. 2: If the UE can trigger resource selection independently for different processes at the same time, the UE can as a whole try to select consecutive resources for multiple processes at the same time with ‘best effort’ in L2. The UE can still perform Alt.1 when a later trigger happens;

The reason why RAN1 sends the LS to RAN2 with Question 2 only for Approach 3 may be that the feasibility of Q2 is mandatory for Approach 3, while it is only an optional choice for Approach 1/2. From our perspective, if we go with Approach 1/2, it is not necessary to support simultaneous resource selection trigger (i.e. Alt 2). Otherwise, it is more reasonable to pursue Approach 3 by L1 guaranteeing candidate MCSt resources for transmission of multiple TBs.

Therefore, we suggest RAN2 to discuss and confirm that approach 1/2 is not required to support simultaneous resource selection trigger for multiple SL processes at the same time. If RAN2 cannot conclude, we can ask RAN1 in the LS reply.

Proposal 1: RAN2 to confirm that RAN2 understanding is that approach 1/2 does not require support of simultaneous resource selection trigger for multiple SL processes at the same time. If RAN2 cannot conclude, ask RAN1 in the LS reply.

- How to design the consecutive-slots criterion to achieve MCSt for the MAC layer for approach 1/2?

To answer about the feasibility of selecting ‘concatenating’ resources for different process, one needs to address some complicated issues about the consecutive-slots criterion design from L2’s perspective, for example:

- How does L2 know the ‘concatenating’ resource forming MCSt resource is a valid resource for L1 to perform transmission?

L1 only provide SA for different independent processes without knowing how L2 decides to use SA for MCSt resource selection for different processes. If L2 performs ‘best effort’ MCSt resource selection and LCP procedure, the L1 may find it as result that the L2-selected MCSt resource is not valid (e.g. additional LBT for TB2 is actually needed while there is no time as the resources are concatenating)

- How to select consecutive resources for new transmission and retransmission?

In legacy resource selection, UE selects sets of resources for new transmission and retransmission. If the UE needs to consider whether or not to select concatenating resources for new transmission across different processes and concatenating resources for retransmission across different processes as a whole, which would be very complicated.

For instance, RAN1 has agreed to support multiple PSFCH occasions. When the gNB configures to support multiple PSFCH occasions, with the legacy DCI for new transmission only supports the UE to indicate reserved resource at most spanning 32 slots, this would lead to very finite candidate usable resources for one process, let alone to select concatenating resources for different processes.

There are still some other issues, yet we do not intend to list them all here to complicate the discussion, as any of example issue listed above will involve intensive RAN2 discussion.

Proposal 2: RAN2 to discuss the following issues related to feasiblity for Question 1 before answering it:

- How does L2 know the ‘concatenating’ resource forming MCSt resource is a valid resource for L1 to perform transmission?

- How to select consecutive resources for new transmission and retransmission?

Since RAN1 only sends LS to RAN2 to ask about the feasibility of the question, not RAN2’s preference of which approach to pursue, we suggest RAN2 not to step into any detailed L2 consecutive-slots criterion design for approach 1/2 until RAN1 makes some progress on which approach to support. All efforts will be usesless if RAN1 decides to pursue approach 3.

Proposal 3: RAN2 postpone discussion about detail design of L2 consecutive-slots criterion for approach 1/2 until RAN1’s further progress on which approach to support.

## Issues for Question 2

- How to understand “at the same time” for Question 2?

According to legacy resource selection procedure, it is possible that UE may trigger resource selection procedures for different triggers at the same time or within a small duration with UE implementation. From this perspective, it is feasible to trigger the procedures at the same time. However, we understand the intention of Question 2 is to confirm whether it is feasible to perform joint resource selection for different SL processes taking the answer for Question 3 into account. Therefore, we propose two alternative ways to conclude Question 2 as below:

Proposal 4: RAN2 to discuss which alternative to conclude the answer for Question 2:

- Alt 1: RAN2 confirms that it is feasible to trigger the resource selection procedures for multiple SL processes at the same time.

- Alt 2: RAN2 confirms that it is feasible to trigger the resource selection procedures for multiple SL processes at the same time, by providing L1 one set of parameters (prioTX, remaining PDB, LsubCH  and Prsvp_TX ) + “number of slots for MCSt” for joint resource selection procedures for multiple SL processes.

## Issues for Question 3

- How does L2 determine the “number of slots for MCSt”?

Both of Approach 2/3 need L2 to determine the “number of slots for MCSt”. However, it should be separately discussed for approach 2 and approach 3 as they serve for different cases, i.e. L1 report MCSt resources for a single TB or multiple TBs.

On determining the “number of slots for MCSt” for approach 2

From the LS description, we cannot tell why L2 needs to inform the “number of slots for MCSt” to L1 for single TB. There are two possible use cases from our perspective:

- L2 decides to transmit a single TB spanning multiple-slot resources;

If a UE intends to occupy the channel and share it to a peer UE, while there’s only buffer for a single TB, it may choose to transmit a single TB spanning multiple-slot resources if necessary.

- L2 decides to perform new transmission and blind retransmission(s) for a single TB;

L2 determines the “number of slots for MCSt” based on the number of blind retransmission(s).

For both cases, we assume it is feasible to determine a “number of slots for MCSt” for transmission of a single TB.

On determining the “number of slots for MCSt” for approach 3

From a RAN2 perspective, we think the following L2 information can be considered to decide the number of slots upon transmission for multiple TBs:

- Buffer size level:

One of the intentions of triggering MCSt is the need of transmission of a large size file. It is straightforward that the more data is buffered, the more slots are needed for transmission. Specifically, a UE may decide to enable MCSt, only when the buffer size level of the available data is relatively high. With the information of the buffer size level, the UE can well utilize it to determine the number of slots for MCSt.

- CAPC of the SL LCH(s) that has data in the buffer:

In legacy resource selection procedure, L2 can determine a priority related to its logical channel that has data in the buffer and indicate it to L1. Similarly, UE can determine a CAPC related to its logical channel. Since each CAPC is corresponding to a maximum COT duration, this information can also be used to determine the number of slots which span in time tomain no longer than the maximum COT duration.

In sum, for both Approach 2/3, we think it is feasible for L2 to provide a new parameter “number of slots for MCSt” to L1.

Proposal 5: RAN2 confirms it is feasible to provide a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt for both approach 2 and approach 3.

# Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: RAN2 to discuss and confirm that approach 1/2 is not required to support simultaneous resource selection trigger. If RAN2 cannot conclude, ask RAN1 in the LS reply.

Proposal 2: RAN2 to discuss the following issues related to feasiblity for Question 1 before answering it:

- How does L2 know the ‘concatenating’ resource forming MCSt resource is a valid resource for L1 to perform transmission?

- How to select consecutive resources for new transmission and retransmission?

Proposal 3: RAN2 postpone discussion about detail design of L2 consecutive-slots criterion for approach 1/2 until RAN1’s further progress on which approach to support.

Proposal 4: RAN2 to discuss which alternative to conclude the answer for Question 2:

- Alt 1: RAN2 confirms that it is feasible to trigger the resource selection procedures for multiple SL processes at the same time.

- Alt 2: RAN2 confirms that it is feasible to trigger the resource selection procedures for multiple SL processes at the same time, by providing L1 one set of parameters (prioTX, remaining PDB, LsubCH  and Prsvp_TX ) + “number of slots for MCSt” for joint resource selection procedures for multiple SL processes.

Proposal 5: RAN2 confirms it is feasible to provide a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt for both approach 2 and approach 3.

# References

- R1-2304257 LS on MCSt resource (re-)selection