3GPP TSG-RAN WG2 Meeting #122	R2-2306268

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.16.2.1

Source:	AT&T

Title:	Architecture and LCM aspects of AI/ML for NR air interface

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. Some of the aspects of the study item include RAN2-led objectives:

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

RAN2 should study how the life cycle management (LCM) for a given AI/ML model can be supported by existing Uu signaling and procedures and if any enhancements are needed, specifically in the areas of model delivery/updates, data collection, and model monitoring. This contribution discusses the protocol aspects of an AI/ML framework applied to the NR air interface, specifically the details and requirements for model and functionality identification.

# Model/functionality identification and LCM

The 5G industry trends which enable network virtualization and deployment of low-latency/high bandwidth services are also making application of power Artificial Intelligence (AI) tools such as machine learning (ML) algorithms to 5G networks feasible and scalable.  These algorithms rely on historical data for deriving system models and training as well as real-time or near-real-time data collection to adapt to different network conditions. Furthermore, a variety of use cases can be supported by AI/ML techniques as noted in the SID including CSI feedback optimization, beam management, and positioning. Different use cases can have vastly different requirements in terms of the impact on network nodes or functionalities. This implies that the appropriate implementation of different AI/ML techniques may involve multiple interfaces, signalling procedures, and processing requirements (including requirements on data aggregation or co-location with different nodes/functions).

For one-sided AI/ML models that reside at the UE/network, one or multiple AI/ML models can be possibly deployed and used at the UE/network. Similarly, for two-sided AI/ML models, there may be multiple AI/ML models that can be possibly deployed. It is key then to monitor the performance of these models for various reasons: (1) Depending on the type of model used, these models may not have performance guarantees, (2) deployment options and environment characteristics may change after deployment, resulting in a sub-optional performance (3) when multiple models are available at the UE/network, model selection decision needs to be made, and this decision cannot be left to implementation (4) for one-sided models at UE the model or input/output parameters of models may be modified over time, from a third party cloud, without notifying the network, resulting in a performance degradation.

For the UE/network to be able to make certain decisions, based on the model performance monitoring or otherwise, a model identification and LCM procedure needs to be agreed on. This identification procedure needs to be flexible so it can cover all different types of models, where the model inference resides, and support all collaboration levels between the network and the UE.  The model and/or functionality identification procedure can also be used to indicate the support of functionalities applicable to the different AI/ML use cases in various deployment scenarios, network conditions, and use cases.

During RAN2#121bis the following was agreed in RAN2:

R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

Model ID can be used to identify model or models for the following LCM purposes:

- model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

- (e.g. for so called “model ID based LCM”)

If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

How to achieve globality of the Model ID is FFS.

- Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

- Direction1: Pre-defined/hard-coded global unique model ID

- Direction3: Assigned global unique model ID via specific ID management node.

- Note: Other global unique model ID definition is not precluded.

- Model ID structure, if any, is FFS

During RAN1#112bis the following was agreed for AI/ML functionality identification and functionality-based LCM and AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

- Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

- Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

  - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

  - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

  - FFS: Other aspects that may constitute Functionality

FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

    - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

    - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

    - FFS: relationship between functionality-based LCM and model-ID-based LCM

Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

As discussed in RAN1, the level of details regarding a given model can be different based on the use case and application. For instance, for a one-sided model which does not require any network collaboration, a functionality-based LCM could be applied where the necessary model functionalities are reported via control plane signalling as part of regular UE capability reporting to correctly identify and differentiate models which have different use cases, model structure, model parameters and/or training datasets. However, to support other parts of LCM such as model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, and testing/calibration performed by a network operator, in addition to any functionality-based LCM meta info, a model ID could be configured/exchanged since for these purposes, the granularity of the model ID needs to support device or vendor identification, model version tracking.

Proposal 1: Functionality-based LCM meta info, including AI/ML-enabled features/FGs indicated by capability signaling, their applicable conditions, and model structure (if provided) can be optionally associated with one or multiple model IDs in order to support both functionality/model identification and LCM functions including model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, model-specific performance monitoring, and testing and calibration performed by a network operator.

During RAN2#121bis the following was agreed in RAN2:

The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage.

Chair: the following was almost agreed (leave it FFS for now): AI/ML functional architecture in Figure 1 in R2-2303674 is the baseline with the modification that Performance Monitoring is changed to Model Mgmt / Performance Monitoring. It is noted that the exact interactions may need some modification depending on how each piece of functionality is specified.

Additionally, during RAN2#121bis the following figure was discussed to capture various functional architecture and LCM aspects:

Figure 1. AI/ML Functional Architecture

The figure captures the functional architecture of AI/ML models that is common for all use-cases. However, the model delivery/transfer is not applicable for all cases. For instance, if the model training is performed on the same device which is performing the model inference (e.g. model trained at gNB and inference at gNB), there is no model delivery/transfer from model storage (as it is also located at the same device). Therefore, in order to include this case in the current framework we need to change “Model delivery/transfer” to “Model deployment/delivery/transfer”.

Proposal 2: Take as baseline the following figure as an overview of the AI/ML functional architecture which includes co-located model deployment and non-collocated model delivery/transfer scenarios.

Model deployment/delivery/transfer

# Conclusion

In this contribution, we discussed the AI/ML model LCM over the NR air interface. The following proposal was made:

Proposal 1: Functionality-based LCM meta info, including AI/ML-enabled features/FGs indicated by capability signaling, their applicable conditions, and model structure (if provided) can be optionally associated with one or multiple model IDs in order to support both functionality/model identification and LCM functions including model transfer/delivery, activation/deactivation, multi-vendor two-sided model pairing, model-specific performance monitoring, and testing and calibration performed by a network operator.

Proposal 2: Take as baseline the following figure as an overview of the AI/ML functional architecture which includes co-located model deployment and non-collocated model delivery/transfer scenarios.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface.