3GPP TSG-RAN WG2 Meeting #122	R2-2306324

Incheon, Korea, May 22-26, 2023

Agenda item:	7.7.4.1.1

Source:	Samsung

Title:	Remaining issues on NTN-TN Cell Reselection Enhancements

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# Introduction

NTN-TN cell reselection enhancements were discussed with the focus on power saving for TN cell measurement. The following agreements were made in the last two meetings.

Agreements 121: - TN coverage area information will be associated to the frequency information. - RAN2 adopts explicit description of geographical TN area, and focuses on the following options for further discussion, taking the signalling overhead into account (FFS on the accuracy of the information): Option 1: The corresponding geographical area information is provided by network with location coordinates of area center and radius. Option 2: a boundary line is provided by network in the format of a list of location coordinates, additionally an indication can be used to indicate which side is the TN side Option 6: for each TN area, a list of locations is provided by network, and the corresponding close shape could be illustrated by a polygon connecting these points within the list. - As a baseline, broadcast signalling is used to provide the information on the TN coverage area for UEs supporting NTN. - Also based on the signalling overhead of the broadcast solution, RAN2 will further consider the option that UE-specific update can be optionally be provided via dedicated signalling, overriding the broadcast configuration (FFS if via RRC or higher layers. FFS on the validity time, if provided by RRC) - We don’t introduce additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17. Agreements 121bis-e: - For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful - Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. FFS if Rel-17 referenceLocation and distanceThresh are directly reused - Decision on the size of TN coverage area list is postponed until more is known on the format of this information and how is it sent. - The discussion on how to indicate the frequency information for each TN coverage area should be combined with the discussion on which SIB will be used to indicate the TN coverage area, possibly based on evaluation of the signalling overhead - The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info Working assumption 121bis-e: 1. We do not introduce new triggers making the UE reacquire the TN coverage information from SI

In this document, we provide our views on the remaining open issues.

# Discussion

In RAN2#121bis-2, regarding how to signal the TN coverage area, it has been agreed that the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). Regarding how to use the TN coverage information, some relevant agreements have been made in previous meetings.

Agreements 119: - RAN2 to work on a solution so that measurements for TN’s coverage are performed only when relevant (FFS what relevant means). Agreements 119bis: - UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. Agreements 121: - We don’t introduce additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17.

Given the TN coverage information, UE is not required to perform TN neighbour cell measurement if there is no TN coverage around. When TN coverage is around, UE can measure TN neighbour cells considering the serving cell quality. To determine if a TN coverage area is around, UE needs to compare its distance to a TN reference location with a certain distance threshold. As the TN coverage area is signalled by the area center location coordinates and radius, these two parameters can be used as the reference location and the distance threshold to determine UE distance to TN coverage area.

In Rel-17, for quasi earth-fixed cell, parameters referenceLocation and distanceThresh are signaled in SIB19 for location-based measurement initiation for cell reselection. Similarly, to signal one TN coverage area, the same parameters can be used in the way that referenceLocation indicates the TN area center location and distanceThresh indicates the TN area radius.

Proposal 1: For a TN coverage area, use referenceLocation and distanceThresh to indicate the center location coordinates and the radius, respectively.

Proposal 2: When TN coverage information is broadcasted, UE is not required to measure TN neighbor cells in a TN area if the distance from UE to the center location is larger than the radius.

In RAN2#121, it has been agreed that TN coverage area information is associated to the frequency information. How to associate the TN coverage area information and the frequency information was discussed in RAN2#121bis-2 [1]. There are basically two options.

Option 1: a list of TN coverage areas under each frequency entry.

Option 2: a list of frequencies under each TN coverage area.

Option 1 is more aligned with the existing signalling structure in SIB4, where a frequency list is included and the TN coverage area list can be added as a new field included in each well-defined InterFreqCarrierFreqInfo, shown as below. To minimize overhead, the TN coverage area can be defined in a separate IE and the TN coverage list under each nterFreqCarrierFreqInfo can include a list of TN coverage area IDs.

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1700      InterFreqCarrierFreqList-v1700              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1720      InterFreqCarrierFreqList-v1720              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1730      InterFreqCarrierFreqList-v1730              OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1610 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqList-v1700 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700

InterFreqCarrierFreqList-v1720 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1720

InterFreqCarrierFreqList-v1730 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1730

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                         OPTIONAL,   -- Need R

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqExcludedCellList           InterFreqExcludedCellList                                   OPTIONAL,   -- Need R

TNcoverageAreaList

...

}

For option 2, the signalling structure would be within a new IE for a list of TN coverage areas, each entry in the list defines a TN area and include a list of frequencies. This option is more suitable if SIB19 or a new SIB includes the TN coverage area information. Along with this option, an alternative mentioned in the last meeting is that the list of frequencies can be indicated by a bitmap for frequencies, where each bit can refer to the frequency entries included in SIB3/4 in order.

Proposal 3: To signal TN coverage area information associated with frequencies, discuss the following options:

Option 1: a list of TN coverage areas under each frequency entry in SIB4.

Option 2: a list of frequencies under each TN coverage area in SIB19 or in a new SIB.

Regarding how UE (re)-acquires TN coverage information, RAN2 has made a working assumption that do not introduce new trigger making the UE reacquire the TN coverage information from SI. We think it is sufficient that UE follows the existing SI acquisition procedure to obtain TN coverage information, as specified in TS 38.331:

“The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB.”.

An NTN quasi-fixed or moving cell can provide sufficient TN area information relevant to its serving area in system information. When UE moves and camps to a new cell, UE checks the validity of the SIB containing TN coverage information and reacquires the SIB if the stored SIB is not valid. When camping to a cell and receiving SI change indication, UE applies the existing SI acquisition procedure to update the TN coverage information. Thus, we think the working assumption can be confirmed.

Proposal 4: Confirm the working assumption that do not introduce new trigger making the UE reacquire the TN coverage information from SI.

Another open issue is that whether additional information for TN coverage area (e.g., more accurate information) in dedicated signaling is needed/useful. The motivation of providing TN coverage information is to enable UE power saving in RRC_IDLE/INACTIVE cell reselection, while the dedicated signaling is only available in RRC_CONNECTED, which may not provide information that is still valid when UE enters RRC_IDLE/INACTIVE. For example, even if UE receives more accurate TN information in connected mode and enters idle mode to use it, UE can move to a new area or camp to a new cell in idle mode and the stored information may become invalid. Thus, we don’t see much benefit to provide additional TN coverage information by dedicated signaling, considering the efforts to specify relevant procedures (e.g., TN coverage information update, how to determine which version to be used at a certain location). On the other aspect, we think for cell reselection in RRC_IDLE/INACTIVE, TN coverage information signaled by area center location + area radius should be sufficient for UE power saving.

Proposal 5: Additional information for TN coverage area in dedicated signaling is not needed.

# Conclusion

Based on the discussion, we have the following observations and proposals.

Proposal 1: For a TN coverage area, use referenceLocation and distanceThresh to indicate the center location coordinates and the radius, respectively.

Proposal 2: When TN coverage information is broadcasted, UE is not required to measure TN neighbor cells in a TN area if the distance from UE to the center location is larger than the radius.

Proposal 3: To signal TN coverage area information associated with frequencies, discuss the following options:

Option 1: a list of TN coverage areas under each frequency entry in SIB4.

Option 2: a list of frequencies under each TN coverage area in SIB19 or in a new SIB.

Proposal 4: Confirm the working assumption that do not introduce new trigger making the UE reacquire the TN coverage information from SI.

Proposal 5: Additional information for TN coverage area in dedicated signaling is not needed.

# Reference

[1] R2-2304246 Report from [AT121bis-e][106][NR NTN Enh] Signalling of TN coverage (Nokia)