3GPP TSG-RAN WG2 Meeting #122	 R2-2306386

Incheon, South Korea, 22th – 26th May, 2023

Agenda Item:	7.15.2

Source: 	ITL

Title: 	Discussion on SL Consistent LBT failure

Document for:	Discussion and decision

Introduction

In RAN2#121 and RAN2#121bis [1-2], the agreements related to this contribution are as follows:

RAN2#121

Agreements on SL consistent LBT failure

1: 	Consistent LBT failure does not trigger the UE in RRC idle/inactive to enter RRC connected.

2:	Working assumption:

- If SL LBT failure granularity is resource pool/RB set, UE uses the MAC CE to report consistent LBT failure to the gNB.

- If SL LBT failure granularity is resource pool/RB set, the MAC CE indicates SL pool/RB set where SL consistent LBT failure was declared.

- If SL LBT failure granularity is SL BWP (and the UE declares SL consistent LBT failure, the UE declares SL RLF and the existing RRC message is used for SL RLF indication for all UC connections. FFS on the need of new cause value.

- If SL LBT failure granularity is resource pool/RB set, UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all resource pools/RB sets.

3:  Working assumption: If SL LBT failure granularity is resource pool/RB set, support the change of resource pool/RB set of which consistent SL LBT failure has not been triggered from SL consistent LBT failure by TX UE upon consistent LBT failure detection. FFS whether/how the triggered consistent SL LBT failure is cancelled.

Agreements on SL LBT failure indication granularity

1: 	SL LBT failure indication granularity is per SL RB set.

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

RAN2#121bis

Agreement

SL C-LBT failure is declared per RB-set

In this contribution, we discuss sidelink un-licensed operation (SL-U) on RAN2 scope.

Discussion

SL consistent LBT failure recovery

Figure 1. Example of resource pool configuration in SL-U

In this section, we show our opinion on Mode 2 UE behavior when the SL consistent LBT failure is triggered. In RAN2#121, RAN2 has agreed that SL LBT failure is detected per SL RB set and following WA.

Working assumption: If SL LBT failure granularity is resource pool/RB set, support the change of resource pool/RB set of which consistent SL LBT failure has not been triggered from SL consistent LBT failure by TX UE upon consistent LBT failure detection. FFS whether/how the triggered consistent SL LBT failure is cancelled.

In RAN2#121, the above WA was discussed before the granularity of SL consistent LBT failure detection was decided. In RAN2#121bis, RAN2 has agreed that SL consistent LBT failure is declared per RB set. In our understanding, since a resource pool can be configured with one or more RB set(s), both resource pool and RB set change can be considered. We think that choice of changing the Resource pool or RB set can be based on the number of RB sets where SL consistent LBT failure has not been triggered within the selected resource pool.

In Figure 1, if the selected resource pool contains only one RB set (i.e., resource pool 2, 3), and there has been a SL consistent LBT failure on that RB set, MAC reselect a different resource pool that includes at least one RB set where SL consistent LBT failure has not been triggered. In this case, it is straightforward since there is no chance to choose the other RB set in the selected resource pool, it is straightforward.

However, If the selected resource pool contains more than two RB sets (i.e., resource pool 0, 1, 4) and an SL consistent LBT failure has not been triggered in any of the RB sets, MAC can reselect resources within RB set(s) where an SL consistent LBT failure has not been triggered, from the selected resource pool. In this case, MAC does not need to reselect a resource pool.

- If SL consistent LBT failure has been triggered in all the RB sets within the selected resource pool, reselect another resource pool where SL consistent LBT failure has not been triggered in at least one RB set

- If SL consistent LBT failure has not been triggered in at least one RB set within the selected resource pool, reselect resource(s) within the RB set(s) where SL consistent LBT failure has not been triggered

Proposal 1: RAN2 to confirm the working assumption “Support the change of resource pool/RB set of which SL consistent LBT failure has not been triggered from SL consistent LBT failure by TX UE upon consistent LBT failure detection.”

Proposal 2: If SL consistent LBT failure has been triggered in all the RB sets within the selected resource pool, reselect another resource pool where SL consistent LBT failure has not been triggered in at least one RB set

Proposal 3: If SL consistent LBT failure has not been triggered in at least one RB set within the selected resource pool, reselect resource(s) within the RB set(s) where SL consistent LBT failure has not been triggered

Conclusion

In this contribution, we discussed sidelink operation in un-licensed. According to discussion in section 2, we have the following proposals:

Proposal 1: RAN2 to confirm the working assumption “Support the change of resource pool/RB set of which SL consistent LBT failure has not been triggered from SL consistent LBT failure by TX UE upon consistent LBT failure detection.”

Proposal 2: If SL consistent LBT failure has been triggered in all the RB sets within the selected resource pool, reselect another resource pool where SL consistent LBT failure has not been triggered in at least one RB set

Proposal 3: If SL consistent LBT failure has not been triggered in at least one RB set within the selected resource pool, reselect resource(s) within the RB set(s) where SL consistent LBT failure has not been triggered

Reference

RAN2#121 Meeting report

RAN2#121bis Meeting report