3GPP TSG-RAN WG2 Meeting #122	 R2-2306389

Incheon, South Korea, 22th – 26th May, 2023

Agenda Item:	7.7.4.1.1

Source: 	ITL

Title: 	Discussion on NTN-TN Cell re-selection

Document for:	Discussion and decision

Introduction

In RAN2#121bis meeting [1], the agreements related to this contribution are as follows:

RAN2#121bis Agreements: - For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful Agreements: - Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. FFS if Rel-17 referenceLocation and distanceThresh are directly reused - Decision on the size of TN coverage area list is postponed until more is known on the format of this information and how is it sent. Agreements: - The discussion on how to indicate the frequency information for each TN coverage area should be combined with the discussion on which SIB will be used to indicate the TN coverage area, possibly based on evaluation of the signalling overhead - The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info Working assumption: - We do not introduce new triggers making the UE reacquire the TN coverage information from SI Agreements: - On a frequency band number shared by TN and NTN (e.g., n1), if NTN-specific assistance information is NOT provided for a neighbour cell configured in SIB3/SIB4, UE assumes this is a TN neighbour cell. This understanding is also applicable for Rel-17 and it does not need any spec update

In this contribution, we focus on cell reselection enhancements to reduce UE power consumption.

Discussion

Geographical TN Area

Figure 1. Candidate options to describe geographical TN Area

In RAN2#121, RAN2 has agreed to discuss explicit way (Option 1, 2 and 6) taking the signalling overhead into account to describe geographical TN Area as in Figure 1. In RAN2#121bis, RAN2 has agreed that network broadcasts a list of TN coverage information, which are defined using a central location and radius to describe geographical information (i.e., Option 1). Also, the area center and radius for TN coverage information is signalled using Ellipsoid-Point and radius separately.

The FFS points related to TN coverage information as follows:

- Which SIB should contain a list of TN coverage information?

  - SIB19

  - Other SIB (which include new SIB)

- Rel-17 referenceLocation and distanceThres can be reused?

Firstly, we show our view on which SIB should be used to broadcast TN coverage information. We think that there is difference in characteristics between SIB19 and TN coverage information. For example, SIB19 is updated periodically with a maximum duration of 900 seconds to update the NTN specific parameters, as determined by T430 (ntn-UlSyncValidityDuration). If SIB19 contains TN coverage information, it could lead to unnecessary updates of that information, since we think that TN coverage data does not need to be updated frequently, even in situations involving earth-moving cells. Additionally, the current SIB19 does not have sufficient space to accommodate TN coverage information. Thus, we prefer to introduce new SIB for TN coverage information.

Secondly, we think that the existing referenceLocation-r17 and distanceThresh-r17 can be reused to describe TN coverage information. We prefer to reuse the Rel-17 design as much as possible to minimize the impact on the specifications.

Proposal 1: Introduce a new SIB to broadcast a list of TN coverage information

Proposal 2: Reuse the referenceLocation-r17 and distanceThresh-r17 to describe TN coverage information

TN coverage information association with frequency information

Figure 2. Example of TN coverage information association with frequency information

In RAN2#121, RAN2 has agreed to associate the TN coverage information with frequency information. In our understanding, this means that if the UE meets the distance condition of TN Area X, it will perform measurements based on the frequency information associated with TN Area X.

RAN2#121 Agreements TN coverage area information will be associated to the frequency information.

In this section, we want to discuss how to associate the TN coverage information with frequency information. We think that there are two ways for associating TN coverage information with frequency information as in Figure 2.

- Option 1: TN coverage information in frequency information (SIB 4, 5)

- Option 2: Frequency information in TN coverage information

In Option 1, in our understanding, if there is a need to change the TN coverage information due to satellite movement or network configuration, it would also require corresponding changes in the frequency information (SIB 4, 5). Additionally, since we think that TN coverage information list is less than Frequency information, Option 1 would lead to increased signaling overhead in the SIB compared to Option 2. Thus, we prefer to introduce Option 2 taking into account the signaling overhead and SI modification.

Proposal 3: RAN2 to discuss how to associate TN coverage information with frequency information

Proposal 4: Introduce frequency information in each TN area information

Conclusion

In this contribution, we discussed NTN-TN cell re-selection enhancement. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce a new SIB to broadcast a list of TN coverage information

Proposal 2: Reuse the referenceLocation-r17 and distanceThresh-r17 to describe TN coverage information

Proposal 3: RAN2 to discuss how to associate TN coverage information with frequency information

Proposal 4: Introduce frequency information in each TN area information

Reference

RAN2#121bis Meeting report