3GPP TSG-RAN WG2 Meeting #122 	R2-2306400

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.18.3

Source:	CATT

Title:	Consideration on UP aspects for MT-SDT

Document for:	Discussion and Decision

1	Introduction

In previous RAN2#121 meeting [1], we have discussed the issues on MT-SDT and have made some baseline principle and some agreements as below:

1. Include a one-bit indication in paging to trigger MT-SDT. We will ensure that the CCCH message can be transmitted over CG. 2. Indication is per UE. FFS on signalling. 3. In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS 4. Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17) - FFS 3a: Check for DVT (if UL data becomes available in UL) - 3b: Check for SDT RSRP threshold - 3c: Check for TA validation before selecting CG (if applicable) - 3d: Check for SSB level RSRP threshold for CG resource (if applicable) 5. When UE resumes for MT-SDT, UE resumes all RBs configured for SDT 6. RBs configured for SDT are common for MO-SDT and MT-SDT 7. If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long 8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement. 1. Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT) 2. UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures 3. If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In this contribution, we will further analyse the remaining issues on UP aspects and provide our views.

2	Discussion

2.1 Background

The following is the possible brief procedure for MT-SDT. When UE in RRC_INACTIVE state receives the Paging message with MT-SDT indication, UE will check the condition for initiating the MT-SDT procedure. And if the conditions of initiating the MT-SDT procedure are fulfilled, UE will select the suitable UL resource to send the response and then receives the DL SDT data or signalling and possibly sends the UL SDT data or signalling until the MT-SDT procedure is terminated.

In the following, we will focus on analysing the remaining issues related to step 2.

Figure 1: The brief procedure for MT-SDT

2.2 Check for DVT

According to the previous agreement, after UE receiving the paging message with MT-SDT indication, UE will check whether the condition, such as SDT RSRP threshold, TA validation for CG and SSB level RSRP threshold for CG resource, are satisfied or not firstly. But it is still FFS whether the data volume threshold need to be checked.

4. Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17) - FFS 3a: Check for DVT (if UL data becomes available in UL)

Although it has been agreed that specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first, the resume cause will be set to MT-SDT), we think this just means that the resume cause should be set to MT-SDT regardless of whether there is UL data available or not. But when UE choose the UL resource to send RRCResumeRequest, UE should consider whether there is pending UL SDT data available at the same time to decide whether to use the RA-SDT resource.

Because in MO-SDT procedure, if there is pending UL SDT data to transmit and no CG-SDT resource is available, UE will send the RA-SDT specific preamble to network and then network will assign a larger UL resource to transmit UL SDT data and RRCResumeRequest to network. If we allow UE to send RA-SDT specific preamble to network after receiving MT-SDT indication even there is no UL SDT data available, the network will still assign the larger UL resource to transmit the response. This will cause the UL resource waste.

Observation 1: Whether there is pending UL SDT data available will have impact on the UL resource selection.

Observation 2: If there is no pending UL SDT data available and UE selects the RA-SDT resource to transmit the response, this will cause the UL resource waste.

So we think after UE receiving the paging message with MT-SDT indication, before UE selects the UL resource to send the response, UE should check whether there is pending UL data available and whether the volume is less or equal to the data volume threshold.

Proposal 1: Before UE selecting the UL resource to transmit the response, UE should check DVT as in R17.

2.3 UL resource selection

As for the UL resource selection, it has been agreed that UE can use non-SDT RA resources/CG resources/MO-RA resources for accessing the network for an MT-SDT transfer. And if there is valid CG-SDT resources, UE should use CG-SDT to transmit the response. But it is still not clear the order and how to select the UL resource.

In our understanding, the CG resource can be used to transmit both UL and DL data and signalling and has less transmission latency, so we should consider the CG resource firstly for MT-SDT as in R17. If there is valid CG-SDT resource, UE should use the CG-SDT resource to transmit the response for MT-SDT.

In last meeting, when UE use the CG-SDT to transmit the response, it is FFS whether we need to optimize for case when CG periodicity is too long.

7. If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long

In our understanding, this issues need to be discussed. As in current spec, the maximum periodicity of CG-SDT is 5120ms. And the maximum value of default paging cycle is 256 radio frames (2.56s). So if the periodicity of CG-SDT is too long, it is possible that UE need to wait for multiple paging cycles to transmit the response after UE receiving the paging message with MT-SDT indication. If the network expects to page UE to access the network immediately due to the DL SDT data or signalling coming, in this case, UE cannot response the network as soon as possible. This may cause some delay. So we think it is beneficial to optimize the case that the CG periodicity is too long.

Proposal 2: It is beneficial to optimize the case when CG periodicity is much longer than paging cycle to reduce the possible delay.

As for RA-SDT resource, if the CG-SDT resource is not valid, UE can consider the RA-SDT resource to transmit the response. In MO-SDT procedure, if there is pending UL SDT data to transmit and no CG-SDT resource is available, UE will send the RA-SDT specific preamble to network and then network will assign a larger UL resource to transmit UL SDT data and RRCResumeRequest to network. If we allow UE to send RA-SDT specific preamble to network after receiving MT-SDT indication even there is no UL SDT data available, the network will still assign the larger UL resource to transmit the response. This will cause the UL resource waste.

So we think the RA-SDT resource can be selected only if there is pending UL SDT data available to avoid the UL resource waste. As in most cases, when UE receives the paging message with MT-SDT indication, there may be no pending UL data to transmit. There are two options to decide whether to use RA-SDT resource to send the response.

- Option 1: UE use the non-SDT RA resources to transmit the response.

- Option 2: Define MT-SDT specific RA resource.

For option1, it seems simple to not consider the RA-SDT resource to send the response. But this will cause UE cannot use RA-SDT resource to access the network during MT-SDT procedure. For option 2, after UE receive the MT-SDT indication in paging message and the CG-SDT resource is not valid, UE can send the MT-SDT specific preamble to network. Then the network can avoid assigning the larger UL resource to transmit the response compare with MO-SDT. So we think we can discuss these two options for selecting the RA-SDT resource.

Proposal 3: When there is pending UL SDT data available, UE can use RA-SDT resource in R17 to transmit the response.

Proposal 4: When there is no pending UL SDT data available, RAN2 can discuss the following options for RA-SDT resource:

- Option 1: UE use the non-SDT RA resources to transmit the response.

- Option 2: Define MT-SDT specific RA resource.

When the CG-SDT resources are not valid and RA-SDT resources are not selected by UE to transmit the response, UE can use the non-SDT RA resources to initiate the MT-SDT procedure. And UE set the resume cause as MT-SDT.

Proposal 5: When CG-SDT resources and RA-SDT resources are not selected, UE can use the no-SDT RA resources to transmit the response for MT-SDT.

Besides, when there is pending UL SDT data, it is possible that network does not configure the MO-SDT resource. As in RAN2#120 meeting, it was agreed that it is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.

It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.

Then in this case, there are two possible UE behaviours:

-	Option A: Legacy Resume procedure is initiated instead of MT-SDT procedure.

-	Option B: MT-SDT procedure is initiated with non-SDT random access resources for the initial UL transmission.

Although there is no enough UL resource for the initial UL transmission of MT-SDT procedure, the pending UL data/signalling can be transmitted via subsequent UL transmission of MT-SDT procedure. So Option B is preferred.

Proposal 6: When there is pending UL SDT data and the MO-SDT RA resources and/or CG-SDT resource is not configured, UE initiates the MT-SDT procedure using non-SDT RA resource.

3 Conclusion

According to the above discussion, the following observations and proposals are formulated:

Observation 1: Whether there is pending UL SDT data available will have impact on the UL resource selection.

Observation 2: If there is no pending UL SDT data available and UE selects the RA-SDT resource to transmit the response, this will cause the UL resource waste.

Proposal 1: Before UE selecting the UL resource to transmit the response, UE should check DVT as in R17.

Proposal 2: It is beneficial to optimize the case when CG periodicity is much longer than paging cycle to reduce the possible delay.

Proposal 3: When there is pending UL SDT data available, UE can use RA-SDT resource in R17 to transmit the response.

Proposal 4: When there is no pending UL SDT data available, RAN2 can discuss the following options for RA-SDT resource:

- Option 1: UE use the non-SDT RA resources to transmit the response.

- Option 2: Define MT-SDT specific RA resource.

Proposal 5: When CG-SDT resources and RA-SDT resources are not selected, UE can use the no-SDT RA resources to transmit the response for MT-SDT.

Proposal 6: When there is pending UL SDT data and the MO-SDT RA resources and/or CG-SDT resource is not configured, UE initiates the MT-SDT procedure using non-SDT RA resource.

4 References

- Report of 3GPP TSG RAN WG2 meeting #121;