3GPP TSG-RAN WG2 Meeting #122	R2-2306467

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	7.7.4.1.1

Source:	Ericsson

Title:	TN – NTN mobility enhancements

Document for:	Discussion, Decision

# 1	Introduction

The work item for NR NTN enhancements in Rel-18 [1] emphasizes NTN-TN and NTN-NTN mobility and service continuity as an important area for improvements. For cell reselection enhancements, the following aspects are considered:

- “For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point.”

- “Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized).”

In this paper, we address the second aspect above.

# 2	Discussion

## 2.1	NTN neighbour cell information broadcast in TN cells

During RAN2#120, some companies raised the question of how a UE can identify whether a neighbour cell frequency broadcast in System Information belongs to a Terrestrial Network or a Non-Terrestrial Network. Even though TN and NTN have generally been assigned separate band numbers, there is an exception in band 1 in which both TN and HAPS can operate [2]. In addition, some NTN bands overlap in frequency with TN bands, for instance band 24 and band 255 [3,4]. Therefore, UEs cannot differentiate network access type based on band numbers. The possibility to add a separate (explicit) indication was discussed at RAN2#121bis and the following agreement was made:

- On a frequency band number shared by TN and NTN (e.g., n1), if NTN-specific assistance information is NOT provided for a neighbour cell configured in SIB3/SIB4, UE assumes this is a TN neighbour cell. This understanding is also applicable for Rel-17 and it does not need any spec update.

The situation slightly differs depending upon the type of network where the UE is camping. The agreement above addresses the case of a UE camping in NTN when TN neighbour cell information is broadcast in SI. However, when a UE is camping on TN and there exists an NTN neighbour cell, it is still FFS whether its information can be broadcast in SIB19, and a new solution is needed. Following statement is copied from the RAN2#121bis meeting report.

(16/29) Proposal 1: in TN cell SIB3/SIB4, NTN-config-r17 is provided for NTN neighbour cells

- Postponed to the next meeting.

For a UE camping in TN with NTN neighbours, neighbour cell information is obtained from SIB3 and SIB4, but for the UE to understand that a neighbour cell is an NTN cell and be able to measure its SSBs, NTN-config-r17 (or similar information) needs to be provided as well. However, broadcast of SIB19 in a TN cell may indeed be problematic according to the current specification. There are a few procedures in which the presence of SIB19 indicates the serving cell is non-Terrestrial. The following excerpt from RRC [5] shows an example. Upon SIB19 acquisition, a UE starts or re-starts T430 timer (ntn-UlSyncValidityDuration), which indicates the maximum validity time of the satellite assistance information, for the serving cell. Hence, whenever SIB19 is broadcast, a UE that supports NTN acquires this SIB and assumes that the serving cell is non-Terrestrial.

During RAN2#121bis-e, one company claimed in the online discussion that this does not constitute a real limitation. However, in our understanding, the text explicitly states that the timer is started for the serving cell, meaning the serving cell is understood as NTN. In addition, changes to this behaviour at this point may imply non-backwards compatibility.

The presence of SIB19 implies that serving cell is non-Terrestrial.

--------------------------------------------------------- 38.331 ---------------------------------------------------------

##### 5.2.2.4.21	Actions upon reception of SIB19

Upon receiving SIB19, the UE in RRC_CONNECTED shall:

1>	start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell;

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

--------------------------------------------------------- 38.331 ---------------------------------------------------------

As a result, it might not be presently possible to broadcast SIB19 in TN cells which creates an additional issue. The current stage 2 specification (see the excerpt below) [6] states that it is up to UE implementation to measure NTN neighbour cells whenever the corresponding satellite assistance information is not provided. This applies to both RRC_CONNECTED and RRC_IDLE modes. Thus, if SIB19 or its contents are not made available in a TN cell, a UE may not be able to measure an NTN neighbour cell leading to restricted TN to NTN RRC_IDLE mode mobility.

It is up to UE implementation to measure an NTN neighbour cell if the corresponding satellite assistance information is not available.

RRC_IDLE mode mobility from TN to NTN may be restricted due to the lack of means to provide a UE with the corresponding satellite assistance information.

--------------------------------------------------------- 38.300 ---------------------------------------------------------

#### 16.14.3.3	Measurements

[…]

For a UE in Idle/Inactive mode it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB4 but not included in SIB19.

For a UE in Connected mode, it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19.

--------------------------------------------------------- 38.300 ---------------------------------------------------------

Given the above measurement restriction, a terrestrial network needs to provide satellite assistance information in the System Information to facilitate RRC_IDLE mode mobility. Although SIB19 broadcast may not be possible, a UE only requires the contents of the IE NTN-config-r17 to carry out neighbour cell measurements. Thus, a simple solution to the problem is to broadcast this information in SIB3/SIB4 only for NTN neighbour cells. As a result, this information allows a UE camping on a TN cell to differentiate whether the neighbour cell belongs to TN or NTN.

During RAN2#121bis-e, a few companies proposed to down-prioritize this issue since the WID states that mobility from NTN to TN is prioritized. Others acknowledged that the problem but stated that a UE could still access an NTN cell after entering the any cell state. The overall objective of NTN Release 18 enhancements is to secure service continuity between TN and NTN. Provided that the solution might be rather straightforward, RAN2 should continue further discussing the issue as it falls within Release 18 scope.

Satellite assistance information, i.e., NTN-config-r17, for NTN neighbour cells is provided in SIB3/SIB4 of TN cells.

## 2.2	Cell reselection enhancements for TN-NTN mobility

This section deals with RRC_IDLE mobility enhancements to secure NTN-TN service continuity which are mainly targeted to minimize UE power consumption [7].

### 2.2.1	Reuse of Release 17 IEs

During RAN2#121bis-e, the format of assistance information of TN cells within the coverage of an NTN cell was discussed and the following relevant agreements were made:

- For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one).

- Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately.

One of the remaining FFS is related to the possible re-use of the Rel-17 referenceLocation and distanceThresh IEs, defined for example in SIB19 for location-based measurement initiation.

If we reuse the Rel-17 referenceLocation IE, each TN coverage area will require 48 bits to describe its geographical location. This seems like a reasonable approach considering the number of TN coverage areas should typically not be that many per NTN cell since, as agreed, each area describes a group of cells.

Reusing the Rel-17 distanceThresh IE as a distance threshold to the TN geographical location is also possible. We share the concern that some other companies raised during the online session: the value range of 50m to 3276km and the step size of 50m might not be optimal for initiation of TN-based measurements within an NTN cell. This value range and step size were optimized for the requirements of NTN cells. In contrast, coarser steps could be more suitable to describe TN coverage areas and an adaption could be considered.

Reuse the Rel-17 referenceLocation IE to describe the geographical location of a TN coverage area within an NTN cell.

Discuss the necessary value range and step size of distanceThresh IE for initiation of the TN measurements within an NTN cell.

### 2.2.3	Assistance information format

RAN2 has agreed to associate the TN coverage information with the frequency information. During RAN2#121bis-e, the following two options were discussed:

Option 1:	Use a list of frequencies under each TN area information

Option 2:	Introduce TN coverage area identity and use this identifier to point out the TN frequencies listed in SIB4 and SIB5

Including a list of frequencies per TN coverage area information following option 1 is simple and straightforward, although option 2 is indeed more efficient in terms of signalling overhead. However, the benefits of option 2 depend on the number of TN coverage areas provided per NTN cell and the number of frequencies provided per TN coverage area. If only a moderate number of TN coverage areas and frequencies are configured, which we consider the most likely case, the difference in signalling efficiency between the two options will not be that obvious. In addition, option 2 has the drawback that SIB4 and SIB5 may need to be updated frequently if the TN coverage area changes, e.g., for an Earth moving cell.

Adopt option 1, provide a list of frequencies under each TN coverage area information.

The following example illustrates how to implement option 1 in an efficient manner if SIB19 were to be selected. Note that whether to use an existing SIB (e.g., SIB19) or introducing a new one is still under consideration (FFS). We consider it should be further discussed once there is more clarity of all the enhancements within the mobility objective that would benefit from a new SIB.

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]]

[[

tn-CoverageAreaList-r18                  TN-CoverageAreaList-r18                     OPTIONAL,       -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

TN-CoverageAreaList-r18 ::=          SEQUENCE (SIZE(1..maxNrofTnCoverageArea-r18))  OF TN-CoverageArea-r18

TN-CoverageArea-r18 ::=              SEQUENCE {

tn-ReferenceLocation-r18                 ReferenceLocation-r17                                  OPTIONAL,       -- Need R

tn-DistanceThresh-r18                    INTEGER(0..65525)                                   OPTIONAL,       -- Need R

tn-CarrierFreqList-r18                   TN-CarrierFreqList-r18                                      OPTIONAL        -- Need R

}

TN-CarrierFreqList-r18 ::=          SEQUENCE (SIZE(1..maxFreq))  OF ARFCN-ValueNR

}

-- TAG-SIB19-STOP

-- ASN1STOP

### 2.2.4	Dedicated signalling

The last FFS relates to the need to use dedicated signalling to convey TN coverage area information. During the online discussion, proponent companies explained that the main advantage of a dedicated message would be to provide more detailed and accurate information specific to the area where the UE is located. Firstly, cell reselection is an RRC_IDLE mode procedure and it would make little sense for a UE to transition to RRC_CONNECTED just to get more accurate information. Secondly, there are not presently any stringent geographical accuracy requirements to start neighbour measurements. Therefore, we think that the potential benefits of this optimization are not very clear.

RAN2 does not specify dedicated signalling for TN coverage within an NTN cell.

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	The presence of SIB19 implies that serving cell is non-Terrestrial.

Observation 2	It is up to UE implementation to measure an NTN neighbour cell if the corresponding satellite assistance information is not available.

Observation 3	RRC_IDLE mode mobility from TN to NTN may be restricted due to the lack of means to provide a UE with the corresponding satellite assistance information.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Satellite assistance information, i.e., NTN-config-r17, for NTN neighbour cells is provided in SIB3/SIB4 of TN cells.

Proposal 2	Reuse the Rel-17 referenceLocation IE to describe the geographical location of a TN coverage area within an NTN cell.

Proposal 3	Discuss the necessary value range and step size of distanceThresh IE for initiation of the TN measurements within an NTN cell.

Proposal 4	Adopt option 1, provide a list of frequencies under each TN coverage area information.

Proposal 5	RAN2 does not specify dedicated signalling for TN coverage within an NTN cell.

# References

RP-222654, NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#97, September 12-16, 2022.

3GPP TS 38.104, Base Station (BS) radio transmission and reception, Release 17, V17.8.0.

R2-2212448, Discussion on NR NTN Cell Reselection Enhancement, Samsung, RAN2#120, November 2022.

3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception, Part 5: Satellite access Radio Frequency (RF) and performance, Release 17, V17.2.0.

3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, Release 17, V17.3.0.

3GPP TS 38.300, Stage 2, Release 17; V17.3.0.

3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), Release 16.