3GPP TSG-RAN WG2 Meeting #122	R2-2306820

Incheon, Korea, May 22-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.822 | CR | 0013 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous updates for TR 38.822 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core, NR_MG_enh-Core, NR_DL1024QAM_FR1, NR_MBS-Core, TEI17, NR_FeMIMO-Core, NR_redcap-Core, NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Update to the latest feature lists from RAN1, RAN4 (R1-2304113 provided in LS R2-2304611, R4-2304659 provided in LS R2-2304633) - Updates to reflect the MBS feature list updates in R1-2302024 provided in LS R2-2302454 - Updates to reflect CRs agreed in R2-121 (R2-2300470, R2-2301739, R2-2301988, R2-2302025, R2-2302077, R2-2302099, R2-2302118, R2-2302276) - Miscellaneous editorial corrections |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to feature lists to align with the RAN1 and RAN4 feature lists and correction of editorial errors |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR will not be capture RAN1, RAN4 agreements on feature lists Miscellaneous errors in the TR remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.1.8, 6.1.11, 6.1.14, 6.2.10, 6.2.12, 6.2.19, 6.3.6, 6.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

### ***** First change *****

### 6.1.1	NR_FeMIMO

Table 6.1.1-1: Layer-1 feature list for NR_FeMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability nterpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra-cell beam management | 1. Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured joint TCI states per BWP per CC in a band 3. One MAC-CE activated joint TCI state per CC in a band 4. TCI state indication for update and activation a) MAC CE based TCI state indication for one active TCI state 5. The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band |  | unifiedJointTCI-r17 { maxConfiguredJointTCI-r17,maxActivatedTCIAcrossCC-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 5 candidate value {1, 2, 4, 8, 16} If a UE supports FG 23-1-1a, the signalled component values (except component 5) also apply to inter-cell beam management Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1a | Unified TCI with joint DL/UL TCI update for inter-cell beam management | 1. Support of unified TCI with joint DL/UL TCI update for inter-cell beam management 2. Support K additional MAC-CE activated joint TCI states per CC in a band 3. Support K additional MAC-CE activated joint TCI states across all CC(s) in a band | 23-1-2, 23-1-1 | unifiedJointTCI-InterCell-r17 { additionalMAC-CE-PerCC-r17, additionalMAC-CE-AcrossCC-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component candidate values for K: {0,1,2,4} Note: A UE that supports 23-1-1a supports K additional MAC-CE activated joint TCI states across all CC(s) in a band in addition to the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band signalled in FG 23-1-1. The signalled value in component 3 of 23-1-1a plus the signalled value in component 5 of 23-1-1 determine the maximum number of MAC-CE activated joint TCI states across all CC(s) in a band that are applied to intra and inter-cell beam management jointly. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | 1. TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated joint TCI states per CC in a band | 23-1-1 | unifiedJointTCI-multiMAC-CE-r17 { minBeamApplicationTime-r17, maxNumMAC-CE-PerCC } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2 Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} Note: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signaled in 23-1-1, component 5 Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1c | Scell BFR with unified TCI framework | 1. Support of Scell BFR with unified TCI framework |  | unifiedJointTCI-SCellBFR-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | The maximum number of CCs configured with Scell BFR with unified TCI framework in a band with SpCell BFR is given by FG 16-1f, in this case FG 16-1f includes SpCell | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1d | Per BWP TCI state pool configuration for CA mode | 1. Support of TCI state pool configuration per BWP for CA mode | 23-1-1 | unifiedJointTCI-perBWP-CA-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1e | TCI state pool configuration with TCI pool sharing for CA mode | 1. Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC 2. The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band | 23-1-1 | unifiedJointTCI-ListSharingCA-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4, 8} A UE that supports 23-1-1 together with CA must support this FG | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1f | Common multi-CC TCI state ID update and activation | Common multi-CC TCI state ID update and activation | 23-1-1 | unifiedJointTCI-commonMultiCC-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1g | Beam misalignment between the DL source RS in the TCI state | Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS | 23-1-1 | unifiedJointTCI-BeamAlignDLRS-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | FR2 only | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1h | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS | For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS | 23-1-1 | unifiedJointTCI-PC-association-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1i | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH | Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | unifiedJointTCI-Legacy-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Note: This has no impact on detail signaling design for SRS TCI indication | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1m | Indication/configuration of R17 TCI states for SRS | Support of indication/configuration of R17 TCI states for SRS (except for periodic/semi-persistent SRS for BM) reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | unifiedJointTCI-Legacy-SRS-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Note: This has no impact on detail signaling design for SRS TCI indication | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | unifiedJointTCI-Legacy-CORESET0-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1k | Maximum number of configured CC lists | Maximum number of configured CC lists per cell group for common multi-CC TCI state ID update and activation | 23-1-1f or 23-10-1f | unifiedJointTCI-commonUpdate-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Component candidate values: {1,2,3,4} | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting (for inter-cell BM and mTRP) | - Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K SSBRI-RSRP pairs in one report where a pair is associated with a PCI different from serving cell PCI can be reported 3. The maximum number of RRC-configured PCI(s) different from serving cell PCI for L1-RSRP measurement 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI across all CC |  | unifiedJointTCI-mTRP-InterCell-BM-r17 { maxNumAdditionalPCI-L1-RSRP-r17, maxNumSSB-ResourceL1-RSRP-AcrossCC-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7} Component 4 candidate values: {1, 2, 4, 8} Note: K is equal to maxNumberNonGroupBeamReporting Note: component 4 is also counted in FG16-1g/16-1g-1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | - Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  | mpe-Mitigation-r17 { maxNumP-MPR-RI-pairs-r17, maxNumConfRS-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate values: {1,2,3, 4} Component 3 candidate values: {1, 2, 4, 8, 12, 16, 28, 32, 48, 64} Note: FR2 only Note: Component 3 is also counted in FG16-1g/16-1g-1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | UE capability value reporting | 1. Supported UE capability value and corresponding max number of SRS ports for each UE capability value |  | srs-PortReport-r17 { capVal1-r17, capVal2-r17, capVal3-r17, capVal4-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 1 candidate values: Up to 4 value each with one value of {1,2,4} Note: the reported list contains only unique value | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4a | Semi-persistent/aperiodic capability value report | Support of Semi-persistent/aperiodic capability value report | 23-1-4, 2-22 or 2-23 or 2-23a or 16-1a-1 or 16-1a-4 or 16-1a-5 | srs-PortReportSP-AP-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Note: UE that supports this FG, supports capability value reporting together with the AP/SP L1-RSRP/L1-SINR reporting(s) that UE supports in Rel-15/16, reported by FG2-22. FG2-23, FG2-23a, FG16-1a-1, FG16-1a-4 and FG16-1a-5 | Optional with capability signaling |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs including PDCCH repetition for Type 3 CSS 2. Required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  | mTRP-PDCCH-Repetition-r17 { numBD-twoPDCCH-r17, maxNumOverlaps-r17, } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | Component 2 candidate values: 2 or 3 Component 3 candidate values: {1,2,3, 5, 10, 20, 40} NOTE 1: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. NOTE 2: for component 3, each unique pair of overlaps is counted as one. NOTE 3: This FG does not include supporting Two QCL-TypeD in time-domain overlapping CORESETs in FR2. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | 23-2-1 | mTRP-PDCCH-individual-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1b | PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | 3-2 23-2-1 | mTRP-PDCCH-anySpan-3Symbols-r17 | MIMO-ParametersPerBand | n/a | FR1 only | n/a | Applicable to 15KHz SCS only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1c | PDCCH repetition with PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot | Support of PDCCH repetition for PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot | 22-12 23-2-1 | mTRP-PDCCH-singleSpan-r17 | MIMO-ParametersPerBand | No | FR1 only | No | Applicable to 15KHz SCS only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1d | PDCCH repetition for Case 2 PDCCH monitoring with a span gap | 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs | 3-5b, 23-2-1 | mTRP-PDCCH-Case2-1SpanGap-r17 { scs-15kHz-r17, scs-30kHz-r17, scs-60kHz-r17, scs-120kHz-r17 } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | This capability is necessary for each SCS. Component 2 candidate values: {intra-span, inter-span, both} Component 3 candidate values: {4, 8, 16, 32, 44, 64, no limit} Component 4 candidate values: {4, 8, 16, 32, 44, 64, 128, 256, 512, no limit} NOTE: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit (X) is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. - Candidate value “no limit” does not imply BD limit can be exceeded. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1e | PDCCH repetition for Rel-16 PDCCH monitoring | 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. X per CC 4. X across all CCs | 11-2, 23-2-1 | mTRP-PDCCH-legacyMonitoring-r17 { scs-15kHz-r17, scs-30kHz-r17 } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: {4, 8, 16, 32, 44, 64, no limit} Component 4: {4, 8, 16, 32, 44, 64, 128, 256, 512, no limit} NOTE: - Components 3 and 4 are reported only if UE supports inter-span PDCCH repetition. - The limit X is associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span, where “received” and “not been received” is w.r.t. the end of the corresponding span of PDCCH candidate. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. - Candidate value “no limit” does not imply BD limit can be exceeded. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition | 23-2-1 | mTRP-PDCCH-TwoQCL-TypeD-r17 | MIMO-ParametersPerBand | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-4 | Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | Support of simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | 23-2-1, 16-2a | mTRP-PDCCH-multiDCI-multiTRP-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a | Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ 3. Supported number of SRS resources in one SRS resource set | 2-14 | mTRP-PUSCH-TypeA-CB-r17 | FeatureSetUplink-v1710 | n/a | n/a | n/a | Component 3 candidate values: {1,2 ,4} NOTE: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2 | Multi-TRP PUSCH repetition (type A) – non-codebook based | 1. Support of multi-TRP PUSCH repetition for non-codebook based PUSCH (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘nonCodebook’ 3. Supported number of SRS resources in one SRS resource set | 2-15 | mTRP-PUSCH-RepetitionTypeA-r17 | FeatureSetUplink-v1710 | n/a | n/a | n/a | Component 3: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2a | Two associated CSI-RS resources | Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH | 2-15a, 23-3-1-2 | mTRP-PUSCH-twoCSI-RS-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2b | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 4. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 5. UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously | 23-3-1-2a | mTRP-PUSCH-CSI-RS-r17 { maxNumPeriodicSRS-r17, maxNumAperiodicSRS-r17, maxNumSP-SRS-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {0 to 8} Component 4: {1 to 16} Component 5: {1,2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping for Multi-TRP PUSCH repetition | Support of cyclic mapping when the number of repetitions is larger than 2 | 23-3-1 or 23-3-1-2 | mTRP-PUSCH-cyclicMapping-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Candidate component values: {for repetition Type A, for repetition Type B, both} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field for Multi-TRP PUSCH repetition | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | 23-3-1 or 23-3-1-2 | mTRP-PUSCH-secondTPC-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs.) | 23-3-1 or 23-3-1-2 | mTRP-PUSCH-twoPHR-Reporting-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 | mTRP-PUSCH-A-CSI-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 | mTRP-PUSCH-SP-CSI-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) | 23-3-1 or 23-3-1-2 | mTRP-PUSCH-CG-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) – codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ 3. Supported number of SRS resources in one SRS resource set | 2-14, 11-5 | mTRP-PUSCH-TypeB-CB-r17 | FeatureSetUplinkPerCC-v1700 | No | No | No | Component 3 candidate values: {1,2,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-3 | Multi-TRP PUSCH repetition (type B) – non-codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. support of two SRS resource sets with usage set to ‘nonCodebook’ 3. supported number of SRS resources in one SRS resource set | 2-15, 11-5 | mTRP-PUSCH-RepetitionTypeB-r17 | FeatureSetUplinkPerCC-v1700 | No | No | No | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition scheme 1 (inter-slot) | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition)- sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource 3. Supported PUCCH formats for this scheme |  | mTRP-PUCCH-InterSlot-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 3 candidate values: {PF0/2, PF1/3/4, PF0-4} Note: power control parameter sets (w/o spatial relation info) only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2b | Cyclic mapping for multi-TRP PUCCH repetition | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | 23-3-2 | mTRP-PUCCH-CyclicMapping-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c | Second TPC field for multi-TRP PUCCH repetition | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1/1_2 | 23-3-2 | mTRP-PUCCH-SecondTPC-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2d | Updating two Spatial relation or two sets of power control parameters for PUCCH group | Support of updating two Spatial Relation Info’s / two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE | 23-3-2 | mTRP-PUCCH-MAC-CE-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2e | Maximum number of power control parameter sets configured for multi-TRP PUCCH repetition in FR1 | Maximum number of power control parameter sets configured for multi-TRP PUCCH repetition in FR1 | 23-3-2 | mTRP-PUCCH-maxNum-PC-FR1-r17 | MIMO-ParametersPerBand | n/a | FR1 only | n/a | Candidate values: {3 to 8} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | 1. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource 3. Supported PUCCH formats for this scheme |  | mTRP-PUCCH-IntraSlot-r17 | FeatureSetUplink-v1710 | n/a | n/a | n/a | Component 3 candidate values: {PF0/2, PF1/3/4, PF0-4} Note: power control parameter sets (w/o spatial relation info) only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | - Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 | 16-2a | mTRP-inter-Cell-r17 { maxNumAdditionalPCI-Case1-r17, maxNumAdditionalPCI-Case2-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7} Component 3 candidate values: {0,1,2,3,4,5,6,7} Note: case1 and case2 cannot be enabled simultaneously as any configuration that is not based on Case 1 is defined as Case 2 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | - Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs |  | mTRP-GroupBasedL1-RSRP-r17 { maxNumBeamGroups-r17, maxNumRS-WithinSlot-r17, maxNumRS-AcrossSlot-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 1 candidate values: {1,2,3,4} Component 2 candidate values: {2,3,4,8,16,32,64} Component 3 candidate values: {8, 16, 32, 64, 128} Note: component 2 and 3 are also counted in FG 16-1g and 16-1g-1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR based on two BFD-RS sets | - Maximum number of supported measured BFD-RS resources per set per BWP 2. The maximum number of CCs per band configured with BFR (including spCell/Scell/MTRP BFR in Rel-15/16/17) 3. Supported maximum number of measured BFD-RS resources across two BFD-RS sets per BWP |  | mTRP-BFR-twoBFD-RS-Set-r17 { maxBFD-RS-resourcesPerSetPerBWP-r17, maxBFR-r17, maxBFD-RS-resourcesAcrossSetsPerBWP-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8, 9} Component 3 candidate values: {2,3,4} Note: component 3 is also counted in FG 16-1g and 16-1g-1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Max number of PUCCH-SR resources for MTRP BFRQ per cell group |  | mTRP-BFR-PUCCH-SR-perCG-r17 | MIMO-ParametersPerBand | No | Yes | No | Component candidate values: {1, 2} Note: A UE that supports FG 23-5-2 must indicate this FG is supported with at least component candidate value 1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2b | Association between a BFD-RS resource set on SpCell and a PUCCH SR resource | Support of association between a BFD-RS resource set on SpCell and a PUCCH SR resource | 23-5-2a | mTRP-BFR-association-PUCCH-SR-r17 | MIMO-ParametersPerBand | No | Yes | No |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2c | MAC-CE based update of explicit BFD-RS | - Support of MAC-CE based update of explicit BFD-RS for mTRP BFR 2. Maximum number of configured candidate BFD-RS per BWP for MAC-CE based update | 23-5-2 | mTRP-BFD-RS-MAC-CE-r17 | MIMO-ParametersPerBand | No | Yes | No | Component 2 candidate values: {4, 8, 12, 16, 32, 48, 64} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. Support of SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH |  | sfn-SchemeA-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1-1 | SFN scheme A (scheme 1) for PDCCH only | Support of SFN scheme A for PDCCH scheduling single TRP PDSCH |  | sfn-SchemeA-PDCCH-only-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching – scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 or 23-6-1b | sfn-SchemeA-DynamicSwitching-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH scheduled by single TRPPDCCH |  | sfn-SchemeA-PDSCH-only-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. Support of SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH |  | sfn-SchemeB-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching – scheme B | Support of dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | 23-6-2 or 23-6-2b | sfn-SchemeB-DynamicSwitching-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH scheduled by single TRP PDCCH |  | sfn-SchemeB-PDSCH-only-r17 | FeatureSetDownlink-v1700 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 or 23-6-1-1 | sfn-SimulTwoTCI-AcrossMultiCC-r17 | MIMO-ParametersPerBand | No | Yes | No |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup for SFN | - Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI format 1_0/1_1/1_2 when PDSCH is scheduled with offset equal or larger than the threshold, if applicable 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | 23-6-1 or 23-6-2 | sfn-DefaultDL-BeamSetup-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Note: FR2 only for component 1 and 3 only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN PDCCH | - Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 23-6-1 or 23-6-2 or 23-6-1-1 | sfn-DefaultUL-BeamSetup-r17 | MIMO-ParametersPerBand | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-5 | Support implicit configuration of RS(s) with two TCI states for beam failure detection | Support RS(s) with two TCI states configured implicitly for beam failure detection enhancement for HST |  | sfn-ImplicitRS-twoTCI-r17 | MIMO-ParametersPerBand | n/a | n/a |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-6 | QCL-TypeD collision handling with CORESET with 2 TCI states | Support of identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI states which overlaps with another CORESET. |  | Sfn-QCL-TypeD-Collision-twoTCI-r17 | MIMO-ParametersPerBand | n/a | n/a |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is b) Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis c) Maximum total number of CMRs for NCJT measurement d) Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI |  | mTRP-CSI-EnhancementPerBC-r17 { maxNumNZP-CSI-RS-r17, cSI-Report-mode-r17, supportedComboAcrossCCs-r17, codebookMode-NCJT-r17 } | CA-ParametersNR-v1700 | n/a | n/a | n/a | Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: {mode 1 with X=0, mode 2, both} Component 4 candidate values: a) {2, 4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {2,3,4, …, 256} Component 5 candidate values: {mode 1, both mode 1 and mode 2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1c | Basic Features of CSI Enhancement for Multi-TRP – number of CPUs | Number of CPUs occupied by a pair of CMRs for NCJT CSI hypotheses | 23-7-1 | mTRP-CSI-numCPU-r1 | MIMO-ParametersPerBand | n/a | n/a | n/a | Component candidate values: {2,3 ,4} Note: Maximum number of CPUs is reported in FG 2-35 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1b | Active CSI-RS resources and ports in the presence of multi-TRP CSI | - List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-7-1 | codebookComboParameterMultiTRP-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {‘NCJT’, NCJT+Type 1 SP (for sTRP)} {Codebook 2, Codebook 3} = {(NULL, NULL}), {“Rel 16 combinations in FG 16-8”}, {“New Rel17 combinations in FG 23-9-5”}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2, 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. Note2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 | Maximum value of numberOfSingleTRP-CSI-Mode1 | 23-7-1 | mTRP-CSI-additionalCSI-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 1 candidate value set: {X=1, X=2} Note: UE reports this capability only when UE reports “mode 1 with X=0” or “both” for component 3 of FG 23-7-1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | mTRP-CSI-N-Max2-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for Rel-17 Multi-TRP CSI enhancement and a single CMR configured for Single-TRP measurement in a CSI reporting setting | 23-7-1 | mTRP-CSI-CMR-r17 | MIMO-ParametersPerBand | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 | srs-TriggeringOffset-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a | Candidate 1 component values: {1, 2, 4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | srs-TriggeringDCI-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | - Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | srs-AntennaSwitchingBeyond4RX-r17 { supportedSRS-TxPortSwitchBeyond4Rx-r17, entryNumberAffectBeyond4Rx-r17, entryNumberSwitchBeyond4Rx-r17 } | BandParameters-v1710 | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: For any indicated value, x shall be equal to or smaller than the one associated with the largest y Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Component 2 and Component 3 are optional. If reported, the reported values for component 2 and component 3 are not valid for the same values of xTyR in component 1 reported with Rel-15/16 UE capability reporting | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 | srs-AntennaSwitching2SP-1Periodic-r17 | FeatureSetUplink-v1710 | n/a | n/a | n/a | Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated at the same time. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 | srs-increasedRepetition-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS with frequency hopping | Support of partial frequency sounding for SRS with frequency hopping | 2-52 | srs-partialFrequencySounding-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 | srs-startRB-locationHoppingPartial-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | srs-combEight-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53, 2-55 | srs-ExtensionAperiodicSRS-r17 | FeatureSetUplink-v1710 | N/A | N/A | N/A | Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-10 | 1 aperiodic SRS resource set for 1T4R | Support of 1 aperiodic SRS resource sets for 1T4R. | 10-11, 2-55 | srs-OneAP-SRS-r17 | FeatureSetUplink-v1710 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-11 | Partial frequency sounding of SRS for non-frequency hopping case | Support of partial frequency sounding for SRS for non-frequency hopping case. | 23-8-6 | srs-partialFreqSounding-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=1 and R=1 2. Support rank 1,2 3. Support parameter combinations with M=1 | 2-35 | codebookParametersfetype2PerBC-r17 | CA-ParametersNR-v1700 | n/a | n/a | n/a | Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4 | codebookComboParameterMixedTypePerBC-r17 | CA-ParametersNR-v1700 | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL}, {FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1}, {eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} NOTE 1: if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations. NOTE 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4. NOTE 3: Up to 4 combinations for component 1. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 2. Support parameter combinations with M=2 | 23-9-1 | fetype2R1-r17 | CodebookParametersfetype2-r17 | n/a | n/a | n/a | Component 1 candidate values - Maximum 8 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | 23-9-1 | fetype2Rank3Rank4-r17 | CodebookParametersfetype2-r17 | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | - Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=2 | 23-9-2 | fetype2R2-r17 | CodebookParametersfetype2-r17 | n/a | n/a | n/a | Component 2 candidate values: • Maximum 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-10-1 | Unified TCI with separate DL/UL TCI update for intra-cell beam management | 1. Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured DL TCI states per BWP per CC 3. The maximum number of configured UL TCI states per BWP per CC 4. One MAC-CE activated DL TCI state per CC in a band 5. One MAC-CE activated UL TCI state per CC in a band 6. TCI state indication for update and activation a) MAC CE based TCI state indication for one active DL/UL TCI state 7. The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band 8. The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band | 23-1-1 | unifiedSeparateTCI-r17 { maxConfiguredDL-TCI-r17, maxConfiguredUL-TCI-r17, maxActivatedDL-TCIAcrossCC-r17, maxActivatedUL-TCIAcrossCC-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate value {4, 8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate value {4, 8, 12, 16, 24, 32, 48, 64} Component 7 candidate value {1, 2, 4, 8, 16} Component 8 candidate value {1, 2, 4, 8, 16} If a UE supports FG 23-10-1m, the signalled component values (except components 7 and 8) apply to intra- and inter-cell beam management jointly | Optional with capability signalling |
| 23. NR_FeMIMO | 23-10-1b | Unified TCI with separate DL/UL TCI update for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC | 1. TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band | 23-10-1 | unifiedSeparateTCI-multiMAC-CE-r17 { minBeamApplicationTime-r17, maxActivatedDL-TCIPerCC-r17, maxActivatedUL-TCIPerCC-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | If a UE supports FG 23-10-1m, the signalled component values also apply to inter-cell beam management | Optional with capability signalling |
| 23. NR_FeMIMO | 23-10-1d | Per BWP DL/UL-TCI state pool configuration for CA mode | 1. Support of DL/UL TCI state pool configuration per BWP for CA mode | 23-10-1 | unifiedSeparateTCI-perBWP-CA-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-10-1e | TCI state pool configuration with DL/UL-TCI pool sharing for CA mode | - Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC 2. The maximum number of configured DL TCI state pools across all BWPs and all CCs in a band 3. The maximum number of configured UL TCI state pools across all BWPs and all CCs in a band |  | unifiedSeparateTCI-ListSharingCA-r17 { maxNumListDL-TCI-r17, maxNumListUL-TCI-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4, 8} Component 3 candidate values: {1, 2, 4, 8} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-10-1f | Common multi-CC DL/UL-TCI state ID update and activation with separate DL/UL TCI update | Common multi-CC DL/UL-TCI state ID update and activation | 23-10-1 | unifiedSeparateTCI-commonMultiCC-r17 | MIMO-ParametersPerBand | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-10-1m | Unified TCI with separate DL/UL TCI update for inter-cell beam management with more than one MAC-CE activated separate TCI state per CC | 1. Support of unified TCI with separate DL/UL TCI update for inter-cell beam management with more than one MAC-CE activated separate TCI state per CC 2. Support K additional MAC-CE activated DL TCI states per CC in a band 3. Support K additional MAC-CE activated UL TCI states per CC in a band 4. Support K additional MAC-CE activated DL TCI states across all CC(s) in a band 5. Support K additional MAC-CE activated UL TCI states across all CC(s) in a band | 23-10-1 | unifiedSeparateTCI-InterCell-r17 { k-DL-PerCC-r17, k-UL-PerCC-r17, k-DL-AcrossCC-r17, k-UL-AcrossCC-r17 } | MIMO-ParametersPerBand | n/a | n/a | n/a | Component candidate values for K: {0,1,2,4} Note: A UE that supports 23-10-1m supports K additional MAC-CE activated DL and K additional MAC-CE activated UL TCI states across all CC(s) in a band in addition to the maximum number of MAC-CE activated DL and UL TCI states across all CC(s) in a band signalled in FG 23-10-1. The signalled value in component 4 (5) of 23-10-1m plus the signalled value in component 7 (8) of 23-10-1 determine the maximum number of MAC-CE activated DL (UL) TCI states across all CC(s) in a band that are applied to intra and inter-cell beam management jointly. | Optional with capability signalling |

### ***** Next change *****

### 6.1.8	NR_cov_enh

Table 6.1.8-1: Layer-1 feature list for NR_cov_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | One of {5-14, 5-16, 11-6} | maxNumberPUSCH-TypeA-Repetition-r17 | BandNR | N/A | N/A | A UE that indicates support of this feature shall support type1-PUSCH-RepetitionMultiSlots, type2-PUSCH-RepetitionMultiSlots, pusch-RepetitionTypeA-r16 or pusch-RepetitionTypeA-v16c0. | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17} | puschTypeA-RepetitionsAvailSlot-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and Type 2 CG without repetition in RRC connected mode. |  | tb-ProcessingMultiSlotPUSCH-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | 30-3 | tb-ProcessingRepMultiSlotPUSCH-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | maxDurationDMRS-Bundling-r17 { fdd-r17, tdd-r17 } | BandNR | N/A | N/A | Candidate values for the maximum duration for FDD are {4, 8, 16, 32} Candidate values for the maximum duration for TDD are {2, 4, 8, 16} NOTE: DM-RS bundling is only applicable for UL transmissions with pi/2 BPSK, BPSK, and QPSK modulation orders for the corresponding physical channels. | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A over consecutive symbols | 30-4 and one of {5-14, 5-16, 5-17} | dmrs-BundlingPUSCH-RepTypeA-r17 | BandNR | N/A | N/A | This capability is applicable to following multiple carrier scenarios in addition to single carrier scenarios - FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time. - FR1 inter-band DL CA with a "single" uplink band configured, meaning no switching to transmit SRS on another carrier. - DL CA with "additional" UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured) - FR1 inter-band UL CA with DMRS bundling - SUL with DMRS bundling For the last three scenarios listed above, DMRS bundling can be applied with the following conditions: - Concurrent transmissions scheduled/configured over multiple carriers are not expected by UE - Only configuration of a single TAG - Only applicable for the back-to-back case (i.e., zero gap between two transmissions within an actual TDW) - Only one band can be configured with DMRS bundling at a time NOTE 1: Under the above conditions, phase continuity and power consistency within any actual TDW on one carrier is not impacted by operations on a different carrier. NOTE 2: Under the above conditions, the events defined in clause 6.1.7 of TS38.214 [20] for the carrier with DMRS bundling are not triggered by any transmission within any actual TDW on the other carrier. NOTE 3: If the modulation scheme higher than QPSK is scheduled for transmission on any carrier configured with DMRS bundling, DMRS bundling is not applicable according to UE feature 30-4 (i.e., the error case and up to UE implementation) | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B over consecutive symbols | 30-4, 11-5 | dmrs-BundlingPUSCH-RepTypeB-r17 | BandNR | N/A | N/A | This capability is applicable to following multiple carrier scenarios in addition to single carrier scenarios - FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time. - FR1 inter-band DL CA with a "single" uplink band configured, meaning no switching to transmit SRS on another carrier. - DL CA with "additional" UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured) - FR1 inter-band UL CA with DMRS bundling - SUL with DMRS bundling For the last three scenarios listed above, DMRS bundling can be applied with the following conditions: - Concurrent transmissions scheduled/configured over multiple carriers are not expected by UE - Only configuration of a single TAG - Only applicable for the back-to-back case (i.e., zero gap between two transmissions within an actual TDW) - Only one band can be configured with DMRS bundling at a time NOTE 1: Under the above conditions, phase continuity and power consistency within any actual TDW on one carrier is not impacted by operations on a different carrier. NOTE 2: Under the above conditions, the events defined in clause 6.1.7 of TS38.214 [20] for the carrier with DMRS bundling are not triggered by any transmission within any actual TDW on the other carrier. NOTE 3: If the modulation scheme higher than QPSK is scheduled for transmission on any carrier configured with DMRS bundling, DMRS bundling is not applicable according to UE feature 30-4 (i.e., the error case and up to UE implementation) | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols | 30-4, 30-3 | dmrs-BundlingPUSCH-multiSlot-r17 | BandNR | N/A | N/A | NOTE 1: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS. This capability is applicable to following multiple carrier scenarios in addition to single carrier scenarios - FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time. - FR1 inter-band DL CA with a "single" uplink band configured, meaning no switching to transmit SRS on another carrier. - DL CA with "additional" UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured) - FR1 inter-band UL CA with DMRS bundling - SUL with DMRS bundling For the last three scenarios listed above, DMRS bundling can be applied with the following conditions: - Concurrent transmissions scheduled/configured over multiple carriers are not expected by UE - Only configuration of a single TAG - Only applicable for the back-to-back case (i.e., zero gap between two transmissions within an actual TDW) - Only one band can be configured with DMRS bundling at a time NOTE 2: Under the above conditions, phase continuity and power consistency within any actual TDW on one carrier is not impacted by operations on a different carrier. NOTE 3: Under the above conditions, the events defined in clause 6.1.7 of TS38.214 [20] for the carrier with DMRS bundling are not triggered by any transmission within any actual TDW on the other carrier. NOTE 4: If the modulation scheme higher than QPSK is scheduled for transmission on any carrier configured with DMRS bundling, DMRS bundling is not applicable according to UE feature 30-4 (i.e., the error case and up to UE implementation). | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 over consecutive symbols | 30-4, 4-23 | dmrs-BundlingPUCCH-Rep-r17 | BandNR | N/A | N/A | This capability is applicable to following multiple carrier scenarios in addition to single carrier scenarios - FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time. - FR1 inter-band DL CA with a "single" uplink band configured, meaning no switching to transmit SRS on another carrier. - DL CA with "additional" UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured) - FR1 inter-band UL CA with DMRS bundling - SUL with DMRS bundling For the last three scenarios listed above, DMRS bundling can be applied with the following conditions: - Concurrent transmissions scheduled/configured over multiple carriers are not expected by UE - Only configuration of a single TAG - Only applicable for the back-to-back case (i.e., zero gap between two transmissions within an actual TDW) - Only one band can be configured with DMRS bundling at a time NOTE 1: Under the above conditions, phase continuity and power consistency within any actual TDW on one carrier is not impacted by operations on a different carrier. NOTE 2: Under the above conditions, the events defined in clause 6.1.7 of TS38.214 [20] for the carrier with DMRS bundling are not triggered by any transmission within any actual TDW on the other carrier. NOTE 3: If the modulation scheme higher than QPSK is scheduled for transmission on any carrier configured with DMRS bundling, DMRS bundling is not applicable according to UE feature 30-4 (i.e., the error case and up to UE implementation). | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | 30-4a or 30-4b or 30-4c | interSlotFreqHopInterSlotBundlingPUSCH-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | 30-4d | interSlotFreqHopPUCCH-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling | Support restarting DM-RS bundling after the events triggered by DCI or MAC CE that violate power consistency and phase continuity Note: Events which are triggered by DCI or MAC CE, but do not require UE capability to resume maintaining power consistency and/or phase continuity as specified in clause 6.1.7 of TS 38.214 [20] v17.3.0 are excluded from this feature | 30-4 | dmrs-BundlingRestart-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | dmrs-BundlingNonBackToBackTX-r17 | BandNR | N/A | N/A | NOTE: This capability is only applicable when UE is configured with single uplink carrier within a frequency range. | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | 4-23 or 25-2 | slotBasedDynamicPUCCH-Rep-r17 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | pusch-RepetitionMsg3-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |

### ***** Next change *****

### 6.1.11	NR_MBS

Table 6.1.11-1: Layer-1 feature list for NR_MBS

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | 1. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH. 3. Support of CFR configuration for broadcast. 4. Support of CORESET and common search space for broadcast. 5. Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. 6. Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. 7. Support MCCH change notification indication via DCI. 8. support of higher layer configured slot-level repetition up to 8 for MTCH 9. One G-RNTI per UE is supported for broadcast reception 10. Support of FDMed MCCH and PBCH 11. Support of up to 64QAM for FR1/FR2 |  | n/a | n/a | N/A | N/A | One G-RNTI per UE is supported for multicast reception | Optional without capability signalling |
| 33. NR_MBS | 33-1-1 | DCI indicated slot-level repetition up to 16 for broadcast MTCH | Support up to 16 times dynamic slot-level repetition for broadcast MTCH. | 33-1 | dci-BroadcastWith16Repetitions-r17 | FeatureSetDownlinkPerCC-v1730 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | fdm-BroadcastUnicast-r17 | FeatureSetDownlinkPerCC-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | 1. Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI for PCell. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. 5. Support of inter-slot TDM between group-common PDSCH for multicast and other PDSCHs in different slots. 6. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast |  | dynamicMulticastPCell-r17 | FeatureSetDownlink-v1700 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast 5) Support of Type-2 HARQ-ACK codebook for multicast on PUSCH/PUCCH with max number X of G-RNTIs | 33-2 | ack-NACK-FeedbackForMulticast-r17 | CA-ParametersNR-v1720 | N/A | N/A | Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2 | 33-2a, 33-2f | ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast dynamic scheduling | Support of PTP retransmission for multicast on the same cell as multicast initial transmission | 33-2a | ptp-Retx-Multicast-r17 | CA-ParametersNR-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for multicast | 33-2 | maxNumberG-RNTI-r17 | BandNR | [Yes] | Yes | Reporting type of FG 33-2e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | dynamicMulticastDCI-Format4-2-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | maxNumberMIMO-LayersMulticastPDSCH-r17 | FeatureSetDownlinkPerCC-v1700 | N/A | N/A | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | dynamicMulticastSCell-r17 | FeatureSetDownlinkPerCC-v1700 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-2i | Supported maximal modulation order for multicast PDSCH | 1. For FR1, up to 1024QAM is supported, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported, candidate values {64QAM, 256QAM} | 33-2 | maxModulationOrderForMulticast-r17 { fr1-r17, fr2-r17 } | BandNR | N/A | N/A | Note: A UE shall support the corresponding mandatory maximum modulation for unicast. | Optional with capability signalling |
| 33. NR_MBS | 33-2j | Supported maximum modulation order used for maximum data rate calculation for multicast PDSCH | 1. For FR1, up to 1024QAM is supported as maximum modulation order used for maximum data rate calculation for multicast PDSCH, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported as maximum modulation order used for maximum data rate calculation for multicast PDSCH, candidate values {64QAM, 256QAM} | 33-2 | maxModulationOrderForMulticastDataRateCalculation-r17 | FeatureSetDownlinkPerCC-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17 | BandNR | Yes | Yes | Candidate values for X is: {8, 16} This FG is reported for TN and licensed. | Optional with capability signalling |
| 33. NR_MBS | 33-3-1a | Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast for NTN and unlicensed | 33-2 | dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17 | BandNR | N/A | N/A | Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for multicast | 1. Support FDM between one dynamically scheduled unicast PDSCH and one dynamically scheduled group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2, or at least one of {33-5-1a, 33-5-1f} | fdm-MulticastUnicast-r17 | FeatureSetDownlinkPerCC-v1720 | N/A | N/A | Note: this FG does not support FDMed SPS | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. 2. Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC 3. Support TDM among N (N>1) group-common PDSCHs in a slot per CC 4. Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC 5. The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Note: The max number of (M+1), N, (K+L) are determined based on the numbers reported by FG5-11 and/or FG5-11a and/or FG5-11b. 6. up to one broadcast PDSCH is supported in a slot. 7. For any two consecutive slots n and n+1, if there are more than 1 broadcast/multicast/unicast PDSCH in either slot, whether to require the minimum time separation between starting time of any two broadcast/multicast/unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz | 33-1 and/or 33-2, 5-11 and/or 5-11a and/or 5-11b | intraSlotTDM-UnicastGroupCommonPDSCH-r17 | FeatureSetDownlinkPerCC-v1730 | N/A | N/A | Candidate value for component 7: require the minimum time separation time {yes, no} | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs/G-CS-RNTIs | 33-3-2, at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} | fdm-CodebookForMux-UnicastMulticastHARQ-ACK-r17 | CA-ParametersNR-v1730 | N/A | N/A | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e or G-CS-RNTIs of FG 33-5-1h Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs/G-CS-RNTIs | 33-2a or 33-4 or 33-5-1a or 33-5-1f | mode2-TDM-CodebookForMux-UnicastMulticastHARQ-ACK-r17 | CA-ParametersNR-v1730 | N/A | N/A | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e or G-CS-RNTIs of FG 33-5-1h Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | 33-3-3b | mode1-ForType1-CodebookGeneration-r17 | CA-ParametersNR-v1730 | N/A | N/A | This FG is for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and for multicast with the same priority and different HARQ-ACK codebook types in a PUCCH or in a PUSCH | 33-2a or 33-4 or 33-5-1a or 33-5-1f | mux-HARQ-ACK-UnicastMulticast-r17 | CA-ParametersNR-v1730 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-2a | nack-OnlyFeedbackForMulticast-r17 | CA-ParametersNR-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-4a | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) Up to 4 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate PUCCH resource configurations from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. up to 4TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits | 33-4 | nack-OnlyFeedbackSpecificResourceForMulticast-r17 | CA-ParametersNR-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2 | 33-4 and 33-2f | nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast on PCell | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast 5. ACK/NACK-based HARQ-ACK feedback for SPS release associated with G-CS-RNTI | 33-2 | sps-Multicast-r17 | FeatureSetDownlink-v1720 | N/A | N/A | one G-CS-RNTI per UE is supported for multicast reception | Optional with capability signalling |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, and first PDSCH after SPS activation 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | ack-NACK-FeedbackForSPS-Multicast-r17 | CA-ParametersNR-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1a, 33-5-1i | ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission associated with CS-RNTI for SPS multicast on the cell same as multicast initial transmission | 33-5-1a | ptp-Retx-SPS-Multicast-r17 | CA-ParametersNR-v1720 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | maxDynamicSlotRepetitionForSPS-Multicast-r17 | BandNR | N/A | N/A | Candidate values for X is: {8, 16} For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2) Support of shared PUCCH resource configurations with unicast 3) One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-5-1 | nack-OnlyFeedbackForSPS-Multicast-r17 | CA-ParametersNR-v1740 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1i | nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | maxNumberG-CS-RNTI-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI | 33-5-1 | sps-MulticastDCI-Format4-2-r17 | BandNR | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Up to 2TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits | 33-5-1f | nack-OnlyFeedbackSpecificResourceForSPS-Multicast-r17 | CA-ParametersNR-v1730 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration on PCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32 | 33-5-1 | sps-MulticastMultiConfig-r17 | BandNR | N/A | N/A | Candidate value set for M is {1, 2, …, 8} For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
| 33. NR_MBS | 33-5-3 | One SPS group-common PDSCH configuration for multicast for Scell | 1. Support one SPS group-common PDSCH configuration for multicast for Scell. 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH for Scell. 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast 5. ACK/NACK-based HARQ-ACK feedback for SPS release associated with G-CS-RNTI | 33-5-1, 33-2h | sps-MulticastSCell-r17 | FeatureSetDownlinkPerCC-v1730 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-4 | Up to 8 SPS group-common PDSCH configurations per CFR for multicast for SCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast for Scell. 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast for Scell. 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32. | 33-5-3 | sps-MulticastSCellMultiConfig-r17 | FeatureSetDownlinkPerCC-v1730 | N/A | N/A | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2a, 33-2f | priorityIndicatorInDCI-Multicast-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority indicator field configured in DCI format 4_2 for multicast HARQ-ACK feedback of SPS multicast | 33-5-1a, 33-5-1i | priorityIndicatorInDCI-SPS-Multicast-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | twoHARQ-ACK-CodebookForUnicastAndMulticast-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands | Optional with capability signalling |
| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a or 33-4 | singlePUCCH-ConfigForMulticast-r17 | CA-ParametersNR-v1740 | N/A | N/A | Note: With 33-2a or 33-4 as prerequisite FG, this FG33-8-1 includes the case of ACK/NACK for multicast or NACK-only mode1 for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-8-2 | Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-ConfigurationList for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-8-1, 33-6-1 | multiPUCCH-ConfigForMulticast-r17 | CA-ParametersNR-v1730 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-3 | PUCCH resource configuration for multicast feedback for SPS GC-PDSCH | Support of a SPS-PUCCH-AN-List for multicast HARQ-ACK feedback of all multicast SPS configuration(s), separate from that of SPS unicast configurations | 33-5-1a | pucch-ConfigForSPS-Multicast-r17 | CA-ParametersNR-v1730 | N/A | N/A |  | Optional with capability signalling |
| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1, 12-2 | releaseSPS-MulticastWithCS-RNTI-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
| 33. NR_MBS | 33-10 | Support group-common PDSCH RE-level rate matching for multicast | 1) Support of SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Support of P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 4) Support of AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns | 2-33a, 33-2 | re-LevelRateMatchingForMulticast-r17 | BandNR | N/A | N/A | For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |

### ***** Next change *****

### 6.1.14	NR_DL1024QAM_FR1

Table 6.1.14-1: Layer-1 feature list for NR_DL1024QAM_FR1

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [20]. | pdsch-256QAM-FR1 | pdsch-1024QAM-FR1-r17 | BandNR | N/A | Applicable only to FR1 | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-2MIMO-FR1-r17. | Optional with capability signalling |
| 36. NR_DL1024QAM_FR1 | 36-1a | 1024QAM for PDSCH for FR1 with maximum 2 MIMO layers restriction | Support 1024QAM for PDSCH with maximum 2 MIMO layers for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [20]. | pdsch-256QAM-FR1 | pdsch-1024QAM-2MIMO-FR1-r17 | BandNR | N/A | Applicable only to FR1 | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-FR1-r17. | Optional with capability signalling |
| 36. NR_DL1024QAM_FR1 | 36-2 | scalingFactor for 1024QAM | Indicates the scaling factor to be applied to the band in the max data rate calculation for 1024-QAM as defined in 4.1.2 when support of 1024-QAM is signalled for the band | 36-1 or 36-1a | scalingFactor-1024QAM-FR1-r17 | FeatureSetDownlink-v1700 | N/A | Applicable only to FR1 | Candidate component values: {0.4, 0.75, 0.8, 1.0} If absent, the scaling factor 1 is applied to the band in the max data rate calculation for 1024-QAM. | Optional with capability signaling |

### ***** Next change *****

### 6.2.10	NR_NTN_solutions

Table 6.2.10-1: Layer-2 and Layer-3 feature list for NR_NTN_solutions

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_NTN_solutions-Core | 34-1 | Support of non Terrestrial Network | Indicates whether the UE supports NR NTN access. |  | nonTerrestrialNetwork-r17 | UE-NR-Capability-v1700 | No | No | If the UE indicates this capability the UE shall support the following NTN essential features, i.e., timer extension in MAC/RLC/PDCP layers and RACH adaptation to handle long RTT, acquiring NTN specific SIB and more than one TAC per PLMN broadcast in one cell. | Optional with capability signalling |
|  | 34-2 | Disabling HARQ feedback for downlink transmission | Indicates whether the UE supports disabled HARQ feedback for downlink transmission. | 34-1 | harq-FeedbackDisabled-r17 | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-3 | HARQ mode B for uplink transmission | Indicates whether the UE supports HARQ mode B and the corresponding LCR restrictions for uplink transmission | 34-1 | uplink-Harq-ModeB-r17 | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-4 | Location based CHO | Indicates whether the UE supports location based CHO | 34-1, and condHandover-r16 is set for NTN bands. | locationBasedCondHandover-r17 | BandNR | No | No | UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
|  | 34-5 | Time based CHO | Indicates whether the UE supports time based CHO | 34-1, and condHandover-r16 is set for NTN bands. | timeBasedCondHandover-r17 | BandNR | No | No | UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
|  | 34-6 | Event A4 based CHO | Indicates whether the UE supports Event A4 based CHO | 34-1, and condHandover-r16 is set for NTN bands. | eventA4BasedCondHandover-r17 | BandNR | No | No | UE shall set the capability value consistently for all FDD-FR1 NTN bands. | Optional with capability signalling |
|  | 34-7 | SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. | 34-1 | sr-TriggeredBy-TA-Report-r17 | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-8 | Supported NTN scenario(s) | Indicates whether the UE supports the NTN features in GSO scenario or NGSO scenario. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports the NTN features for both GSO and NGSO scenarios, and also supports mobility between GSO and NGSO scenarios. | 34-1 | ntn-ScenarioSupport-r17 | UE-NR-Capability-v1700 | No | No |  | Optional with capability signalling |
|  | 34-9 | Time-based measurement initiation | It's optional for UE to start neighbour cell measurements before the broadcast cell service stop-time |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 34-10 | Location-based measurement initiation | It's optional for UE to perform location-based measurement initiation for neighbour cells |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 34-11 | SMTC adjustment in idle/inactive | It's optional for UE to perform SMTC adjustment in RRC_IDLE/RRC_INACTIVE |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 34-12 | Reporting of service link propagation delay difference between serving cell and neighbour cell(s) | Indicates whether the UE supports the reporting of service link propagation delay difference between serving cell and neighbour cell(s). | 34-1 | serviceLinkPropDelayDiffReporting-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-13 | Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) | 34-4 | eventD1-MeasReportTrigger-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 34-14 | RRC_INACTIVE in NTN | Indicates whether the UE supports RRC_INACTIVE in NTN | 34-1 | inactiveStateNTN-r17 | NTN-Parameters-r17 | No | No |  | Conditional mandatory with capability signalling |
|  | 34-15 | RA-SDT in NTN | Indicates whether the UE supports transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state in NTN via Random Access procedure (i.e., RA-SDT) with 4-step RA type and if UE supports twoStepRACH-r16 for NTN, with 2-step RA type. | 34-1 | ra-SDT-NTN-r17 | NTN-Parameters-r17 | No | No |  | Optional with capability signalling |
|  | 34-16 | SRB-SDT in NTN | Indicates whether the UE supports the usage of signalling radio bearer SRB2 over RA-SDT or CG-SDT in NTN. | 34-1 and {34-15 or 30-2 in NTN bands} | srb-SDT-NTN -r17 | NTN-Parameters-r17 | No | No |  | Optional with capability signalling |
|  | 34-17 | TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | n/a | n/a | n/a | n/a |  | Conditional mandatory without capability signalling |
|  | 34-18 | Inter-Satellite measurements | Indicates whether the UE supports inter-satellite measurement as specified in TS 38.331 [2] | 34-1 | interSatMeas-r17 | MeasAndMobParametersCommon | No | No |  | Conditional mandatory for UEs supporting nonTerrestrialNetwork-r17 |

### ***** Next change *****

### 6.2.12	NR_RedCap

Table 6.2.12-1: Layer-2 and Layer-3 feature list for NR_RedCap

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_redcap-Core | 36-1 | Support 16 DRBs | Defines whether the RedCap UE supports 16 DRBs as specified in TS 38.331 [2]. |  | supportOf16DRB-RedCap-r17 | RedCapParameters-r17 | No | No |  | Optional with capability signalling |
|  | 36-2 | Support 18 bit length of PDCP sequence number | Defines whether the RedCap UE supports 18 bit length of PDCP sequence number as specified in TS 38.331 [2]. |  | longSN-RedCap-r17 | PDCP-Parameters | No | No |  | Optional with capability signalling |
|  | 36-3 | Support AM DRB with 18 bit length of RLC sequence number | Defines whether the RedCap UE supports AM DRB with 18 bit length of RLC sequence number as specified in TS 38.331 [2]. |  | am-WithLongSN-RedCap-r17 | RLC-Parameters | No | No |  | Optional with capability signalling |
|  | 36-4 | Support of RRM relaxation in RRC_CONNECTED | Defines whether the RedCap UE supports Rel-17 relaxed RRM measurements in RRC_CONNECTED as specified in TS 38.331 [2]. |  | rrm-RelaxationRRC-ConnectedRedCap-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 36-5 | Support of extended DRX in RRC_INACTIVE | Defines whether a UE (RedCap or Non-RedCap) supports the extended DRX in RRC_INACTIVE with values of 256, 512 and 1024 radio frames as specified in TS 38.331 [2]. The UE may indicate support for extended DRX in RRC_INACTIVE only if it supports extended DRX in RRC_IDLE. |  | extendedDRX-CycleInactive-r17 | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 36-6 | Support of RedCap-specific initial DL BWP associated with NCD-SSB for SDT | Indicates that the UE supports using RedCap-specific initial DL BWP associated with NCD-SSB for SDT. | UE supporting this feature shall indicate support of supportOfRedCap-r17 and ra-SDT-r17 and/or cg-SDT-r17. | ncd-SSB-forRedCapInitialBWP-SDT-r17 | RedCapParameters-v1740 | No | No |  | Optional with capability signalling |

### ***** Next change *****

### 6.2.19	TEI17 and Others

Table 6.2.19-1: Layer-2 and Layer-3 feature list for TEI17 and Others

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. TEI17/Others | 43-1 | Minimization of service interruption | It is optional for UE to support minimization of service interruption including reporting to NAS of disaster roaming information for available PLMNs and Access Barring check for Access Identity 3, as specified in TS 38.331 [2]. |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 43-2 | Use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state | Indicates whether the UE supports to use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state. |  | inactiveStatePO-Determination-r17 | UE-NR-Capability-v1700 | No | No |  | Optional with capability signalling |
|  | 43-3 | Cell reselection priority handling for NR HSDN | It is optional for UE to support HSDN cell reselection priority handling in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [19] and TS 38.331 [2]. |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 43-4 | Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [2]. |  | n/a | n/a | n/a | n/a |  | Conditional mandatory without capability signalling |
|  | 43-5 | CHO with target SCG for NR-DC | Indicates whether the UE supports conditional handover with NR SCG configuration for NR-DC. | 17-3 and at least one NR-DC band combination | condHandoverWithSCG-NRDC-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 43-6 | CHO with target SCG for EN-DC | Indicates whether the UE supports conditional handover with NR SCG configuration for EN-DC. | cho-r16 in TS 36.306 [14] and at least one EN-DC band combination. | condHandoverWithSCG-ENDC-r17 | MeasAndMobParametersMRDC-Common-v1700 | No | No |  | Optional with capability signalling |
|  | 43-7 | CHO with target SCG for NE-DC | Indicates whether the UE supports conditional handover with E-UTRA SCG configuration for NE-DC. | 17-3 and at least one NE-DC band combination. | condHandoverWithSCG-NEDC-r17 | MeasAndMobParametersMRDC-Common-v1700 | No | No |  | Optional with capability signalling |
|  | 43-8 | Flexible gNB ID length for NR-DC | Indicates whether the UE supports acquisition and reporting of gNB ID length from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired gNB ID length to the network as specified in TS 38.331 [2] when (NG)EN-DC and NE-DC are not configured or, when consistent DRX is configured in NR-DC. The consistent DRX configuration implies that MN and SN have the same DRX cycle and on-duration configured by MN completely contains on-duration configured by SN. It is mandated if UE supports NR CGI reporting (NG)EN-DC and NE-DC are not configured or, when consistent DRX is configured in NR-DC. |  | gNB-ID-Length-Reporting-r17 | MeasAndMobParametersCommon | No | No |  | Conditional mandatory with capability signalling |
|  | 43-9 | Flexible gNB ID length for (NG)EN-DC | Indicates whether the UE supports acquisition and reporting of gNB ID length from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired gNB ID length to the network as specified in TS 38.331 [2] when the (NG)EN-DC is configured. It is mandated if UE supports NR CGI reporting when (NG)EN-DC is configured. |  | gNB-ID-Length-Reporting-ENDC-r17 | MeasAndMobParametersCommon | No | No |  | Conditional mandatory with capability signalling |
|  | 43-10 | Flexible gNB ID length for NE-DC | Indicates whether the UE supports acquisition and reporting of gNB ID length from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired gNB ID length to the network as specified in TS 38.331 [2] when the NE-DC is configured. It is mandated if UE supports NR CGI reporting when NE-DC is configured. |  | gNB-ID-Length-Reporting-NEDC-r17 | MeasAndMobParametersCommon | No | No |  | Conditional mandatory with capability signalling |
|  | 43-11 | Flexible gNB ID length | Indicates whether the UE supports acquisition and reporting of gNB ID length from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired gNB ID length to the network as specified in TS 38.331 [2] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. It is mandated if UE supports NR CGI reporting when NR-DC is configured. |  | gNB-ID-Length-Reporting-NRDC-r17 | MeasAndMobParametersCommon | No | No |  | Conditional mandatory with capability signalling |
|  | 43-12 | Flexible gNB ID length | Indicates whether the UE supports acquisition of NPN-relevant gNB ID length from a neighbouring intra-frequency or inter-frequency NR NPN cell by reading the SI of the neighbouring cell and reporting the acquired gNB ID length to the network as specified in TS 38.331 [2]. It is mandated if UE supports NPN CGI reporting. |  | gNB-ID-Length-Reporting-NPN-r17 | MeasAndMobParametersCommon | No | No |  | Conditional mandatory with capability signalling |
|  | 43-13 | CG Time Domain Allocation extension | Indicates whether UE supports the timeDomainAllocation-v1700 configured in rrc-ConfiguredUplinkGrant to indicate more than 16 entries in PUSCH TDRA table. | This field is only applicable if the UE supports both pusch-RepetitionTypeB-r16 and either configuredUL-GrantType1 or configuredUL-GrantType1-v1650. | cg-TimeDomainAllocationExtension-r17 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 43-14 | n77 Canada | This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 in Canada as specified in Note 12 of Table 5.2-1 in TS 38.101. If absent, the UE supports only restriction to the 3450 - 3650 MHz range of band n77 in Canada. A UE that indicates this field shall also support NS value 57 as specified in TS 38.101-1. A UE supporting NS value 57 shall indicate this field. |  | extendedBand-n77-2-r17 | RF-Parameters | No | No |  | Optional with capability signalling |
|  | 43-15 | DRX HARQ RTT UL timer with bundling | Indicates whether the UE supports starting the drx-HARQ-RTT-TimerUL after the end of the last transmission within a bundle as specified in TS 38.321 [10]. |  | lastTransmissionUL-r17 | MAC-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 43-16 | Idle/Inactive measurement for voice fallback | It is optional for UE to support the idle/inactive measurement for EPS fallback in RRC_IDLE/RRC_INACTIVE as specified in TS 38.331 [2]. |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 43-17 | Selection of acceptable E-UTRA cell upon HO failure during EPS fallback for emergency call | It is optional for UE to support selecting an acceptable E-UTRA cell supporting emergency call if no suitable E-UTRA cell is available upon handover failure during EPS fallback when the UE has an ongoing emergency call as specified in TS 38.331 [2]. |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 43-18 | E-UTRA cell selection upon HO failure during EPS services fallback | It is optional for UE to support selecting a suitable E-UTRA cell, and support selecting an acceptable E-UTRA cell supporting emergency call if no suitable E-UTRA cell is available upon handover failure when the UE is performing emergency services fallback as specified in TS 38.331 [2]. |  | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 43-19 | Higher granularity for per-FR gap capability | This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 as specified in clause 9.1.2 of TS 38.133 while the number of configured serving cells is less than or equal to the indicated number. UE indicating support of this feature shall not indicate support of independentGapConfig. |  | independentGapConfig-maxCC-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 43-20 | Support of the value 'n3' for repetitionFactor-r17 | Indicates the support of the value "n3" for repetitionFactor-r17. | The UE indicating support of this feature shall also indicate support of srs-increasedRepetition-r17. | srs-AdditionalRepetition-r17 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 43-21 | Support to derive SSB Index from cell centre | Indicates whether the UE supports configuration of deriveSSB-IndexFromCellInter-r17 in MeasObjectNR. |  | deriveSSB-IndexFromCellInterNon-NCSG-r17 | MeasAndMobParametersCommon | No | No | This field applies only to non-NCSG capable UEs (i.e. UEs not supporting ncsg-MeasGapNR-Patterns-r17 | Optional with capability signalling |

### ***** Next change *****

### 6.3.6	NR_MG_enh

Table 6.3.6-1: RF and RRM Feature List for NR_MG_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19. Network controlled small gap (NCSG) | 19-1a | Reporting of Network controlled small gap (NCSG) for NR | Reporting of the NCSG and measurement gap requirement information for SSB based measurement |  | nr-NeedForGapNCSG-Reporting-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 19-1b | Reporting of Network controlled small gap (NCSG) for EUTRA | Reporting of the NCSG and measurement gap requirement information for E-UTRA target bands |  | eutra-NeedForGapNCSG-Reporting-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 19-1-1 | per FR Network controlled small gap | Support of per-FR NCSG | 19-1a | ncsg-MeasGapPerFR-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 19-1-2 | Network controlled small gap (NCSG) supported patterns | Supported NCSG patterns | 19-1a or 19-1b | ncsg-MeasGapPatterns-r17 | MeasAndMobParametersCommon | No | No | NCSG patterns #0, #1 are conditional mandatory if the UE is NCSG capable NCSG patterns #13, #14 are conditional mandatory if UE supports 19-1-1 or if the UE is NCSG capable and supports FR2 band in standalone mode. UEs supporting this shall indicate support of 19-1a or 19-1b | Optional with capability signalling |
|  | 19-1-3 | Network controlled small gap (NCSG) supported NR-only patterns | Supported NR-only NCSG patterns | 19-1a | ncsg-MeasGapNR-Patterns-r17 | MeasAndMobParametersCommon | No | No | NCSG patterns #2 and #3 are conditional mandatory if the UE is NCSG capable NCSG patterns #17 and #18 are conditional mandatory if the UE is NCSG capable and supports a FR2 band | Optional with capability signalling |
|  | 19-1-4 | Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter | Support performing measurement with NCSG based on flagderiveSSB-IndexFromCell-inter and meeting the following requirements: - Scheduling restriction in FR2 serving cell during NCSG ML is on SSB symbol level | 19-1a | ncsg-SymbolLevelScheduleRestrictionInter-r17 | MeasAndMobParametersCommon | No | FR2 only |  | Optional with capability signalling |
| 19. Concurrent measurement gap | 19-2 | Concurrent measurement gaps | Support of more than 1 per-UE measurement gap configurations Support of more than 1 per-FR gap measurement gap configurations in an FR, or simultaneous 1 per-UE measurement gap plus 1 per-FR measurement gap configurations in an FR, or more than 1 per-UE measurement gap configurations for UE capable of Rel-15 per-FR gap (independentGapConfig) Note: The above 2 bullets are not 2 separate indications but a single indication with different interpretations, depending on the support of independentGapConfig. |  | concurrentMeasGap-r17 CHOICE { concurrentPerUE-OnlyMeasGap-r17, concurrentPerUE-PerFRCombMeasGap-r17 } | MeasAndMobParametersCommon | No | No | This is the baseline capability is to indicate UE support multiple concurrent gaps. | Optional with capability signalling |
| 19. Concurrent measurement gap | 19-2-1 | Concurrent measurement gaps for E-UTRAN measurement objectives | Capability of supporting configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps | 19-2 | concurrentMeasGapEUTRA-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
| 19. Pre-configured gap | 19-3-1 | Pre-configured measurement gap with network-controlled activation and deactivation mechanism | Capability of supporting preconfigured measurement gap with network-controlled mechanism for activation and deactivation |  | preconfiguredNW-ControlledMeasGap-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
| 19. Pre-configured gap | 19-3-2 | Pre-configured measurement gap with UE autonomous activation and deactivation mechanism | Capability of supporting preconfigured measurement gap with UE autonomous mechanism for activation and deactivation |  | preconfiguredUE-AutonomousMeasGap-r17 | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |

### ***** Next change *****

### 6.3.12	NR_NTN_Solutions

Table 6.3.12-1: RF and RRM Feature List for NR_NTN_Solutions

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-1 | Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier |  | parallelSMTC-r17 | MeasAndMobParametersCommon | FDD only | FR1 only | UE is mandatory to support 2 and can optionally support 4 if the feature is supported | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-2 | Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-17). |  | parallelMeasurementWithoutRestriction-r17 | BandNR | FDD only | FR1 only | For UEs not able to perform measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-3 | Parallel measurements with multiple measurement gaps | Support of 2 measurement gaps |  | parallelMeasurementGap-r17 | MeasAndMobParametersCommon | FDD only | FR1 only | UE is mandatory to support 1 measurement gaps | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-4 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | If UE does not support the capability, other NTN measurement requirements (as specified in TS 38.133, section 4.2C.2) are applied for both LEO and GEO. |  | n/a | n/a | FDD only | FR1 only |  | Optional without capability signaling |
| 25. NR_NTN_solutions | 25-5 | Parallel measurements on multiple NGSO satellites within a SMTC | Support of simultaneously measurements on target cells belonging to different NGSO satellites within a SMTC |  | maxNumber-NGSO-SatellitesWithinOneSMTC-r17 | BandNR | FDD only | FR1 only | Candidate values for the number of NGSO satellites are 1,2,3, or 4 |  |
| 25. NR_NTN_solutions | 25-6 | Relaxed cell reselection on GEO | Support of relaxed cell reselection on GEO |  | n/a | n/a | FDD only | FR1 only | Only applicable for GEO | Optional without capability signaling |
| 25. NR_NTN_solutions | 25-7 | The number of target LEO satellites the UE can monitor per carrier including serving satellite | On serving carrier, it indicates the number of target LEO satellites the UE can monitor per carrier including serving satellite On non-serving carrier, it indicates the number of target LEO satellites the UE can monitor per carrier. |  | maxNumber-LEO-SatellitesPerCarrier-r17 | BandNR | FDD only | FR1 only | Candidate values for the number of NGSO satellites are 2,3, or 4. The value shall be larger than or equal to the reported value on FG 25-5. | Optional with capability signaling |