3GPP TSG-RAN WG2 Meeting #122	R2-2305004

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.19.1

Source:	Huawei, HiSilicon

Title:	How to capture “eRedCap UE” in the running CRs/Rel-18 specifications

Document for: Discussion

Introduction

In this contribution, we discuss how to define and capture the capability of eRedCap UEs in the Rel-18 specification.

Discussion

Due to these shared capabilities between RedCap UEs and eRedCap UEs, most of the behaviors of RedCap UEs can be applicable to the eRedCap UEs. On the other hand, the eRedCap UEs would have specific behaviors owning to the different capabilities from RedCap Ues (for example, the separate early identification of the eRedCap UEs which is already included in the WID). Based on this, RAN2 needs to discuss how to distinguish the behaviors of RedCap UEs and eRedCap Ues in the specifications.

One potential solution is to only clarify the different behaviors between the RedCap UEs and the eRedCap UEs in the specifications. In this manner, if one behavior of RedCap UEs is also applicable to the eRedCap UEs, then there is no any change and it is automatically applicable to the eRedCap UEs. If the behavior is only applicable to RedCap UEs but not eRedCap UEs, it can be clarified as “ RedCap UE other than eRedCap UE”. However, as mentioned in the WID, the eRedCap UE is a new UE type other than RedCap UE, so it is better to have a clear distinction in the description in the specifications. This way may have ambiguity between the two UE types.

Observation 1: RedCap UE and eRedCap UE are two types of UEs. eRedCap UE is NOT one sub-type of RedCap UE.

Observation 2: To avoid any possible ambiguity in Rel-18, like the term “initial BWP” in Rel-17 is not always clear on whether it also includes the “RedCap-specific initial BWP”, “RedCap UE” in the specification should NOT cover the “eRedCap UE” by default.

Another potential solution is to explicitly update all the current descriptions of the eRedCap UEs. If one behavior of RedCap UEs is also applicable to the eRedCap UEs as same description, then it should be clarified/updated to be “ (e)RedCap UE”. And if the behavior is only applicable to RedCap UEs, then there is no any change.

Hence, we propose:

Proposal 1a: In the R18 specification descriptions, the R17 legacy texts for RedCap UEs descriptions are NOT inherited/applied by default to the eRedCap UEs, i.e. we use following terminologies:

- “(e)RedCap UE” to describe the same behaviors for both RedCap and eRedCap UEs;

- “RedCap UE” to describe the RedCap UE only/specific behaviors;

- “eRedCap UE” to describe the eRedCap UE only/specific new behaviors.

It is also important to keep the same style among different WGs when to capture the R18 eRedCap feature in the specifications, to avoid any misunderstanding. Therefore, it is proposed to send LS to other WGs about the RAN2 agreements:

Proposal 1b: Inform other WGs about RAN2 agreement on how to use (e)RedCap UE terminologies in the Rel-18 specifications.

In Rel-17, the definition of RedCap UE is captured in the current TS 38.306, clause 4.2.21.1.

4.2.21.1 Definition of RedCap UE RedCap UE is the UE with reduced capability: - The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs; - The maximum mandatory supported DRB number is 8; - The mandatory supported PDCP SN length is 12 bits while 18 bits being optional; - The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional; - For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs; - CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

Based on the RAN#99 meeting conclusion below, the Rel-18 eRedCap UE may include two detailed cases: Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1.

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Compared to the Rel-17 RedCap UEs, the further reduced capability of the eRedCap UE includes lower peak data rate and optional narrower baseband bandwidth. The supported peak data for Rel-18 eRedCap targets to 10Mbps. The maximum baseband bandwidth for RedCap UEs is 20MHz for FR1, but only 5MHz for PDSCH/PUSCH for eRedCap UEs. Based on these differences, it is reasonable to introduce a new clause to define the eRedCap UE clearly.

In other impacts of UE capability, e.g. the number of the Rx branches and the maximum mandatory supported DRB number, the capability of the eRedCap UEs is quite identical to the capability of the RedCap UEs so far. Therefore, the capability parameters of RedCap UEs except the supportOfRedCap-r17 can be reused for eRedCap UEs, i.e., supportOf16DRB-RedCap-r17, longSN-RedCap-r17, am-WithLongSN-RedCap-r17 and rrm-RelaxationRRC-ConnectedRedCap-r17.

Proposal 2: RAN2 captures the eRedCap UE definition by adding a new sub-clause “4.2.21.1a Definition of eRedCap UE” under the “4.2.21 RedCap Parameters” in TS 38.306, which:

- copies the existing sentences which are same as RedCap UEs;

- adds/clarifies the eRedCap UE specific capabilities (e.g. about peak data rate and BB bandwidth, detailed capabilities are FFS).

Conclusion

Based on the above analyses, we propose:

Observation 1: RedCap UE and eRedCap UE are two types of UEs. eRedCap UE is NOT one sub-type of RedCap UE.

Observation 2: To avoid any possible ambiguity in Rel-18, like the term “initial BWP” in Rel-17 is not always clear on whether it also includes the “RedCap-specific initial BWP”, “RedCap UE” in the specification should NOT cover the “eRedCap UE” by default.

Proposal 1a: In the R18 specification descriptions, the R17 legacy texts for RedCap UEs descriptions are NOT inherited/applied by default to the eRedCap UEs, i.e. we use following terminologies:

- “(e)RedCap UE” to describe the same behaviors for both RedCap and eRedCap UEs;

- “RedCap UE” to describe the RedCap UE only/specific behaviors;

- “eRedCap UE” to describe the eRedCap UE only/specific new behaviors.

Proposal 1b: Inform other WGs about RAN2 agreement on how to use (e)RedCap UE terminologies in the Rel-18 specifications.

Proposal 2: RAN2 captures the eRedCap UE definition by adding a new sub-clause “4.2.21.1a Definition of eRedCap UE” under the “4.2.21 RedCap Parameters” in TS 38.306, which:

- copies the existing sentences which are same as RedCap UEs;

- adds/clarifies the eRedCap UE specific capabilities (e.g. about peak data rate and BB bandwidth, detailed capabilities are FFS).