3GPP TSG-RAN WG2 Meeting #122	R2-2305072

Incheon, Korea, 22– 26 May 2023

Agenda item:	7.5.3

Source:	Apple

Title:	C-DRX enhancements for XR

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

According to the revised WID for NR XR [1], RAN2 should address the following objectives to specify the enhancements for XR-specific power saving:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

During RAN2 #121bis-e, we have discussed three main directions, namely:

- Non-Integer DRX Cycle

- Integer DRX cycle with non-uniform OnDuration periodicity

- Multiple active DRX configurations

It was concluded that the option based on Multiple active DRX configurations is ruled out, mainly due to concerns on the UE complexity of managing multiple DRX timers simultaneously:

RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration.

Hence, RAN2 will further examine the remaining two options, including Non-Integer DRX Cycle and Integer DRX cycle with non-uniform OnDuration periodicity. This paper provides some of our views on these topics.

# Discussions

To continue the discussions on the issues of non-integer periodicity of XR traffics, RAN2 needs to select between (1) non-integer DRX cycle, and (2) integer DRX cycle with non-uniform OnDuration. From our point of view, both of these approaches would work, so we think RAN2 should prioritize the approach with lower implementation complexity, such as the need of additional RRC parameters and MAC operations.

In all options, it seems additional RRC configuration parameters cannot be avoided. For instance, to enable non-integer DRX cycles that can accommodate XR traffic periodicities such as 16.67ms, the UE have to be configured with non-integer DRX cycle values or cadence values that are currently not available in TS 38.331. Similarly, for the approach based on integer DRX cycle with non-uniform OnDuration, the UE has to be pre-configured new parameters such as additional offset values (in DRX configuration) or non-uniform DRX cycle patterns. Hence, there is no much difference between these two approaches in terms of RRC impacts.

On the other hand, if we consider from the angle of MAC implementation, then it becomes obvious that the DRX formula becomes more complicated if the approach based on non-integer DRX cycle is adopted. Some concerns have been raised regarding the rounding errors caused by floor and modulo operations on rational numbers required in formula for non-integer DRX cycle, and the proponents of non-integer DRX cycle have argued that rounding errors can be avoided when specific method is used for the floor and modulo operations on rational numbers. Nevertheless, it also implies some restrictions on the implementation choices of such DRX formulas, and we think we should try to minimize such restrictions for implementations in our specification work. On the other hand, some solutions for non-integer DRX cycle require the UE to use the configured “cadence” value in the DRX formula, this is also undesirable due to the departure from the legacy method. We also have some concerns about its operations in TDD, as the modulo operation may cause the slot to land on an UL slot, wherein the UE cannot monitor PDCCH. Alternatively, the ON-duration may start somewhere close to the UL slots and the UE may not have sufficient DL symbols for PDCCH monitoring.

Conversely, we think the approach based on integer DRX cycle with non-uniform OnDuration seems to be simpler. In particular, if the UE can simply follow a non-uniform DRX cycle pattern (e.g. a pattern of DRX cycle can be {17ms, 17ms, 16ms}) explicitly provided by the network, the implementation complexity can be much lower. Moreover, although some updates of the DRX formula are still needed, we think the complexity is more acceptable than the approach based on non-integer DRX cycle.

Based on these considerations, we think RAN2 should prioritize the approach based on Integer DRX cycle with non-uniform OnDuration periodicity.

Proposal 1: To handle XR frame rate with non-integer periodicity, RAN2 should prioritize the approach based on integer DRX cycle with non-uniform OnDuration periodicity.

# Conclusions

This contribution presents some of our views on C-DRX enhancements to handle XR traffics with non-integer periodicity. The following is proposed:

Proposal 1: To handle XR frame rate with non-integer periodicity, RAN2 should prioritize the approach based on integer DRX cycle with non-uniform OnDuration periodicity.

# References

- RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm, 3GPP TSG RAN #99, March 2023, Rotterdam, Netherlands.