3GPP TSG-RAN WG2 Meeting #122	R2-2305079

Incheon, South Korea, 22 May – 26 May 2023

Agenda item:	7.23.2

Source:	Apple

Title:	RAN feedback for burst sending time adjustment

WID/SID:	NR_TRS_URLLC

Document for:	Discussion / Decision

# Introduction

SA2 has studied architectural enhancements for supporting 5G Timing Resiliency and URLLC in the 5G system. To address the RAN impacts out of this work, RAN has approved a new R18 WI in the last RAN plenary at RAN#99.

One of the objectives is about RAN enhancements to adapt downstream and upstream scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication, among with Key Issue #6 in [2]. Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication. Note 3: Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

This paper provides a view on reactive RAN feedback for upstream scheduling from a UE perspective.

# Discussion

Use cases related to the NR_TRS_URLLC work item target low E2E latency communication (~2ms). One of the aspects SA2 has not concluded on is about the UE/RAN impacts of reactive RAN feedback, as described in [2][3]. With regards to scheduling adaptations envisioned in the SA2 method, from our perspective it would be beneficial if the network and not the application (i.e., the place where the data is produced) can adapt to the packet arrival pattern. Otherwise, if the solution results in applications withholding available data (artificially), it may result in suboptimal user experience.

In other words, if the provisioning of an offset happens as part of QoS flow establishment then it should be fine for the application to accordingly setup the offset right from the beginning of the connection. BAT adaptation in the middle of an ongoing connection may lead to a situation where data is withheld at the application. Assuming the E2E latency is already very short, then suboptimal operation may happen.

Thus, even though an adaptation mechanism is technically feasible, we are not sure if the application layer should indeed adapt timing on its own. Rather, the RAN may forward the offset to the SMF (which is part of the SA2 solution) and in the end, there could be a QoS flow modification request where the timing is explicitly adjusted. Such a method may create additional signalling load depending on how often such adjustments are required. The SA2 solution does not mention this case though.

Proposal 1: RAN2 should discuss the preferred method of RAN feedback operation, including whether the gNB scheduler (rather than the application) can apply the BAT offset based on feedback from the UE.

Considering the actual BAT offset derivation, we think it is feasible for the UE to measure the offset and provide it to the gNB in a report. A mechanism to do this can be defined (e.g., the offset could be determined based on reference time) and it should be possible for the UE to assist the gNB in a feedback message.

Other than a definition of, e.g., a reference time to derive the BAT offset and a ‘reference location’ for the data arrival (e.g., at the DS-TT, PDCP or MAC), details as to how the UE calculates the offset do not need to be specified and can be left to UE implementation. The UE could report the offset as assistance information to the gNB. The exact parameter set can be discussed once more details are available from the Rel-18 XR work item.

We further note that there are some differences between XR and IIoT/URLLC when it comes to BAT. While XR traffic has a quasi-periodic characteristic with burst size fluctuating, IIoT/URLLC traffic is very deterministic. Therefore, very frequent BAT updates are not expected. In other words, signalling overhead may not be that big.

Observation 1: While XR traffic has a quasi-periodic characteristic with burst size fluctuating, IIoT/URLLC traffic is very deterministic.

Regarding the actual method to signal the BAT, naturally a MAC CE provides for faster adjustments compared to RRC signalling, and this may suit the lower latency requirements of the IIoT/URLLC traffic. On the other hand, even RRC may be workable depending on the scenario, for example, if it’s only to indicate burst arrival time adaptation in exceptional cases for QoS notification. The latter is in line with RAN2’s earlier Reply LS to SA2 on the UL scenario of reactive RAN feedback in [4].

# Conclusions

This contribution provides initial views on reactive RAN feedback for burst arrival time adaptation. We have following observations and proposals:

Proposal 1: RAN2 should discuss the preferred method of RAN feedback operation, including whether the gNB scheduler (rather than the application) can apply the BAT offset based on feedback from the UE.

Observation 1: While XR traffic has a quasi-periodic characteristic with burst size fluctuating, IIoT/URLLC traffic is very deterministic.

# References

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements, Nokia, RAN#99

- TR 23.700-25, v18.1.0, Study on timing resiliency and TSC and URLLC enhancements, Rel-18

- S2-2301419, TR 23.700 KI#6 conclusion update, CR 0003, Nokia, Samsung

- R2-2213070, Reply LS on UL scenario of reactive RAN feedback for burst sending time adjustment, RAN2 to SA2