3GPP TSG-RAN WG2 Meeting #122                                                             R2-2305117

Incheon, Korea, May 22 – 26, 2023

Agenda item: 		7.4.2.2

Source: 		Qualcomm Incorporated

Title: 	RRC aspects of LTM

Document for:	Discussion

# 1	Introduction

In RAN2#121bis-e, the following agreements have been achieved [1]:

- Whether the Reference configuration is a complete configuration or not is up to the network implementation. - Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. - The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current). - Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch. - The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed. - The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS. - RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration). - RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). - RAN2 assumes that whether filtering, hysteresis, and time-to-trigger are needed for LTM specific L1 measurements is up to RAN1. - FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status. - Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

In RAN2#119bis-e the following agreements were achieved [2]:

RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between. RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release). L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

In RAN2#119-e the following agreements were achieved [3]:

Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS) R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases: - the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change)

In RAN1#112bis-e the following agreements were achieved [4]:

For the beam selection for SSB based L1-RSRP measurement report, - Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells - FFS: How to select the L cells and M beams per cells is up to UE - M x L beams are reported in a single report instance - Max values of M and L are based on UE capability, and at least M x L=4 is supported as a UE capability, other UE capabilities are FFS - FFS if UE is allowed to report less than M x L beams - The values of M and L are configured to the UE in the reporting configuration - FFS: The following configurability is introduced in the report configuration - 1) Whether serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 - 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency Periodic and semi-persistent report on PUCCH are also supported for gNB scheduled L1-measurement reporting

This contribution discusses subsequent LTM, delta configuration for LTM, decoding and validity check of LTM candidate configurations, and configuration of measurement and reporting of candidate LTM cells.

# 2	Discussion on subsequent LTM

The baseline for L3 mobility is that the UE releases the source configuration per HO execution. Thus, a new RRC reconfiguration (the HO command) is required per HO execution.

For LTM, RAN2#119bis-e agreed that subsequent LTM is supported, where the UE performs a sequence of LTM execution with no need for RRC reconfiguration per LTM execution.

A question arises on whether subsequent LTM should be assumed the baseline mode of operation for the UE:

  - Option 1: Subsequent LTM is the baseline/default mode of operation:

    - The network does not have to explicitly configure the UE with subsequent-LTM mode of operation.

    - The UE does not release the candidate cell configurations at LTM execution by default.

    - If the network wants to cancel LTM after the first LTM execution, the network explicitly tells the UE to release the candidate cell configurations via a new RRC message following the execution.

  - Option 2: Subsequent LTM is NOT the baseline/default mode of operation:

    - The network explicitly configures the UE with subsequent-LTM mode of operation.

    - The UE determines whether to release the candidate cell configurations at LTM execution based on whether subsequent LTM is configured by the network.

    - If the network wants to cancel LTM after the first LTM execution, the network does not configure subsequent LTM mode of operation during LTM preparation.

To select between Option 1 and Option 2, we note that subsequent LTM maximizes the benefits of LTM compared to L3 mobility:

- Minimize backhaul overhead: subsequent LTM allows to have one preparation phase for a sequence of cell switches for the UE.

- Minimize Uu overhead: subsequent LTM allows to perform a sequence of cell switches with a single RRC message.

- Improve robustness: since LTM preparation is driven by L3 measurements and LTM execution is driven by L1 measurements, cell switches become faster with subsequent LTM since LTM preparation which relies on L3 measurements is skipped.

Observation 1a: Subsequent LTM minimizes the overhead over the backhaul for preparation of candidate cells.

Observation 1b: Subsequent LTM minimizes the overhead over Uu by performing a sequence of cell switches with a single RRC message.

Observation 1c: Subsequent LTM improves robustness of UE mobility since cell switches rely on swift L1 measurement reporting and skip interleaved LTM preparations that require slow L3 measurement reporting.

Given the merits of subsequent LTM, this should be the default mode of operation of the UE if LTM is configured. If the network wants to stop subsequent LTM, explicit release of the candidate configurations is required by the network, which as aligned with the working assumption in RAN2#119bis-e. We support Option 1.

Proposal 1a: Subsequent LTM is baseline mode of operation of the UE. Subsequent LTM is not explicitly configured.

Proposal 1b: To stop subsequent LTM, the network explicitly releases the candidate configurations via an RRC message.

The support of subsequent LTM should encompass the scenario where the UE ping-pongs between the UE’s serving cell group when LTM configuration is received by the UE and the other cells indicated as candidates in the LTM configuration. Therefore, the UE’s serving cell group when LTM configuration is received shall be assumed a candidate cell group for LTM by default.

Proposal 1c: The configuration of the UE’s current serving cell group when LTM is configured is by default a candidate LTM configuration following the first LTM execution.

# 3	Discussion on delta configuration

RAN2#121bis-e agreed that the sum of the reference configuration and an LTM candidate configuration is a complete configuration. This implies that the LTM candidate configuration is a complete configuration for scenarios where the reference configuration is an empty configuration. However, it is also possible that the reference configuration may be non-empty, but it is only used for delta signaling for a subset of the LTM candidate configurations. This implies the scenario where the reference configuration is non-empty but a subset of the LTM candidate configurations are complete should be supported.

Proposal 2: The UE may receive a complete LTM candidate configuration and a non-empty reference configuration, where the LTM candidate configuration does not use delta signaling on top of the reference configuration.

Figure 1: Efficient delta signaling in PCell/SCell-swap LTM scenario

RAN2#119bis-e agreed that a candidate LTM PCell may be a current serving SCell, and a candidate LTM SCell may be a current serving PCell. This implies that a PCell of one candidate LTM cell group be the same as an SCell of another candidate LTM cell group.

Observation 2a: A PCell of one candidate LTM cell group and an SCell of another candidate LTM cell group may be the same.

Figure 1 illustrates the PCell/SCell-swap LTM scenario where the UE switches b/w two candidate cell groups:

- Candidate CG 1 that comprises Cell1 as a PCell and Cell2 as an SCell

- Candidate CG 2 that comprises Cell2 as a PCell and Cell1 as an SCell

In this scenario, using a common reference cell configuration for delta signaling of the PCell config of candidate CG1 and the SCell config of candidate CG2 is the most efficient. Similarly, using a common reference cell configuration for delta signaling of the SCell config of candidate CG1 and the PCell config of candidate CG2 is the most efficient.

Observation 2b: If the same cell is a PCell in one candidate LTM cell group and an SCell in another candidate LTM cell group, using a common reference cell configuration to configure delta signaling for this cell is the most efficient.

Consequently, configuring a PCell with respect to a reference SCell configuration and configuring an SCell with respect to a reference SpCell configuration should be supported.

Proposal 3a: An SpCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

Proposal 3b: An SCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

# 4	Discussion on decoding and validity-check of LTM candidate configurations

RAN2#121bis-e agreed that the UE “may” perform early decoding and early validity check of the LTM candidate configurations. It is observed that this will be wasted effort if LTM execution is never triggered towards an LTM candidate for which the associated configuration had this early processing.

Observation 3: Decoding and validation of a configuration of an LTM candidate cell is wasteful if LTM is never triggered towards that cell.

This implies whether to do early decoding and validity check of an LTM candidate configuration should be the UE’s choice.

Proposal 4: The UE is not expected to receive a request from the network to perform early decoding and early validity check of an LTM candidate configuration.

Assuming the UE decodes an LTM candidate configuration and compliance check fails, it is not useful if the UE simply reports an error flag. On the other hand, providing a detailed report on which IEs in the LTM candidate configuration cause the compliance check failure may be an overwhelming task. Furthermore, the spec effort to determine how the UE identifies and reports compliance failure to portions of an RRC Reconfiguration message will be major.

Observation 4a: Simple error flagging of an LTM candidate configuration is not useful.

Observation 4b: The UE may be overchallenged to provide a detailed report of erroneous network configuration.

Observation 4c: The spec effort to determine how the UE identifies and reports IEs of an RRC Reconfiguration message for which compliance check fails is major.

Therefore, the simplest behavior of the UE is to re-establish RRC following legacy behavior.

Observation 4d: The simplest behavior of the UE is to re-establish its RRC connection at compliance check failure for an LTM candidate cell.

On the other hand, performing connection re-establishment for compliance check of an LTM candidate configuration is too costly if LTM execution is never triggered to the corresponding LTM candidate cell.

Observation 4b: Reestablishing connection due to compliance check failure for a configuration of an LTM candidate cell is too costly if LTM execution is never triggered towards that cell.

Therefore, connection re-establishment due to compliance check failure should only be performed if LTM execution is triggered towards the corresponding LTM candidate cell.

Proposal 5: The UE performs connection reestablishment due to compliance check failure for a candidate LTM configuration after LTM execution is triggered towards the corresponding LTM candidate cell.

# 5	Discussion on measurement/reporting configuration of candidate LTM cells

## 5.1	On the use of reference configuration

RAN2#121bis-e agreed that the location of RS configuration for SSB-based measurements of candidate cells is external to the configuration of the LTM candidate cells (as well as the ServingCellConfig(s) of the current serving cells).

Observation 5a: RAN2 agreed that the location of RS configuration for SSB-based measurements of candidate cells is external to the configuration of the LTM candidate cells.

Since the configuration of the LTM candidate cells does not carry the RS configuration for measurement of these cells, then including the RS configuration for the measurements of the candidate cells in the reference configuration is not useful for delta signaling of the LTM candidate configurations.

Observation 5b: Including the RS configuration for SSB-based measurements of candidate cells in the reference configuration is not beneficial for delta signaling of the LTM candidate configurations.

Therefore, the location of RS configuration for SSB-based measurements of candidate cells should also be external to the reference configuration.

Proposal 6: The location of RS configuration for SSB-based measurements of candidate cells is external to the reference configuration.

## 5.2	Support of subsequent LTM

RAN2#121bis-e agreed that the location of RS configuration for SSB-based measurements of candidate cells is external to both the ServingCellConfig(s) of the current serving cells and the configuration of the LTM candidate cells. This implies the RS configuration for SSB-based measurements of candidate cells is independent of the current serving cells. This implies that this RS configuration should remain valid when the current serving cells change due to subsequent LTM executions. Same reasoning applies to the configuration of TCI states for the candidate cells since they have the same location as the RS configuration.

Proposal 7a: The RAN2#121bis-e agreements on location of RS configuration for SSB-based measurements of candidate cells and location of configurations of TCI states for the candidate cells apply to subsequent LTM.

On the other hand, RAN2#121bis-e agreed that the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

Observation 6a: RAN2 agreed that the reporting configuration for L1 measurements of candidate cells is part of the ServingCellConfig of the current serving cell(s).

At LTM execution, the target cell becomes the new serving cell for the UE. In this case, the UE extracts the ServingCellConfig of the LTM candidate configuration, which becomes the new ServingCellConfig for the new serving cells.

Observation 6b: The target cell of LTM execution becomes the new serving cell after LTM execution.

Observation 6c: The ServingCellConfig inside the configuration of an LTM candidate cell becomes the ServingCellConfig of the new serving cell after LTM execution towards that cell.

Therefore, the ServingCellConfig inside the LTM candidate configuration of an LTM candidate cell should carry a reporting configuration, which is only used by the UE after LTM execution towards that cell. On the other hand, the reporting configuration within the ServingCellConfig of the current serving cell(s) should only be used prior to LTM execution. Thus, to support subsequent LTM, separate reporting configurations should be provided within the ServingCellConfig of the current serving cell(s) and the ServingCellConfig within the LTM candidate configurations.

Proposal 7b: To support subsequent LTM, separate reporting configurations for L1 measurements of LTM candidate cells are provided in the following locations:

- As part of the ServingCellConfig of the current serving cell(s) [already agreed]:

  - The UE uses this configuration before LTM execution towards a candidate cell.

  - The UE stops using this configuration after LTM execution towards a candidate cell.

- As part of the ServingCellConfig within the configuration of a candidate LTM cell

  - The UE does not use the configuration before LTM execution towards that candidate cell.

  - The UE starts uses the configuration after LTM execution towards that candidate cell.

In case of inter-DU LTM, support of subsequent LTM implies that each serving/candidate DU serving a cell generates the reporting configuration of other cells within the ServingCellConfig of that cell. This requires each DU to know the RS configuration of the other cells served by the other DUs. This has RAN3 impact, and RAN3 should be involved.

Proposal 7c: Send LS to RAN3 with proposals 7a and 7b to support measurement and reporting of candidate LTM cells for subsequent LTM.

## 5.3	Measurement/reporting of a candidate LTM cell that is a current SCell

In legacy behavior, UE reports L1 measurements of a serving SCell in activated state and stops these reports in the deactivated state. In case a cell is both a serving SCell and an LTM candidate cell, the UE would still need to submit L1 measurement reports of the cell in deactivated state for the sake of LTM or else LTM to the cell is never triggered. This implies that measurement of a cell that is both a serving SCell and an LTM candidate cell is independent of the activation status of that cell.

Proposal 8: LTM specific L1 measurements of an LTM candidate cell that is also a current serving SCell is independent of the activation status of that SCell.

# Conclusion

This contribution discussed subsequent LTM, delta configuration for LTM, decoding and validity check of LTM candidate configurations, and configuration of measurement and reporting of candidate LTM cells. The following observations and proposals have been made:

Discussion on subsequent LTM

Observation 1a: Subsequent LTM minimizes the overhead over the backhaul for preparation of candidate cells.

Observation 1b: Subsequent LTM minimizes the overhead over Uu by performing a sequence of cell switches with a single RRC message.

Observation 1c: Subsequent LTM improves robustness of UE mobility since cell switches rely on swift L1 measurement reporting and skip interleaved LTM preparations that require slow L3 measurement reporting.

Proposal 1a: Subsequent LTM is baseline mode of operation of the UE. Subsequent LTM is not explicitly configured.

Proposal 1b: To stop subsequent LTM, the network explicitly releases the candidate configurations via an RRC message.

Proposal 1c: The configuration of the UE’s current serving cell group when LTM is configured is by default a candidate LTM configuration following the first LTM execution.

Discussion on delta configuration

Observation 2a: A PCell of one candidate LTM cell group and an SCell of another candidate LTM cell group may be the same.

Observation 2b: If the same cell is a PCell in one candidate LTM cell group and an SCell in another candidate LTM cell group, using a common reference cell configuration to configure delta signaling for this cell is the most efficient.

Proposal 2: The UE may receive a complete LTM candidate configuration and a non-empty reference configuration, where the LTM candidate configuration does not use delta signaling on top of the reference configuration.

Proposal 3a: An SpCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

Proposal 3b: An SCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

Discussion on decoding and validity-check of LTM candidate configurations

Observation 3: Decoding and validation of a configuration of an LTM candidate cell is wasteful if LTM is never triggered towards that cell.

Observation 4a: Simple error flagging of an LTM candidate configuration is not useful.

Observation 4b: The UE may be overchallenged to provide a detailed report of erroneous network configuration.

Observation 4c: The spec effort to determine how the UE identifies and reports IEs of an RRC Reconfiguration message for which compliance check fails is major.

Observation 4d: The simplest behavior of the UE is to re-establish its RRC connection at compliance check failure for an LTM candidate cell.

Observation 4b: Reestablishing connection due to compliance check failure for a configuration of an LTM candidate cell is too costly if LTM execution is never triggered towards that cell.

Proposal 4: The UE is not expected to receive a request from the network to perform early decoding and early validity check of an LTM candidate configuration.

Proposal 5: The UE performs connection reestablishment due to compliance check failure for a candidate LTM configuration after LTM execution is triggered towards the corresponding LTM candidate cell.

Discussion on measurement/reporting configuration of candidate LTM cells

Observation 5a: RAN2 agreed that the location of RS configuration for SSB-based measurements of candidate cells is external to the configuration of the LTM candidate cells.

Observation 5b: Including the RS configuration for SSB-based measurements of candidate cells in the reference configuration is not beneficial for delta signaling of the LTM candidate configurations.

Observation 6a: RAN2 agreed that the reporting configuration for L1 measurements of candidate cells is part of the ServingCellConfig of the current serving cell(s).

Observation 6b: The target cell of LTM execution becomes the new serving cell after LTM execution.

Observation 6c: The ServingCellConfig inside the configuration of an LTM candidate cell becomes the ServingCellConfig of the new serving cell after LTM execution towards that cell.

Proposal 6: The location of RS configuration for SSB-based measurements of candidate cells is external to the reference configuration.

Proposal 7a: The RAN2#121bis-e agreements on location of RS configuration for SSB-based measurements of candidate cells and location of configurations of TCI states for the candidate cells apply to subsequent LTM.

Proposal 7b: To support subsequent LTM, separate reporting configurations for L1 measurements of LTM candidate cells are provided in the following locations:

- As part of the ServingCellConfig of the current serving cell(s) [already agreed]:

  - The UE uses this configuration before LTM execution towards a candidate cell.

  - The UE stops using this configuration after LTM execution towards a candidate cell.

- As part of the ServingCellConfig within the configuration of a candidate LTM cell

  - The UE does not use the configuration before LTM execution towards that candidate cell.

  - The UE starts uses the configuration after LTM execution towards that candidate cell.

Proposal 7c: Send LS to RAN3 with proposals 7a and 7b to support measurement and reporting of candidate LTM cells for subsequent LTM.

Proposal 8: LTM specific L1 measurements of an LTM candidate cell that is also a current serving SCell is independent of the activation status of that SCell.

# References

- Chairman Notes; TSG RAN WG2 Meeting #121bis-e

- Chairman Notes; TSG RAN WG2 Meeting #119bis-e

- Chairman Notes; TSG RAN WG2 Meeting #119-e

- Chairman Notes; TSG RAN WG1 Meeting #112-bis-e