3GPP TSG-RAN WG2 Meeting #122	R2-2305224

Incheon, Korea, May 22-26, 2023

Agenda Item:	6.7.3

Source: 	Xiaomi

Title:  	Discussion on the usage of default CBR values for case 2b

Document for:	Discussion and Decision

1	Introduction

In RAN2#119bis meeting, RAN2 further discussed the different use cases of R16/17 default CBR parameters, and reached the following agreement [1].

[Proposal 18] Changes related to default CBR parameters are postponed to next meeting. (6/10)

[Session chair]: Check companies’ understanding (assuming R17 default CBR is configured)

- Case 1: partial sensing, R17 normal pool, R17 default CBR – partial

- Case 2a: random selection, R17 normal pool, R17 default CBR – random

- Case 2b: random selection, R16/17 exceptional pool, R16 default CBR

- Case 3: full sensing, R16/17 normal pool, R16 default CBR or invalid case?

- Case 1, 2a, 2b are confirmed. Case 3 will be revisited next meeting.

In last RAN2 meeting, we discussed about the usage of default CBR value based on the reply from RAN1. RAN2 confirmed the case 3 is a valid case, but there should not be any change on R16 specification [2].

Agreement: RAN2 confirm the validity of Case-3 (usage of R16 default CBR for full sensing in normal pool). But no spec change for R16 at least.

In addition, during the offline discussion, we further discussed about the detailed changes and achieved the following agreement. Regarding the changes on exceptional pool, there was on consensus, so this issue was postponed [2].

Proposal 1 [12/15] RAN2 adopt the change as proposed in R2-2303215 in MAC for Case 1 (usage of R17 sl-DefaultCBR-PartialSensing for partial sensing in R17 normal pool). Proposal 2 [11/15] RAN2 adopt the change as proposed in R2-2302619/R2-2302647 in MAC for Case 2a (usage of R17 sl-DefaultCBR-RandomSelection for random selection in R17 normal pool). Proposal 3 [15/15] for Case 2b (usage of R16 sl-defaultTxConfigIndex for random selection in R16 exceptional pool), RRC change is not needed. And postpone the issue on whether to adopt the change as proposed in R2-2302619/R2-2302647/R2-2303215 in MAC [8/15] or not [6/14]. Proposal 4 [13/15] R17 spec change for Case 3 (usage of R16 sl-defaultTxConfigIndex for random selection in R16 normal pool) is not needed.

In this contribution, we will further discuss about this issue and provide corresponding proposals.

2	Discussion

Based on the proposed change in R2-2302619/R2-2302647/R2-2303215 in MAC, see below, it is proposed to clarify that the default CBR value is applicable only when the exceptional pool is used.

R2-2302619/R2-2302647/ R2-2303215

3> select the number of HARQ retransmissions from the allowed numbers, if configured by RRC, in sl-MaxTxTransNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped in sl-MaxTxTransNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available in case the sl-TxPoolExceptional is used;

However we think the handling of case 2b should be aligned with case 3, if we adopt this change, we should have corresponding change for case 3. Otherwise, the default R16 CBR value is only applicable for exceptional case but actually we already agreed case 3 is valid, i.e., in R17 normal pool if sensing result is not available, R16 default CBR value should be applied.

Agreement: RAN2 confirm the validity of Case-3 (usage of R16 default CBR for full sensing in normal pool). But no spec change for R16 at least.

Since we have agreed to have no change for case 3, we should not adopt this change for exceptional pool. Generally this part is kept the same as R16 specification and aligned with the guidance from RAN1.

Proposal 4 [13/15] R17 spec change for Case 3 (usage of R16 sl-defaultTxConfigIndex for random selection in R16 normal pool) is not needed.

Proposal1: For case 2b, RAN2 does not agree the change as proposed in R2-2302619/R2-2302647/R2-2303215.

3	Conclusion

In this contribution, we discussed about the usage of default CBR values case 2b and have the corresponding proposals:

Proposal1: For case 2b, RAN2 does not agree the change as proposed in R2-2302619/R2-2302647/R2-2303215.

4	Reference

[1] RAN2#119bis meeting minutes.

[2] RAN2#121bis meeting minutes.