3GPP TSG-RAN WG2 Meeting #122                                                                                   R2-2305318

Incheon, Korea, May 22 – 26, 2023

Source:	CATT

Title:	Discussions on Two TAs for Multi-DCI Multi-TRP

Agenda Item:	7.20.2

Document for:	Discussion and Decision

- 1. Introduction

In RAN2#121-bis meeting, the discussion on the Rel-18 MIMO enhancement mainly focused on the per TRP UE-initiated RACH procedure and the corresponding agreement was included in the LS [1] to RAN1 with some further questions that need to check with RAN1.

In this paper, we further discuss some open issues on multi-DCI based multi-TRP with two TAs in Section 2, except for these issues that need to wait for RAN1 response. The conclusion of this paper is summarized in Section 3.

- 2. Discussions

The application of TA upon RACH procedure

Currently, if UE decides to perform RACH for the initial TA acquisition, RAR/the Absolute Timing Advance Command MAC CE can be used by the network to include the TA command to inform UE of the value for timing advance. Regarding how to let UE know to which TRP the TA information shall be applied, this needs further consideration.

- On the inter-cell scenario for multi-DCI based multi-TRP with two TAs

For the inter-cell scenario, if UE performs CFRA by PDCCH order, RAN1 has agreed that the dedicated RACH resource shall be configured for UE to acquire the TA for each configured additional PCI,

Agreement Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

So there is no need to make enhancement for TAG indication as UE can know which TRP the acquired TA should be applied based on resource used for RACH procedure.

When UE performs CBRA for TA acquisition, according to the previous RAN1 agreement, CBRA triggered by PDCCH order is not supported.

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to support enhancements for CBRA triggered by PDCCH order.

When considering the UE initialized CBRA, as per the RAN2 agreement in last meeting,

From RAN2 perspective, per TRP UE-initiated RACH procedure is not supported.

UE shall firstly trigger CBRA upon the TRP with the serving cell PCI to recover the UL transmission, and thus has also no confusion on the UE side. If the NW wants to control UE to recover the UL transmission on another TRP by PDCCH order, the dedicated RACH resource shall be used by UE to trigger CFRA on the cell with additional PCI, so also with no confusion on this case.

Observation 1: For the inter-cell multi-DCI multi-TRP with two TAs, UE can know to which TRP the acquired TA upon RACH procedure shall be applied based on the RACH resource.

- On the intra-cell scenario for multi-DCI based multi-TRP with two TAs

For the intra-cell scenario, now RAN1 is still discussing this issue with the following candidate solutions,

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1, 2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

As can be seen from above alternatives, Alt 2 and Alt 3 have no need to enhance RAR/the Absolute Timing Advance Command MAC CE. Therefore RAN2 can wait for RAN1 further output on this issue.

Based on above analysis, it is suggested that,

Proposal 1: For the inter-cell multi-DCI multi-TRP with two TAs, no need to enhance RAR/the Absolute Timing Advance Command MAC CE to indicate the associated TAG ID.

Proposal 2: For the intra-cell multi-DCI multi-TRP with two TAs, RAN2 to wait for RAN1 on the application of TA upon RACH procedure.

On the definition of P-TAG

In legacy, a Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (P-TAG). For a serving cell associated with two TAGs, the SpCell could be contained in two TAGs. How to define the PTAG should be discussed.

RAN1 agree that for the RAR based solution for TA acquisition, RAR is only received from the TRP that is associated with Type 1 CSS,

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support at least RAR-based solution where RAR is only received from a TRP that is associated with Type 1 CSS - RAR based - FFS: RAR-less solution reusing the solution agreed in Rel-18 Mobility Enh

From this perspective, whether the P-TAG that includes the TRP associated with Type 1 CSS is the “real” P-TAG may need further clarification/discussion by RAN1/RAN2.

On the other hand, from RAN2 perspective, since RAN1 only requires a serving cell associated with two TAGs, it is straightforward that the two TAGs that include the corresponding TRPs in SpCell are both defined as P-TAGs.

Note that whether to support one/two P-TAG(s) for multi-DCI multi-TRP with two TAs will impact the UP resource handling when the expiry of TAT(s) associated with P-TAG(s) occur, therefore we propose,

Proposal 3: RAN2 to discuss the SpCell associated with two TAGs for multi-DCI multi-TRP with two TAs, i.e.,

Option 1 – Only one P-TAG for SpCell, i.e., TAG containing the TRP that associated with Type 1 CSS;

Option 2 – Two independent P-TAGs for SpCell.

TA acquisition in handover scenario

In legacy L3 handover, when UE receives ReconfigWithSync, UE will trigger RACH to acquire the TA information in the target cell.

When considering multi-DCI multi-TRP with two TAs, there may be the case where the target cell is associated with two TAGs, therefore a basic issue is whether we need enhancement for UE to acquire two TAs for two TRPs associated with different TAGs upon handover execution.

- Option 1 – Only acquire one TA for a TRP associated with a certain TAG

Option 1 aims to follow legacy as much as possible, which has less spec impact. By Option 1, when UE performs handover, UE only triggers RACH at a certain TRP to acquire the TA for the associated TAG. Before the expiry of T304, UE considers the successful access of the target cell upon completion of the RACH procedure. Then based on NW implementation, the gNB can send PDCCH order in the target cell to trigger RACH on another TRP to acquire the TA for the corresponding associated TAG.

- Option 2 – Acquire two TAs for two TRPs associated with different TAGs

Option 2 requires UE to perform RACH procedure at the two TRPs during handover. With Option 2, there may need much effort from RAN2 perspective. Currently, the MAC protocol only supports one ongoing RACH procedure at a time. Even UE can perform another RACH procedure after the completion of the first RACH procedure, there also exist the RACH resource indication issue that needs further RAN2 consideration. In legacy handover, UE is provided by the target cell with the dedicated RACH resource for access to the target cell. When considering two RACH procedures upon handover, dedicated RACH resource for two TRPs are needed but the current RRC configuration does not support this.

In summary, Option 1 can work well and we do not see much motivation to make enhancement on the handover scenario, therefore it is suggested that,

Proposal 4: For multi-DCI multi-TRP with two TAs, during handover to the target cell associated with two TAGs, UE performs RACH to acquire one TA for a single TRP associated with a certain TAG in the target cell.

- 3. Conclusion

This paper discusses some open issues on the multi-DCI based multi-TRP operation with two TAs. The contribution of this paper is summarized as follows,

The application of TA upon RACH procedure

Observation 1: For the inter-cell multi-DCI multi-TRP with two TAs, UE can know to which TRP the acquired TA upon RACH procedure shall be applied based on the RACH resource.

Proposal 1: For the inter-cell multi-DCI multi-TRP with two TAs, no need to enhance RAR/the Absolute Timing Advance Command MAC CE to indicate the associated TAG ID.

Proposal 2: For the intra-cell multi-DCI multi-TRP with two TAs, RAN2 to wait for RAN1 on the application of TA upon RACH procedure.

On the definition of P-TAG

Proposal 3: RAN2 to discuss the SpCell associated with two TAGs for multi-DCI multi-TRP with two TAs, i.e.,

Option 1 – Only one P-TAG for SpCell, i.e., TAG containing the TRP that associated with Type 1 CSS;

Option 2 – Two independent P-TAGs for SpCell.

TA acquisition in handover scenario

Proposal 4: For multi-DCI multi-TRP with two TAs, during handover to the target cell associated with two TAGs, UE performs RACH to acquire one TA for a single TRP associated with a certain TAG in the target cell.

- 4. Reference

[1] R2-2304342 LS on 2TA for multi-DCI multi-TRP    Ericsson	LS out	Rel-18	To:RAN1