3GPP TSG-RAN WG2 #122	Tdoc R2-2305354

Electronic meeting, May 22nd – 26th, 2023

Agenda Item:	7.21.3

Source:	Ericsson

Title:	Discussion on Multiple PRACH Transmission Procedures

Document for:	Discussion

# 1 Introduction

In the first RAN2-meeting for Coverage Enhancements Rel-18 (RAN2#121bis-e) the following agreements where made:

Agreements - RAN2 assumes that MSG1 repetition can be applicable to all 4-step CBRA procedures (FFS for SI request) - CFRA support is FFS - RAN2 assumes that MSG1 repetition can be applicable to NUL - RAN2 assumes that MSG1 repetition can be applicable to SUL - Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE) - RAN2 waits for further inputs from RAN1 for how to associate RA-RNTI to the PRACH occasion for multiple PRACH transmissions and also for ra-ResponseWindow start point - General assumption is that various feature combinations can be configured (which is up to network implementation), unless explicitly specified otherwise - RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS - BWP selection mechanism is not impacted by PRACH coverage enhancements. Legacy BWP selection mechanism is re-used - RA type selection mechanism is not impacted by PRACH coverage enhancements. Legacy RA type selection mechanism is re-used

In RAN1#112bis-e, the following agreements (among others) where made:

Agreements - There is no consensus to support multiple PRACH transmissions within one RACH attempt located at same time instance in Rel-18. - There is no consensus to support utilizing different preambles during the multiple PRACH transmissions with the same Tx beam in one attempt. - Multiple PRACH transmissions within one RACH attempt are only performed within one RO group. - The number of valid ROs in the RO group is equal to one of the configured number(s) of multiple PRACH transmissions. Note1: If only one value is configured for multiple PRACH transmissions, then the number of valid ROs in the RO group is equal to this value. Note2: If multiple values are configured for multiple PRACH transmissions, for each value, the number of valid ROs in the RO group is equal to the corresponding number of multiple PRACH transmissions. Note3: Valid RO(s) refers to what is defined in existing specification. - The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

Based on these agreements from RAN1 and RAN2, this discussion paper handles further transmission procedures aspects.

# 2 Discussion

## 2.2 Contention Free Random Access

During RAN2#121bis-e the following agreement was made:

- RAN2 assumes that MSG1 repetition can be applicable to all 4-step CBRA procedures (FFS for SI request) - CFRA support is FFS

It is our view that CFRA will be beneficial for certain use cases like beam failure recovery and handover. For these cases, the characteristics for the radio link is known to the gNB, and thus there is a possibility to reserve preambles that are mapped to a specific number of Multiple PRACH Transmissions. The argument has been raised that CFRA targets good radio environment while Multiple PRACH Transmission does not. That is true for other CFRA cases, but not BFR and handover.

Multiple PRACH Transmission together with CFRA could bring gains to the UE.

CFRA is supported for Multiple PRACH Transmissions, at least for BFR and handover.

By assigning a CFRA preamble to a specific set(s) of Multiple PRACH Transmissions (MuPT) [2, 4 or 8] the gNB can order the MuPT in advance, since it can be chosen by the gNB for CFRA in RRC_CONNECTED where the knowledge of the radio link is very good. The selection of a number of Multiple PRACH Transmissions (K) could be done in different ways:

Option A) One set of MuPT applicable for all SSBs.

Option B) One individual K factor for each SSB.

Option C) A set of repetition factors (denoted K) per SSB, which the UE can select between based on specified criteria.

Different strategies for assigning CFRA preambles towards Multiple PRACH Transmissions could be adopted, with different complexity levels.

RAN2 to discuss:
Option A) One set of MuPT applicable for all SSBs.
Option B) One individual K factor for each SSB.
Option C) A set of repetition factors (denoted K) per SSB, which the UE can select between based on specified criteria.

2.3 RAR Window

Since agreements in RAN1 have now been made [agreement 14], it is now possible for RAN2 to also agree on that the RAR window starts after all PRACH transmissions in a random access attempt has been sent.

The RAR window starts after the last of the PRACH transmissions have been sent.

2.4 Fallback

A UE that determines by the MuPT thresholds that it should perform repetitions might find itself in a situation where, in spite of the multiple PRACH transmissions, it receives no RAR. This will result in a failed RACH attempt and would raise the question if the repetition factor should be increased. There is also a relation to the output power of the UE, where power ramping is another solution to try to achieve a successful RACH attempt.

If, a UE transmits multiple PRACHs without reaching its maximum transmission power, it could increase repetition factors and/or output power to increase the chance of a successful RACH re-attempt.

To avoid a complex decision making between how the UE should choose between output power and increase number of PRACH transmissions it is our preferred behaviour that the UE only does multiple PRACH transmissions on the highest possible output power.

The UE only does Multiple PRACH Transmissions on the highest possible output power.

Furthermore, RAN2 agreed on RAN2#121bis-e that no fallback scenarios will be defined for legacy RACH (no MuPT) and MuPT and vice versa.

## 2.5 UE Capability

Release 17 introduced the possibility for a UE to support msg3 repetitions. It is our view that msg1 repetitions should be signalled as a separate capability. Furthermore, we believe that the UE should support all repetition factors (2, 4 and 8) since the network configuration will already use a lot of resources and the use of all repetition factors might not be needed.

A UE capability for msg1 repetitions is introduced, and the UE then supports all repetition factors.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Multiple PRACH Transmission together with CFRA could bring gains to the UE.

Observation 2	Different strategies for assigning CFRA preambles towards Multiple PRACH Transmissions could be adopted, with different complexity levels.

Observation 3	The RAR window starts after the last of the PRACH transmissions have been sent.

Observation 4	If, a UE transmits multiple PRACHs without reaching its maximum transmission power, it could increase repetition factors and/or output power to increase the chance of a successful RACH re-attempt.

Based on the discussion in the previous sections we propose the following:

Proposal 1	CFRA is supported for Multiple PRACH Transmissions, at least for BFR and handover.

Proposal 2	RAN2 to discuss: Option A) One set of MuPT applicable for all SSBs. Option B) One individual K factor for each SSB. Option C) A set of repetition factors (denoted K) per SSB, which the UE can select between based on specified criteria.

Proposal 3	The UE only does Multiple PRACH Transmissions on the highest possible output power.

Proposal 4	A UE capability for msg1 repetitions is introduced, and the UE then supports all repetition factors.

# References

3GPP WID, Further NR coverage enhancements, RP-221858, Link

Chairs notes with RAN1 agreements, RAN1#111 Toulouse

Chairs notes with RAN1 agreements, RAN1#112 Athens