3GPP TSG-RAN WG2 Meeting #122	R2-2305466

# Incheon, Korea, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0678 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on eDRX and RRM measurement relaxation for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In clause 9.2.10, it is mentioned that before establishing an RRC connection, the UE verifies that stored system information remains valid when the RRC_IDLE eDRX cycle is longer than the system information modification period. This is to avoid the SI missing case when eDRX acquisition period is used. It applies to both IDLE and Inactive state. 38.331 description. 1> if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary.. In RRC resume procedure, the UE should also verify the validity of the stored system information before resuming an RRC connection. - In TS 38.300 clause 16.13.4, it is specified that “Network may configure stationary criterion for a RedCap UE in RRC_CONNECTED and the UE shall report its RRM measurement relaxation status using UE Assistance Information when the stationarity criterion is met or no longer met”. However, it is not aligned with the description in TS 38.331 clause 5.7.4.2, which is “A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode”. The RedCap UE is allowed to not initiate UAI procedure upon the stationarity criterion is met. So the “shall” in the description in TS 38.300 should be changed. - Based on the WID for RedCap, “Power saving enhancement solutions specified in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) shall be assumed to be available also to RedCap UEs by default”. In current TS 38.300 section 16.3.4, it it not clear whether the Rel-16 RRM measurement relaxations can be applied to RedCap UEs. |
| Summary of change: |  | - In clause 9.2.10, Clarify that the UE also verifies that stored system information remains valid before resuming an RRC connection, when the RRC_IDLE eDRX cycle is longer than the system information modification period. - Change the “shall” to “can” in clause 16.13.4. - In clause 16.13.4, clarify that Rel-16 RRM measurement relaxation can also be applied to RedCap UEs. Impact analysis Impacted 5G architecture options: NR Standalone Impacted functionality: eDRX, RRM measurement relaxations Inter-operability: - If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. - If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE may not have valid system information upon or after resuming an RRC connection. - The RedCap UE is mandatory to initiate UAI procedure upon the stationarity criterion is met or no longer met, which is not aligned with RRC specfication. - RedCap UEs may not be able to use Rel-16 RRM measurement relaxation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.10, 16.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 9.2.10	Extended DRX for RRC_IDLE and RRC_INACTIVE

When extended DRX (eDRX) is used, the following applies:

-	For RRC_INACTIVE, eDRX configuration for RAN paging is decided and configured by NG-RAN. In RRC_INACTIVE the UE monitors both RAN and CN paging;

-	For RRC_IDLE, eDRX for CN paging is configured by upper layers. In RRC_IDLE the UE monitors only CN paging;

-	Information on whether eDRX for CN paging and RAN paging is allowed on the cell is provided separately in system information;

-	The maximum value of the eDRX cycle is 10485.76 seconds (2.91 hours) for RRC_IDLE and 10.24 seconds for RRC_INACTIVE, while the minimum value of the eDRX cycle is 2.56 seconds for both RRC_IDLE and RRC_INACTIVE;

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging according to DRX during a Paging Time Window (PTW) used in RRC_IDLE. The PH and PTW are determined based on a formula (see TS 38.304 [10]) that is known by the AMF, UE and NG-RAN;

-	H-SFN, PH and PTW are used if the eDRX cycle is greater than 10.24 seconds;

-	When the RRC_IDLE eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before resuming/establishing an RRC connection.

Next CHANGE

16.13.4	RRM measurement relaxations

RRM measurement relaxation is enabled and disabled by the network. In RRC_IDLE and RRC_INACTIVE a RedCap UE is allowed to relax neighbour cell RRM measurements when the stationary criterion is met or when both stationary criterion and not-at-cell-edge criterion are met. RedCap UEs can also apply RRM measurement relaxation as specified in clause 11 (e.g. RRM measurements based on low mobility and/or not at cell edge criterion). Network may configure stationary criterion for a RedCap UE in RRC_CONNECTED and the UE can report its RRM measurement relaxation status using UE Assistance Information when the stationarity criterion is met or no longer met.

END OF CHANGE