3GPP TSG-RAN WG2 Meeting #122	R2-2305473

Incheon, Korea, 22th – 26th May, 2023

Agenda item:	7.22.3

Source:	Huawei, HiSilicon

Title:	High layer procedures for LP-WUS in RRC_CONNECTED state

Document for:	Discussion and Decision

- Introduction

The study item [1] includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this paper, we discuss RAN2 impacts to support low-power wake-up signals (LP-WUS) in RRC_CONNECTED state.

- Discussion

LP-WUS is introduced to further reduce the power consumption and UE complexity, and both RRC_IDLE/INACTIVE and RRC_CONNECTED modes are to be studied as part of the LP-WUS/WUR SI in RAN1, including evaluation methodology, signal design and L1 procedure design.

RAN1#110bis-e Agreement Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI. - FFS: Further prioritization if needed during the study item. RAN1#112bis-e Agreement - For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g. UE-group, -subgroup or -ID - FFS: cell information - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information - For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring. - Other information candidates are not precluded - Study pros and cons of including above information to LP-WUS. - Note: the information may be explicitly or implicitly indicated. Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques.

For now, RAN1 focused on LP-WUS design for RRC_IDLE/INACTIVE, and considers to reuse the LP-WUS waveform for RRC_CONNECTED with further study on RRC_CONNECTED specific procedures. From RAN2 perspective, the higher layer procedures for RRC_CONNECTED mode can be studied, but whether LP-WUS can achieve power saving benefits with limited network overhead/impact on legacy Uu interface for RRC_CONNECTED mode, is also subjected to L1 signal design, LP-WUS bandwidth, receiver architecture of LP-WUR, etc, which are still under-discussion in RAN1 and RAN4. In this case, RAN2 can study the higher layer procedures for RRC_CONNECTED state, meanwhile wait for more RAN1 progresses on procedures for RRC_CONNECTED state.

Observation 1: Both RRC_IDLE/INACTIVE and RRC_CONNECTED modes are to be studied as part of the LP-WUS SI in RAN1.

Proposal 1: RAN2 studies RRC_CONNECTED state procedures for LP-WUS from RAN2 perspective, meanwhile waits for RAN1 conclusion on the benefit and potential solution for RRC_CONNECTED state.

For RRC_CONNECTED state, C-DRX was introduced in Rel-15, PDCCH-based wake-up signaling (DCP), BWP-based MIMO adaptation, fast RRC release request, UE power saving assistance information and cross-slot scheduling were introduced in Rel-16, PDCCH monitoring adaptation (including PDCCH skipping and SSSG switching) was introduced in Rel-17. One effective way to reduce power consumption in RRC_CONNECTED state is to reduce PDCCH monitoring, i.e. the motivation of introducing DCP and PDCCH monitoring adaptation. In Rel-18, LP-WUS is introduced to reduce paging monitoring in RRC_IDLE/INACTIVE state. However, the functionality and procedure of LP-WUS in RRC_CONNECTED state is different from that in RRC_IDLE/INACTIVE state, for RRC_CONNECTED state, LP-WUS at least can be used to reduce unnecessary PDCCH monitoring for power saving.

Proposal 2: LP-WUS at least aims to reduce unnecessary PDCCH monitoring in RRC_CONNECTED state.

When UE monitors LP-WUS by low-power wake-up receiver (LR), UE’s main receiver (MR) enters sleep mode and does not monitor PDCCH, the sleep mode can be micro/light/deep sleep. Once the UE detects LP-WUS that targets itself, the UE wakes up MR and starts PDCCH monitoring by MR. If the UE determines to stop the PDCCH monitoring by MR, UE can let MR go to sleep mode and use LR to monitor LP-WUS to further reduce the power consumption.

Proposal 3: The LP-WUS received by LR is used to trigger MR wake-up and start of PDCCH monitoring by MR.

Regarding the modelling of how to trigger PDCCH monitoring by MR, the following two ways on procedure design for LP-WUS in RRC_CONNECTED state can be considered.

- LP-WUS impacts C-DRX timers to reduce C-DRX active-time (DCP-like scheme)

During the C-DRX non-active-time, the UE is not required to monitor PDCCH and monitors LP-WUS by LR. If the DL data arrives, the network sends LP-WUS to the UE, triggering the UE’s MR to enter the C-DRX active-time, the UE will follow the existing UE behaviour in C-DRX active-time to monitor PDCCH. To enter the C-DRX active-time, LP-WUS can trigger the start of drx-onDurationTimer, or maybe drx-InactivityTimer considering more frequent transmission for LP-WUS and benefit on reduced latency.

- LP-WUS impacts PDCCH monitoring behaviour with/without C-DRX (PDCCH skipping-like scheme)

When UE monitors LP-WUS by LR, UE does not monitor PDCCH by MR. If the UE detects LP-WUS that targets itself, the UE wakes up MR and monitors PDCCH as existing UE behaviour until the UE is informed to stop the PDCCH monitoring, or monitors PDCCH for a period of time instead of relying on existing C-DRX timers. The impacts of combination with other UE power saving feature, e.g. C-DRX, DCP, PDCCH skipping, SSSG switching, needs to be further studied. For example, if C-DRX is configured, the LP-WUS monitoring is not impacted by C-DRX active-time, once the UE wakes up MR, it monitors PDCCH monitoring according to the C-DRX, the running status of C-DRX timers are not impacted by LP-WUS monitoring.

Proposal 4: Two ways on procedure design for LP-WUS in RRC_CONNECTED state can be considered:

- LP-WUS impacts C-DRX timers to reduce C-DRX active-time

- LP-WUS impacts PDCCH monitoring behaviour with/without C-DRX

  - If C-DRX is configured, no impacts on C-DRX timers

- Conclusion

In this paper, we discuss RAN2 impacts to support low-power wake-up signals (LP-WUS) in RRC_CONNECTED state and have the following observation and proposals.

Observation 1: Both RRC_IDLE/INACTIVE and RRC_CONNECTED modes are to be studied as part of the LP-WUS SI.

Proposal 1: RAN2 studies RRC_CONNECTED state procedures for LP-WUS from RAN2 perspective, meanwhile waits for RAN1 conclusion on the benefit and potential solution for RRC_CONNECTED state.

Proposal 2: LP-WUS at least aims to reduce unnecessary PDCCH monitoring in RRC_CONNECTED state.

Proposal 3: The LP-WUS received by LR is used to trigger MR wake-up and start of PDCCH monitoring by MR.

Proposal 4: Two ways on procedure design for LP-WUS in RRC_CONNECTED state can be considered:

- LP-WUS impacts C-DRX timers to reduce C-DRX active-time

- LP-WUS impacts PDCCH monitoring behaviour with/without C-DRX

  - If C-DRX is configured, no impacts on C-DRX timers

- References

RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo.