3GPP TSG-RAN WG2 Meeting #122                        	R2-2305722

Incheon, Korea, May 22-26, 2023

Agenda item:	7.13.2

Source:	Samsung

Title:	MRO for inter-system handover for voice fallback

Document for:	Discussion & Decision

# Introduction

In previous meetings, RAN2 discussed MRO enhancement for inter-system handover voice fallback and made the following agreements.

RAN2#119-e Agreements:

1	RAN2 discuss the following scenarios:

Suitable EUTRA cell found after MobilityFromNR failure

No suitable EUTRA cell found after MobilityFromNR failure

RAN2#119bis-e Agreements:

1	An explicit indication is included in RLF-report when mobility from NR fails and the corresponding MobilityFromNRCommand includes voiceFallbackIndication

2	The below content is included in RLF-report when reestablishment procedure is initiated due to mobility From NR failure.

a. reestablishmentCellID.

RAN2#121bis-e Agreements:

1	RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.

2	FFS: Introduce an indication for the scenario of RLF after successful voice fallback HO in the LTE RLF report regarding voice fallback.

3	UE logs the agreed indication regarding voice fallback in the NR RLF report.

4	FFS: RAN2 agree to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF. FFS explicit or implicit indications.

In this contribution we discusses further on this.

# Discussion

## Differentiation of acceptable and suitable cells in RLF report

In RAN2#121bis-e [2], there is an FFS on the need to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report for voiceFallback HOF. The proponent companies think that not differentiating acceptable cell from suitable cell that UE connected to after the failure, in the RLF report, can mislead the network to conclude that an acceptable cell can be a candidate for the next handovers with different services type. As we understand, eutraReconnectCellId is not useful for the network when it is an acceptable cell. Moreover, network can easily identify that eutraReconnectCellId is an acceptable cell or a suitable cell as ReconnectCellId includes PLMN identity.

Observation 1: ReconnectCellId includes information such as PLMN identity by which suitable cell and acceptable cell can be distinguished.

Proposal 1: No additional information is needed to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF.

## Indication for voice fallback in RLF report

In RAN2#119bis-e [3], it was decided to include an explicit indication in RLF report when mobility from NR fails during voice fallback. One way to do this is by defining a new flag. But we observe that the existing field lastHO-Type-r17 in the RLF report has two spare values left and we could define a new type as voiceFallback utilizing one spare value.

In [4], there was a proposal to include a clarification in field description that when the HOF is initiated by reception of MobilityFromNRCommand, only failure case is considered. The rationale mentioned was that RLF happens shortly after successful mobility from NR failure is not considered for voice fallback. But we made an agreement to consider this scenario also in RAN2#121[2]. Moreover, there is no need to specify all the scenarios in the field description, as it affects extensibility, and anyways UE will not store the information if the scenario is not applicable.

Observation 2: There is no need to update the field description of HO type, if used for voicefallback.

Proposal 2: A new type is included for lastHO-Type-r17 to indicate voice fallback.

## Voice Fallback failure scenarios

RAN2 has agreed to discuss the following scenarios in relation to voicefallback optimization in RAN2#119-e.

Suitable EUTRA cell found after MobilityFromNR failure

No suitable EUTRA cell found after MobilityFromNR failure

It was agreed in RAN2#119bis-e that scenario 2 (No suitable EUTRA cell found after MobilityFromNR failure) can be handled through the field reestablishmentCellID.

In the case where the UE has succeeded in selecting an E-UTRA cell after MobilityFromNR failure, the UE will move to RRC_IDLE in E-UTRA as per the below in TS 38.331 section 5.4.3.5.

2> if voiceFallbackIndication is included in the MobilityFromNRCommand message: 3> attempt to select an E-UTRA cell: 4> if a suitable E-UTRA cell is selected: 5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 4> else: 5> revert back to the configuration used in the source PCell; 5> initiate the connection re-establishment procedure as specified in clause 5.3.7;

In such a scenario, the reestablishmentCellID will not be set. With the presence of the explicit flag for indicating voice fallback and the absence of reestablishmentCellID, network can deduce that the UE has selected a suitable E-UTRA cell.

Observation 3: Network can understand the UE has selected a suitable E-UTRA cell through the presence of the explicit flag for indicating voice fallback and the absence of re-establishmentCellID.

Proposal 3: No additional indication is needed to handle the identified scenarios.

# Conclusion

In the previous sections, we have discussed MRO for inter-system handover for voice fallback and have made following observations and proposals.

Observation 1: ReconnectCellId includes information such as PLMN identity by which suitable cell and acceptable cell can be distinguished.

Proposal 1: No additional information is needed to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF.

Observation 2: There is no need to update the field description of HO type, if used for voicefallback.

Proposal 2: A new type is included for lastHO-Type-r17 to indicate voice fallback.

Observation 3: Network can understand the UE has selected a suitable E-UTRA cell through the presence of the explicit flag for indicating voice fallback and the absence of reestablishmentCellID.

Proposal 3: No additional indication is needed to handle the identified scenarios.

# References

- RP-221825	Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity).

- RAN2#121bis-e meeting report

- R2-2211101 RAN2#119bis-e meeting report.

- R2-2303143, Consideration on MRO for inter-system handover for voice fallback, ZTE Corporation, Sanechips