3GPP TSG-RAN WG2 Meeting #122	R2-2305735

Incheon, Korea, May 22-26, 2023

Agenda item:		7.18.2

Source:		Lenovo

Title:		Discussion on remaining CP issues for MT-SDT

Document for:		Discussion and Decision

# Introduction

In RAN2#121 meeting, the following agreements were achieved [1].

Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In this contribution, we discuss remaining CP issues of the MT-SDT procedure.

# Discussion

CG-SDT triggered by MT-SDT

Figure 1: CG-SDT triggered by NW for DL (MT) data

As shown in the figure 1 which displays some exemplary MT CG-SDT procedure, UE transmits on the selected CG resources, the RRCResumeRequest message (CCCH). Since UE may not have any uplink data available for transmission, it may be beneficial if UE transmits in addition to the RRCResumeRequest message some DL related control information which supports link adaptation/scheduling functionality in the NW, e.g. channel state information (CSI) and/or RSRP information, e.g. RSRP of pathloss reference, and/or beam related information on the selected CG resources. The CSI information and/or RSRP info and/or beam information can be for example transmitted within a new MAC CE.

Proposal 1: The RRCResumeRequest message and some DL related control information can be multiplexed and transmitted on the selected CG resources in response to the MT-SDT if there is no UL data available for transmission.

Proposal 2: The downlink related control information may be transmitted within a new MAC CE.

It has been discussed and agreed that UE should set the ResumeCause for the RRCResumeRequest message to mt-Access for cases when the (UL) SDT procedure was triggered by the NW for DL (MT) data. There could be another case that the UL SDT data arrive at the same time with the paging (DL SDT indication). Referring to the agreement in RAN2#121, it is allowed to initiate a MO-SDT before initiation of RRCResumeRequest for MT-SDT. Since the gNB can determine whether there is UL SDT, e.g. according to the CG-SDT/RA-SDT resource, the ResumeCause for the RRCResumeRequest message shall be set to mt-SDT when the UL SDT arrive and the paging for DL SDT has been also received.

Proposal 3: The ResumeCause for the RRCResumeRequest message shall be set to mt-SDT when the UL SDT arrive and the paging for DL SDT has been also received.

Supporting of mobility

Considering the mobility of the UE which supports MT-SDT, it is possible that the UE leaves the last serving cell and camps on a cell which supports MT-SDT and is controlled by another gNB within the RNA. It is natural that a paging message for MT-SDT is transmitted by the last serving cell when the DL small data arrives. However, due to UE mobility, it could be that one or more MT-SDT are triggered by the last serving cell within the RNA after the paging for MT-SDT terminated in the last serving cell or multiple MT-SDT are triggered by the last serving cell simultaneously.

Proposal 4: RAN2 to discuss the details to support MT-SDT within the RNA.

# Conclusion

In this contribution, we discuss the procedure of MT-SDT and provide the proposals:

Proposal 1: The RRCResumeRequest message and some DL related control information are multiplexed and transmitted on the selected CG resources for MT-SDT if no UL data available.

Proposal 2: The downlink related control information can be transmitted within a new MAC CE.

Proposal x: The ResumeCause for the RRCResumeRequest message shall be set to mt-SDT when the UL SDT arrive and the paging for  DL SDT is also received.

Proposal 4: RAN2 to discuss the details to support MT-SDT within the RNA.

# References

RAN2#121 chairman notes.