3GPP TSG-RAN WG2 Meeting #122                             	R2-2305739

Incheon, Korea, May 22–26, 2023

Agenda item:	7.23.2

Source:	Samsung

Title:	Time Synchronization Status Update via EventID

Document for:	Discussion & Decision

Introduction

In RAN2#121bis-e meeting, RAN2 made the following agreements:

Agreements - RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. - Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information: - RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally. - RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message - Postpone the UAC until CT1’s feedback on UAC framework to randomize UE access to network in the time domain - 5G Clock Quality information is carried in DLInformationTransfer message. - FFS if update of event ID is informed to UE by SI modification procedure - For Idle/Inactive mode RAN2 assumes that the NAS layer triggers the RRC connection procedure based on inputs from AS. - The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers. FFS to double check for connected mode and inactive

This document discusses details of time synchronization status update.

Discussion

In RAN2#121bis-e meeting, RAN2 agree that time synchronization update is based on EventID. Also, both RAN2 and RAN3 agreed that this time synchronization update is possible within a single gNB associated with multiple DUs. If a UE moves to a reselected cell’s coverage whose associated gNB is changed, the UE does not have any information to trace the change of the clock quality. In this case, the UE may need to enter RRC_CONNECTED. Since RAN2 agreed to indicate the information to NAS layer, it is natural to have the same behaviour for the case of gNB change.

Proposal 1. If gNB ID is changed, the UE indicates it to the upper layer.

In order to make it work, EventID should uniquely determine the clock quality supported by the cell. We should avoid the case that the same EventID is used by different DUs with different clock qualities at the same time.

Proposal 2. Within a single gNB coverage, EventID uniquely determines the clock quality supported by the cell.

Change of EventID means that at least one UE’s guaranteed clock quality is changed. Thus, EventID needs to be linked to UE’s clock quality information signalled via DLInformationTransfer. A simple way is to signal EventID together with other clock quality information via DLInformationTransfer. By using this EventID, UE checks if the EventID is changed in SIB9. If the UE’s stored EventID is different EventID received in SIB9, the UE needs to indicate it to the upper layer.

Proposal 3. EventID is included in DLInformationTransfer. UE stores the received EventID.

Proposal 4. If the UE’s stored EventID is different from EventID received in SIB9, the UE indicates it to the upper layer.

It is assumed that RRC_CONNECTED UE periodically receives time information (i.e. ReferenceTimeInfo) and gNB can directly send clock quality information via DLInformationTransfer. EventID is needed only if clock quality information cannot be signalled to UE. Thus, checking EventID change is not necessary for RRC_CONNECTED UE.

Proposal 5. RRC_CONNECTED UE does not check if EventID is changed.

How to react the change of EventID will be determined based on application layer status, so RAN2 agreed NAS can handle. We see there is no difference among RRC_INACTIVE and RRC_IDLE states. Thus, we prefer a common procedure for those RRC states unless a big issue is found.

Proposal 6. Time Synchronization Status procedure based on EventID is common for RRC_INACTIVE and RRC_IDLE.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. If gNB ID is changed, the UE indicates it to the upper layer.

Proposal 2. Within a single gNB coverage, EventID uniquely determines the clock quality supported by the cell.

Proposal 3. EventID is included in DLInformationTransfer. UE stores the received EventID.

Proposal 4. If the UE’s stored EventID is different from EventID received in SIB9, the UE indicates it to the upper layer.

Proposal 5. RRC_CONNECTED UE does not check if EventID is changed.

Proposal 6. Time Synchronization Status procedure based on EventID is common for RRC_INACTIVE and RRC_IDLE.