3GPP TSG-RAN WG2 Meeting #122	R2-2305811

Incheon, Korea, 22 – 26 May, 2023

Agenda item:	7.14.3

Source:	Huawei, HiSilicon

Title:	Discussion on Rel-17 left-over issues

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss two issues which are seen as left-overs from Rel-17:

- Threshold based RAN visible QoE reporting.

- RAN overload handling enhancements

# Threshold based RAN visible QoE reporting

## Background

At the last RAN2 meeting, the following SA4 LS was treated and noted [1]. The relevant topic was not discussed.

R2-2304492 Reply LS on buffer level threshold-based RVQoE reporting (S4-230684; contact: Apple) SA4 LS in Rel-18 NR_QoE_enh-Core To:RAN2 Cc: RAN3 SA4 thanks RAN2 on their LS on buffer level threshold-based RVQoE reporting. Given that • There already exists mechanism before Rel-18 for application layer to be configured for QoE reporting, and that this mechanism can be reused by the application layer to do RVQoE reporting based on the trigger of the buffer level threshold, and • The application layer can make a buffer-threshold based decision in a more timely fashion compared to the AS layer, since the corresponding application layer reporting, based strictly on reporting periodicity, may be unable to submit QoE reports at the exact time that buffer level threshold is reached. As result, and depending on the reporting periodicity, the delay between a threshold occurrence and the next scheduled QoE report may precluding a more timely remedial response by the gNB. Hence SA4 can confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. Noted (any RAN2 actions can be considered in RAN2#122)

RAN3#119bis-e made the following progress [2].

R17 Leftover - LS to SA5 on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload in R3-232047 Agreed - The assistance information can be introduced only when the clear definition is defined. (captured in the minutes, not the formal agreement) - Radio-related event triggers for RVQoE reporting is not supported in Rel-18. - If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the application session or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time. - Discuss whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two thresholds. - RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options: - Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports) - Option 2: Periodically based on a gNB configured reporting periodicity - Option 3: A certain number of times based on gNB configured report amount - WA: A class-2 procedure is used for DU to deactivate the RVQoE reporting over F1AP. - RAN3 should discuss whether the deactivation of RVQoE reporting over F1 is performed per RVQoE configuration or not. To be continue: Further discuss whether to introduce TTT(time to trigger) for threshold-based triggers. Further discuss the details of the procedure used for RVQoE deactivation over F1, e.g., legacy or new procedure, UE associated or non-UE associated signaling. Clarify whether the DU triggered deactivation of RVQoE reporting over F1 pertains only to the present application session. Further discuss and clarify the necessity of DU participation in assembling RVQoE configuration.

## RAN2 impacts for threshold based RAN visible QoE reporting

In the original RAN2 LS R2-2302042, RAN2 prefers APP layer handling, and the details are:

- With APP layer triggering, APP layer provides RVQoE measurements reports to AS layer when the measured buffer level satisfies a buffer level threshold, and the AS layer reports to gNB the RVQoE measurements received from the APP layer.

In the SA4 reply LS R2-2304492, SA4 confirms RAN2 preference.

Observation 1: Both RAN2 and SA4 prefer APP layer handling for buffer level threshold-based triggering of RVQoE reporting.

The next step is about the RAN2 impacts due to APP layer handling.

In the RAN3#119b-e meeting, RAN3 had a discussion on whether it is feasible to activate both the RVQoE threshold based trigger feature and RVQoE periodic reporting feature at a same time. However, we feel it is important for us to first define what the threshold is before discussing the combination of different features. Without any features activated, in APP layer, there is one default measurement periodicity and one default reporting periodicity as defined in the QoE configuration container. In addition, the default reporting periodicity is not the same as the default measurement periodicity, where in usual cases one report contains multiple measurement results.

When it comes to threshold based trigger, it simply means APP layer will not report measurement reports as a default way. Instead, some filtering work should be done by the APP layer, and only the results which cross certain threshold will survive and be forwarded to the AS, and later to the RAN.

In our view, there are two types. The first type is of course buffer level lower than a certain threshold, in which case, RAN will try to improve the performance. The other type is the good result which offers a ‘safe guard’, i.e. threshold larger than a certain value. After receiving such good result, RAN node understands the measurements are good and no action is needed.

- RAN2 to discuss two types of results: a) Buffer level lower than a certain threshold b) Buffer level higher than a certain threshold.

Then RAN2 should decide the mechanism of reporting RVQoE results when certain threshold is met. We note that this also relates to one agreement achieved in the previous RAN3 meeting:

RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options: a. Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports) b. Option 2: Periodically based on a gNB configured reporting periodicity c. Option 3: A certain number of times based on gNB configured report amount

In our view, option 1 is feasible and simple. Note that, RAN node mainly cares about the bad results, so we can first set one threshold as buffer level smaller than a certain value (e.g, 5ms). Once one measurement result exceeds the lower bound, the APP layer forwards this result to AS layer, and AS forwards it to RAN. Then there is no need to keep reporting the bad results, since in usual cases the bad performance will continue for a while, and RAN will start some resource optimization and adjustment once receiving the bad report. In addition, after one threshold is met and the result is reported, we can set another new threshold as buffer level larger than a certain value (e.g, 8ms). This is meaningful, as APP layer will only forward report to AS layer again when the performance is back to the good state which implicitly indicates that RAN’s adjustment takes effect. However, reconfigure the threshold every time a result is reported may increase the signalling overhead.

Also we note that one possible drawback of option 1 is it is vulnerable to accidental results. That is, in some cases, the good/bad results are only momentarily. To avoid such situation, we think a ‘time-to-trigger’ (TTT) proposed by some company is useful. However, it should be noted that buffer level may fluctuate dynamically within a certain time period which could prevent the report to be triggered if the buffer drops below (/increase above) the threshold during TTT even for a very short time. TTT would be then restarted and the report could never be sent. To avoid this, the results within TTT should be averaged over TTT time and if the average exceeds the threshold the report should be sent.

- There should be only one threshold defined at a certain time.

- UE should send the RVQoE reports just once when the threshold is met.

- RAN2 to support TTT for buffer level reporting.

- The report should be triggered when the average buffer level within TTT time meets the threshold.

# QoE reporting enhancement for overload scenario

RAN2 tentatively discussed QoE reporting enhancement for overload scenario during RAN2#119bis meeting and it was decided to postpone the discussion and wait for further RAN3 progress.

7: RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3).

During the last RAN2 meeting, RAN2 also made the following agreement in the scope of QoE for MBS broadcast discussion:

- 4: As a default behavior, when the UE’s buffer for storing QoE reports is full and a new report arrives, the UE should discard older report(s) to make room for the new one. - 5: FFS whether it is possible to provide information (e.g. priority, service type, etc.) to UE about buffering for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full.

In general, companies agreed that selection policies or priorities for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full would be useful. However, it was unclear how such priorities or policies would be defined and it was eventually decided to postpone the decision. It should be noted that a related topic was discussed by RAN3 in the last meeting and it was decided to request some clarifications on this aspect from SA5, see the LS in [3]. RAN3’s main intention is to support this mechanism to deal with RAN overload scenario, e.g. for the gNB to decide which QOE configurations to pause. From RAN2 point of view, if such information is forwaded to the UE, the UE may use it to decide which reports to keep either while reporting is paused or when buffering MBS QOE reports while in RRC INACTIVE/IDLE state. We then propose that RAN2 agrees about the usefulness of this mechanism, but only discusses the details once the reply from SA5 is received and RAN3 clarifies the contents of the intended assistance information provided from OAM to RAN.

- RAN2 agrees that assistance information for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full is useful, e.g. for storing QoE reports during pause or in RRC IDLE/INACTIVE state. RAN2 should wait for RAN3 conclusion on the contents of assistance information provided from OAM to RAN before working on the details.

# Conclusion

For discussing the threshold based RAN visible QoE reporting, our observation and proposals are as below:

Observation 1: Both RAN2 and SA4 prefer APP layer handling for buffer level threshold-based triggering of RVQoE reporting.

- RAN2 to discuss two types of results: a) Buffer level lower than a certain threshold b) Buffer level higher than a certain threshold.

- There should be only one threshold defined at a certain time.

- UE should send the RVQoE reports just once when the threshold is met.

- RAN2 to support TTT for buffer level reporting.

- The report should be triggered when the average buffer level within TTT time meets the threshold.

- RAN2 agrees that assistance information for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full is useful, e.g. for storing QoE reports during pause or in RRC IDLE/INACTIVE state. RAN2 should wait for RAN3 conclusion on the contents of assistance information provided from OAM to RAN before working on the details.

# References

[1] Draft_R2_121bis_meeting_report_v1

[2] RAN3_119bis-e_agenda_20230426_EOM1

[3] R3-232047, LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload, RAN3