3GPP TSG-RAN WG2 #122	Tdoc R2-2305842

Incheon, South Korea, May 22 – 26, 2023

Agenda Item:	7.3.4

Source:	Ericsson

Title:	NES Cell selection/reselection

Document for:	Discussion, Decision

# 1	Introduction

In the WI description for NW energy savings (RP-223540), the following objective is listed:

4.	Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

This contribution further discusses this topic and whether there is a need to specify a new mechanism.

# 2	Discussion

The WI objective states that the mechanism to be specified is related to legacy UEs, i.e. it is not within the WI scope any additional functionality on cell selection/reselection for Rel-18 UEs capable of any feature specified in this WI.

Enhancements for cell selection/reselection are not within the WI scope. It is only required to specify a way to prevent legacy UEs from camping on a cell applying a NES technique.

In our understanding, the objective was only included since there could be a NES technique specified that is non backwards compatible with legacy UEs, and hence finding a way to prevent legacy UEs from camping on those cells would be required.

As discussed previously in our contribution for RAN2#121 (R2-2301401), the only WI objective that could have impact on legacy UEs is the one on cell DTX/DRX. However, as currently agreed in RAN2, the cell DTX/DRX feature should not have impact to legacy UEs, since it was agreed that there will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs.

All techniques that are being specified for NES should be backwards compatible.

In this release, RAN2 will not specify any mechanism to prevent legacy UEs from camping on a cell adopting a NES technique.

Furthermore, it has also been discussed on how to define a NES cell. The definition of a NES cell may be useful when implementing the NES techniques in the specifications as an umbrella term when one may want to refer to a cell applying any of the specified techniques. But it should be noted that this is merely a term that may be used if convenient, which can be easily seen when drafting CRs for NES techniques. Hence, a definition of NES cell is not essential for the WI objectives. If really needed, we could simply refer to the technique that we actually mean e.g. cell applying DTX/DRX.

The definition of NES cell is not essential for the WI objectives. RAN2 can refer to the specific technique that it wants to discuss e.g. cell applying DTX/DRX.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Enhancements for cell selection/reselection are not within the WI scope. It is only required to specify a way to prevent legacy UEs from camping on a cell applying a NES technique.

Observation 2	All techniques that are being specified for NES should be backwards compatible.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In this release, RAN2 will not specify any mechanism to prevent legacy UEs from camping on a cell adopting a NES technique.

Proposal 2	The definition of NES cell is not essential for the WI objectives. RAN2 can refer to the specific technique that it wants to discuss e.g. cell applying DTX/DRX.