3GPP TSG-RAN WG2 Meeting #122	R2-2305875

Incheon, South Korea, 22– 26 May 2023

Agenda item:	6.1.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on per-band per-BC power class capability signalling

Document for:	Discussion and Decision

# 1	Introduction

In the last RAN2 meeting, there was a discussion on ue-PowerClassPerBandPerBC-r17 and how RAN4 reply LS to RAN2 should be interpreted, especially the meaning of the word “supersede” used in RAN4 LS. This contribution is to present our understanding on the issue.

# 2	Discussion

In RAN4 reply LS to RAN2 (R4-2303630), it is stated as below:

Response to 2): ue-PowerClassPerBandPerBC-r17 indicates the power class that a UE supports for each individual band within a given band combination, while powerClass or powerClass-v1610 indicates the power class for this band combination, in other words, the maximum total output power. If indicated, ue-PowerClassPerBandPerBC-r17 shall supersede other power class capabilities such as ue-PowerClass/powerClass and its extensions in determining the power class of the individual bands within a band combination.

According to our interpretation, the maximum total power in case of inter-band UL CA (i.e., when there is uplink configured in two individual band within a given band combination) and which is signalled using powerClass or powerClass-v1610, always apply. Similarly, the maximum total power in case of intra-band UL CA (i.e., when there is uplink CA configured within a band) and which is again signalled using powerClass or powerClass-v1610, always apply. This is also because there already exists another parameter that indicates whether UE can ”exceed” the per-BC power class, i.e. higherPowerLimit-r17.

Proposal 1: Maximum total power for a band combination always applies even if UE indicates per-band-per-BC power class for inter-band CA.

However, the confusion arises because RAN4 mentioned both ue-PowerClass and powerClass in the LS which could be superseded by the Rel. 17 capability, i.e., ue-PowerClassPerBandPerBC-r17. In our view, this is a matter of interpretation which is clarified by the following examples.

Example 1:  Let’s assume the inter-band UL CA for BC: n2A-n77A and UE signals the following capabilities:

- ue-PowerClass for band n1: PC2 (i.e., 26 dBm)

- ue-PowerClass for band n77: PC2 (i.e., 26 dBm)

- powerClass for band combination in case of CA_n1A-n77A: PC2 (i.e., 26 dBm)

- ue-PowerClassperBandperBC-r17 for n77: PC3 (i.e., 23 dBm)

In this case, UE follows both powerClass for n1A-n77A (PC2) and ue-PowerClassperBandperBC-r17 for n77 (PC3) when configured with inter-band CA for n1A-n77A. When UE is not configured with inter-band CA, UE follows ue-PowerClass for band n1 or n77 when operating on any of these bands as single carrier.

Example 2:  Let’s assume the intra-band UL CA for BC: n77C. In this case, ue-PowerClassperBandperBC-r17 is not signalled as it is applicable only to inter-band UL CA (based on RAN4 reply in R2-2302435).

- ue-PowerClass for band n77: PC2 (i.e., 26 dBm)

- powerClass for band combination in case of CA_ n77C: PC2 (i.e., 26 dBm)

In this case, there is no ambiguity.

Example 3: UE signals the following capabilities:

- ue-PowerClass for band n1: PC2 (i.e., 26 dBm)

- ue-PowerClass for band n78: PC2 (i.e., 26 dBm)

- powerClass for band combination in case of CA_n78C: PC3 (i.e., 23 dBm – not signalled but this is the default PC for all band combinations)

- powerClass for band combination in case of CA_n1A-n78C: PC3 (i.e., 23 dBm)

- ue-PowerClassperBandperBC-r17 for n78: PC2 (i.e., 26 dBm)

In case UE is configured with inter-band UL CA, UE follows both powerClass for n1A-n78C (PC3) and ue-PowerClassperBandperBC-r17 for n78C (PC2). One interpretation could be that UE “supersedes” other powerClass for CA_n78C and ue-PowerClass for n78 when performing inter-band UL CA. However, in our view, it is a matter of interpretation and UE is essentially not “superseding” other power limitations but rather using the capability signalling which is only applicable to a given configuration. In other words, powerClass for band combination CA_n78C is not applicable in case of band combination n1A-n78C. Similarly, in case UE is configured with intra-band UL CA (CA_n78C), UE follows powerClass for band combination CA_n78C (i.e. PC3). In this case, powerClass for band combination n1A-n78C is not applicable.

Observation 1: The use of word “superseding” is ambiguous and should be avoided in specifications.

Based on this, we propose to implement the changes as below.

| ue-PowerClassPerBandPerBC-r17 Indicates the UE power class per band per band combination. This field is applicable only to NR inter-band UL CA (i.e., when there is uplink configured in two different operating bands. Each uplink band contains only single UL CC or intra-band contiguous UL CA). If the UE signals this field, ue-PowerClassPerBandPerBC-r17 shall determine the maximum power per band in this band combination as defined in TS38.101-1 [2]. Note: UE shall follow both per band combination and per band power classes. | FS | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Proposal 2: Implement the above changes in TS 38.306.

# 3	Conclusion

Further, we observe that:

Observation 1: The use of word “superseding” is ambiguous and should be avoided in specifications.

Based on it, we propose the following.

Proposal 1: Maximum total power for a band combination always applies even if UE indicates per-band-per-BC power class.

Proposal 2: Implement the above changes in TS 38.306.