3GPP TSG-RAN WG2 Meeting#122                                	R2-2305890

Incheon, Korea, May 22-26, 2023

Agenda item:	7.3.5

Source: 	Google Inc.

Title: 	CHO Enhancements Supporting NES

Document for:	Discussion and Decision

# Introduction

The NES techniques in time and frequency domains aim to reduce the power consumption by turning off some symbols/slots/frames on one or more carriers, and hence the gNB is allowed to perform some micro/light/deep sleep, depending on the interval between the contiguous active transmission/reception occasions. However, these NES techniques may increase the data transfer latency for a connected UE, as allowing a gNB to sleep means the UE is not always able to communicate with the gNB. It is especially intolerable for the UEs having stringent data latency requirement if the gNB has to sleep for a very long period of time. For those UEs not tolerable of such NES techniques, it is important to handover the UEs in time to another gNB that does not enable these NES techniques, before the serving gNB goes to sleep. Regarding the CHO enhancements, RAN2 has made the following agreements during the RAN2#121bis_e meeting [1].

Agreements in RAN2#121bis_e - RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”. FFS further details - For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode” - As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration. FFS what trigger is used for execution of CHO

This paper first discuss if the existing CHO execution conditions need to be improved or enabled for the NES scenario, and then comes out with several proposals that aim to enhance CHO based on the analysis/observation provided in the paper.

# Discussion

Regarding the gNB behaviors during the non-active (i.e., off) period in the cell DTX/DRX mode, there are four possibilities according to TR 38.864 [2]:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS)

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement)

If a gNB acts like Example 1 above, before the gNB starts a cell non-active period, all connected UEs have to leave the gNB and wish to be handed over properly to another gNB. If gNB acts like Example 2 or Example 4 above, UEs that are actively transmitting/receiving data or have stringent requirement on data latency may wish to be handed over to another gNB that does not enable the time-domain energy saving technique.

Since the gNB may need to handover a large number of UEs before starting a cell non-active period, it might be better if the gNB can start configuring the candidate cells and/or the CHO execution conditions to different UEs as early as possible, and let these UEs autonomously activate the CHO execution conditions a short time before the gNB starts the cell non-active period. It is also important to ensure that the CHO execution conditions won’t be blocked by the signal strength/quality of the serving cell, otherwise the CHO procedure may never be executed. As condEventA4 considers only the signal strength/quality of the neighbor cell, it is an ideal CHO execution condition that can make the UE move to another cell easier. However, according to the current condExecutionCond field description (in TS 38.331 [3]), condEventA4 can be configured to a UE as a CHO execution condition ONLY in NTN.

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for CHO, CPA, intra-SN CPC without MN involvement or MN initiated inter-SN CPC. When configuring 2 triggering events (Meas Ids) for a candidate cell, the network ensures that both refer to the same measObject. For CHO, if the network configures condEventD1 or condEventT1 for a candidate cell, the network configures a second triggering event condEventA3, condEventA4 or condEventA5 for the same candidate cell. The network does not configure both condEventD1 and condEventT1 for the same candidate cell. For CHO in terrestrial networks, the network does not indicate a MeasId associated with condEventA4. For CPA and for MN-initiated inter-SN CPC, the network only indicates MeasId(s) associated with condEventA4. For intra-SN CPC, the network only indicates MeasId(s) associated with condEventA3 or condEventA5.

CondEventA4 is an ideal CHO execution condition that allows UE to move to a candidate neighbor cell easier before the serving cell enters the “NES mode”, but is only applicable in the NTN scenario.

To facilitate the CHO execution before the serving cell starts the NES mode, the network should be allowed to configure a MeasId associated to condEventA4 to a UE supporting the NES feature.

CondEventA4 can be configured as a CHO execution condition in the NES scenario.

Given that the interval between when the UE is provided with the CHO configuration and when the cell non-active period actually starts could be quite long, the UE may not want to move to the candidate cell immediately upon being provided with the candidate cell configuration, as the serving cell may still provide much better signal strength/quality. Therefore, it is very reasonable for the NW to deactivate the MeasId associated to condEventA4 upon configuring the MeasId to the UE, and then to activate the deactivated MeasId at a later time before the occurrence of the cell non-active period. The NW may deactivate the MeasId explicitly or implicitly upon configuring the MeasId to the UE. For instance, the NW may signal a MeasId together with a flag associated to the MeasId, which indicates whether this MeasId is being activated or deactivated. On the other hand, a MeasId can be considered (by both the UE and NW) as being deactivated by default if the MeasId is associated with condEventA4.

The MeasId associated to condEventA4 can be deactivated upon being configured to a NES-compatible UE. FFS on how to deactivate the MeasID associated to condEventA4 (e.g., implicitly or explicitly deactivate).

Regarding the mechanism/timing to activate a deactivated MeasId, we think a deactivated MeasId can be activated upon the activation of the serving cell DTX/DRX/NES mode, regardless of whether the serving cell DTX/DRX/NES mode is being activated explicitly (e.g., using L1 signaling) or implicitly (e.g, upon being configured with RRC signaling). RAN2 can further consider if a deactivated MeasId can be activated X time units before the occurrence of a cell non-active period, if the UE is able to determine exactly the arrival time of an aperiodic/periodic cell non-active period, where the value X can be a pre-defined or a configurable value.

The MeasId associated to CondEventA4 can be activated upon the activation of a cell DTX/DRX/NES mode. FFS whether the MeasId associated to CondEventA4 can be activated X time units before the arrival of a cell non-active period.

On the other hand, as a neighbor cell may also activate the NES mode after being configured by the serving cell as a CHO candidate cell for a UE, it is better if the serving cell can inform the UE of whether the neighbor cell is or will be operating in the NES mode in a dynamic way. With such information, the UE will become able to dynamically adjust the CHO execution condition (e.g., activating or deactivating the MeasId associated to condEventA4) for the neighbor cell, based on the whether the neighbor cell is/will be activating the NES mode.

The network can inform the UE of whether a neighbor cell is or will be operating in the NES mode. FFS on the signaling.

# Conclusions

In this paper, the CHO enhancements for supporting NES are discussed, with the following observation being made.

Observation 1	CondEventA4 is an ideal CHO execution condition that allows UE to move to a candidate neighbor cell easier before the serving cell enters the “NES mode”, but is only applicable in the NTN scenario.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	CondEventA4 can be configured as a CHO execution condition in the NES scenario.

Proposal 2	The MeasId associated to condEventA4 can be deactivated upon being configured to a NES-compatible UE. FFS on how to deactivate the MeasID associated to condEventA4 (e.g., implicitly or explicitly deactivate).

Proposal 3	The MeasId associated to CondEventA4 can be activated upon the activation of a cell DTX/DRX/NES mode. FFS whether the MeasId associated to CondEventA4 can be activated X time units before the arrival of a cell non-active period.

Proposal 4	The network can inform the UE of whether a neighbor cell is or will be operating in the NES mode. FFS on the signaling.

# Reference

Draft Report of 3GPP TSG RAN WG2 meeting #121bis_e.

3GPP TR 38.864 v1.0.0, Study on network energy savings for NR.

3GPP TS 38.331 v18.1.0, NR, Radio Resource Control (RRC) protocol specification.