3GPP TSG-RAN WG2 Meeting#122                                	R2-2305894

Incheon, Korea, May 22-26, 2023

Agenda item:	7.6.2.2

Source: 	Google Inc.

Title: 	Issues for the GNSS Validity Reporting

Document for:	Discussion and Decision

# Introduction

According to the Rel-17 specification (TS 36.300 v17.4.0) [1], a UE shall obtain its valid GNSS position before connecting to a NTN cell, and shall move to the idle state upon outdated GNSS position. This is because UE’s location information is essential for performing time and frequency compensation and a NB-IoT UE is not able to perform the mobile communication task with the BS while accessing the GNSS module. In Rel-18, the enhancements on the GNSS operation are discussed in RAN1/RAN2, with the intention to allow UE to remain in the connected state while conducting the GNSS position fix. In the RAN2#121bis_e meeting, the following agreements were made with regard to the GNSS operation enhancement [2].

Agreements in RAN2#121bis_e: - There is no need for UE to provide GNSS position fix time duration in Msg3. - RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS - UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry - For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap - For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE - RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting - Working Assumption: GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

In this paper, we discuss several leftover issues based on the agreements highlighted in yellow above, and attempt to resolve them with some remedies proposed in the paper.

# Discussion

## GNSS measurement gap

Given that now a UE can stay in RRC_CONNECTED when the current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap, the next step we need to consider is what if the UE fails to conduct the GNSS position fix during the GNSS measurement gap, and the GNSS position has become out-of-date? As according to the agreement, the UE failing to conduct the GNSS position fix can still remain in the connected state at the time when the UE exits the measurement gap. To address this issue, we think it is relatively simple on the UE side, as if UE doesn’t conduct the GNSS position fix successfully in a measurement gap, we can let the UE transition to the idle state immediately at the end of the measurement gap, if the GNSS validity duration has also expired.

UE transitions to the idle state at the end of the measurement gap configured for the GNSS position fix, if the GNSS validity has expired and the UE failed to conduct the GNSS position fix in the measurement gap.

However, on the NW side, the NW may need to rely on whether the UE has reported the GNSS validity duration after the GNSS measurement gap ends, to determine whether to keep the UE in the connected state or not. Therefore, we need a mechanism to make UE report the GNSS validity duration, preferably within a period of time after the GNSS measurement gap ends. If the UE is not able to report the GNSS validity duration within the period, the NW considers the GNSS position fix was not conducted successfully by the UE, and hence transits the UE to the idle state if the GNSS validity duration has also expired. Therefore, we should introduce a new timer and let UE/eNB start the new timer at the end of the GNSS measurement gap, where the new timer is stopped upon UE reporting (or eNB receiving) the GNSS validity duration. Upon the expiry of the new timer, UE transits to the idle state and eNB releases the UE connection, if the GNSS validity duration has also expired.

UE/eNB starts a new timer at the end of the measurement gap configured for the GNSS position fix.  UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE transitions to the idle state, when both the new timer and UE’s GNSS validity duration expire.

Regarding the FFS on “whether the UE reports every time or only if the validity duration changes” left in the RAN2#121 meeting, we think the UE shall report every time after conducting the GNSS position fix, regardless of whether the validity duration has changed or not, as reporting the validity duration shall be regarded as an acknowledgement to a GNSS measurement gap configured by the network. Otherwise the network does not know whether and when to release the UE.

RAN2 to confirm the WA: UE reports the remaining GNSS validity duration every time after the GNSS measurement.

After the UE has reported a GNSS validity duration, the GNSS validity duration may change due to UE mobility. Therefore, the next GNSS measurement gap configured to the UE, which is based on the GNSS validity duration reported by the UE, may become not appropriate any more, as the UE may already lose its UL synchronization with the NW before entering the GNSS measurement gap. To address this issue, UE should be allowed to report again its GNSS validity duration (with a smaller value) to the NW, once certain triggering condition(s) is/are met, where the triggering condition(s) should be configured by the NW. The triggering condition(s) can be for instance: 1) UE has received a TA command from the eNB indicating a TA adjustment larger than a threshold value; 2) UE has received more than k times the TA command from the eNB in a given period of time; 3) UE’s accumulated TA adjustment value (i.e., NTA) has exceeded a threshold value.

UE can be configured with a triggering condition(s) for reporting the GNSS validity duration. UE transmits an updated GNSS validity duration to the NW, once the triggering condition(s) is/are met.

## GNSS position fix with c-DRX

Apart from the measurement gap, we may also need to consider other mechanisms that could also provide the UE with an inactive period for conducting the GNSS position fix, such as the c-DRX mechanism. In RAN2#121bis_e meeting, it was concluded that RAN2 can discuss UE autonomously reacquire GNSS during inactive state of c-DRX based on RAN1’s input in the next RAN2 meeting. Moreover, RAN1 has made some progress and has sent a LS [3] to RAN2 stating the following:

- From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2

To utilize the c-DRX mechanism, we think if the UE has been configured with the c-DRX, it is very likely the GNSS validity duration is going to expire in the c-DRX inactive state (i.e., the c-DRX OFF duration). In this case, similar to the case where the UE is within a GNSS measurement gap, the UE should be allowed to stay in the connected state until it gets back to the c-DRX active state (i.e., the c-DRX ON duration). This is because the UE may need to prioritize the configured DL/UL transmissions (according to RAN1 LS) and hence may not be able to conduct the GNSS position fix in time (i.e., before the GNSS validity duration expires) during the inactive state of the c-DRX.

UE can remain in the connected state when the GNSS validity duration expires in a c-DRX inactive state (i.e., c-DRX OFF duration).

After the UE has conducted a GNSS position fix in the c-DRX inactive state, we should let UE/eNB start a new timer at the end of the c-DRX inactive state (i.e., the beginning of the c-DRX active state), where the new timer is stopped upon UE reporting (or eNB receiving) the GNSS validity duration. Upon the expiry of the new timer, UE transitions to the idle state and eNB releases the UE, if the GNSS validity duration has also expired.

When UE’s GNSS validity duration expires in a c-DRX inactive state, UE/eNB starts a new timer at the beginning of the next c-DRX active state.  UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE transitions to the idle state, when both the new timer and UE’s GNSS validity duration expire.

# Conclusion

In this paper, we discuss several issues on the GNSS measurement gap, and also extend the discussion to cover also the c-DRX scheme. Based on the analysis and discussion in the paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	UE transitions to the idle state at the end of the measurement gap configured for the GNSS position fix, if the GNSS validity has expired and the UE failed to conduct the GNSS position fix in the measurement gap.

Proposal 2	UE/eNB starts a new timer at the end of the measurement gap configured for the GNSS position fix.  UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE transitions to the idle state, when both the new timer and UE’s GNSS validity duration expire.

Proposal 3	RAN2 to confirm the WA: UE reports the remaining GNSS validity duration every time after the GNSS measurement.

Proposal 4	UE can be configured with a triggering condition(s) for reporting the GNSS validity duration. UE transmits an updated GNSS validity duration to the NW, once the triggering condition(s) is/are met.

Proposal 5	UE can remain in the connected state when the GNSS validity duration expires in a c-DRX inactive state (i.e., c-DRX OFF duration).

Proposal 6	When UE’s GNSS validity duration expires in a c-DRX inactive state, UE/eNB starts a new timer at the beginning of the next c-DRX active state.  UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE transitions to the idle state, when both the new timer and UE’s GNSS validity duration expire.

# Reference

3GPP TS 36.300 (v17.4.0), E-UTRA and E-UTRAN Overall Description, stage 2.

Draft Report of 3GPP TSG RAN WG2 meeting #121bis_e.

R2-2304612, LS on GNSS position fix during inactive state of Connected DRX for improved GNSS operations, MediaTek.