3GPP TSG RAN WG2#122	R2-2305905

Incheon, Korea, 22nd - 26th May, 2023

Agenda item:	7.19.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On eDRX for enhanced RedCap

WID/SID:	NR_redcap_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The work item on enhanced support of reduced capability NR devices was approved in RP-223544 with the following objectives for eDRX in INACTIVE:

Power saving/energy efficiency enhancements

- Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4]

  - Note that this objective requires SA2, CT1 and CT4 involvement

The following was agreed in RAN2#121 meeting:

The formula of PH/PTW for IDLE eDRX can be reused for enhanced INACTIVE eDRX, for eDRX cycles longer than 10.24s. RAN2 confirms the R17 agreements made at RAN2#114 for enhanced INACTIVE eDRX: - - It is up to RAN to configure the length for PTW for RAN paging, the RAN PTW length can be different from the CN PTW length. - - When RAN and CN paging coincide in the same PH, the actually used PTW starting location is the same for RAN and CN paging. FFS how to calculate the PTW starting location so that it is the same for RAN and CN PTW. PTW length value range of enhanced INACTIVE eDRX is same as IDLE eDRX, i.e. from 1.28s to 40.96s in the step of 1.28s. Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s, i.e. hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024 Add the configuration of eDRX cycle (>10.24 s) and PTW length for enhanced INACTIVE eDRX in the RRCRelease message Introduce 1 bit indication in SIB1 whether UEs are allowed to use the enhanced INACTIVE eDRX cycle. FFS if/how to fallback for a UE which is configured with R18 eDRX but the gNB doesn’t indicate support for this. RAN2 confirms the enhanced INACTIVE eDRX can be applied to all R18 UEs. FFS if it can only be supported by UEs which support R17 eDRX.

The following was agreed in RAN2#121bis meeting:

Introduce an optional UE capability with signalling for Rel-18 enhanced eDRX in RRC_INACTIVE. UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_IDLE eDRX. TBD if it must also support Rel-17 RRC_INACTIVE eDRX. A cell can allow Rel-18 INACTIVE eDRX, only if eDRX-AllowedIdle is configured. TBD if it must also configure Rel-17 RRC_INACTIVE eDRX. UEs configured with Rel-18 enhanced INACTIVE eDRX should apply Rel-18 enhanced INACTIVE eDRX if Rel-18 enhanced INACTIVE eDRX is allowed in the serving cell, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell. UEs configured with Rel-18 enhanced INACTIVE eDRX should apply INACTIVE DRX if both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX are not allowed in the serving cell. Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX. Introduce a new IE for INACTIVE eDRX to include the eDRX cycle values larger than 10.24s. Following cases are invalid: - Case 1: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle but not configured with the IDLE eDRX cycle. - Case 2: UE is configured with a Rel-18 enhanced INACTIVE eDRX cycle longer than the IDLE eDRX cycle. RAN PTW length is mandatorily present within Rel-18 INACTIVE eDRX’s configuration. Use the same UE_ID_H as IDLE eDRX for calculating the PH for RAN paging when INACTIVE eDRX is longer than 10.24s. Use TeDRX_RAN instead of TeDRX_CN to calculate the PH for RAN paging when TeDRX_RAN is longer than 10.24s. For the overlapping PH, RAN PTW starting location is determined based on CN eDRX cycle. For the non-overlapping PH, PTW starting location for RAN PTW is determined based on the CN eDRX cycle. In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}. Legacy systemInfoModification-eDRX indication in Short message and eDRX modification boundaries are also applicable for Rel-18 UEs configured with INACTIVE eDRX > 10.24sec, and in this case, the CN eDRX cycle is used to compare with the modification period.

In this TDoc further details on eDRX in RRC_INACTIVE are discussed.

# 2	Discussion

In Rel-17 eDRX-AllowedIdle and eDRX-AllowedInactive were specified:

eDRX-AllowedIdle The presence of this field indicates that extended DRX for CN paging is allowed in the cell for UEs in RRC_IDLE or RRC_INACTIVE. The UE shall stop using extended DRX for CN paging in RRC_IDLE or RRC_INACTIVE if eDRX-AllowedIdle is not present.

eDRX-AllowedInactive The presence of this field indicates that extended DRX for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive is not present.

The following WA was agreed in RAN2#121bis:

Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

In case UE is not configured with Rel-17 INACTIVE eDRX we think that it would be simplest to follow the same approach than in R17 for the enhanced INACTIVE eDRX cycle.

Proposal 1: The UE shall stop using the “INACTIVE eDRX > 10.24s” for RAN paging in RRC_INACTIVE if the “INACTIVE eDRX > 10.24s allowed” is not present in the system information and the UE is not configured with Rel-17 INACTIVE eDRX.

Proposal 2: The UE configured with “INACTIVE eDRX > 10.24s” monitors paging according to DRX cycle in case the “INACTIVE eDRX > 10.24s allowed” is not present in the system information and the UE is not configured with Rel-17 INACTIVE eDRX.

# 3	Conclusion

Based on the discussion following is proposed:

Proposal 1: The UE shall stop using the “INACTIVE eDRX > 10.24s” for RAN paging in RRC_INACTIVE if the “INACTIVE eDRX > 10.24s allowed” is not present in the system information and the UE is not configured with Rel-17 INACTIVE eDRX.

Proposal 2: The UE configured with “INACTIVE eDRX > 10.24s” monitors paging according to DRX cycle in case the “INACTIVE eDRX > 10.24s allowed” is not present in the system information and the UE is not configured with Rel-17 INACTIVE eDRX.